Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

July 31, 2010

Water samples were collected at five stations for particle size distribution measurements using the LISST-100X particle counter. A total of 77 LISST samples were analyzed, including duplicates. Samples at depths of elevated fluorescence or other significance were selected from the CTD trace for fluorescence intensity ratio measurements and analyzed using a Quantech Life Sciences fixed wavelength fluorometer.

| Station | Latitude | Longitude |
| --- | --- | --- |
| BM149 | 28.446980 | -88.758778 |
| BM150 | 28.345700 | -88.777950 |
| BM151 | 28.324841 | -88.938187 |
| BM152 | 28.235683 | -89.128995 |
| BM153 | 28.131220 | -88.998907 |

All 5 stations sampled showed low small particle concentrations at all depths with the exception of sample BM15007-2 and BM15107-2 which were slightly elevated. CTD in-situ fluorometry profiles from all stations did not show elevated fluorescence throughout the water column.

Fluorescence intensity ratios were measured at the surface along with depths where changes were observed in the CTD in-situ dissolved oxygen profile. The results of fluorescence intensity ratios showed that low ratios were observed in both the near surface waters (3m or less) and in the deeper water samples.

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations BM149 to BM153.