30 July 2010 - Brooks McCall Status Report - Cruise 15 - Day 3

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Sampling yesterday completed a baseline of subsurface conditions in the vicinity of the wellhead. The strategy for Day 3 was to target the predicted location of any remaining subsurface oil approximately 50-90km southwest of the wellhead.  The starting point was OV141, the most western location that the Ocean Veritas reported evidence of a subsurface plume on their last voyage.  Overnight, we also received an ASA SIMAP subsurface oil trajectory prediction which indicated likely sampling locations in this area.

In addition to the subsurface monitoring usually undertaken, a great deal of additional effort has also been put into collecting and analysing DO samples from a range of depths using automated titration methods in accordance with Dispersant Monitoring and Assessment Directive – Addendum 4.

Water samples were also collected on behalf of Jan Kurtz (EPA) at three sites.

Weather for the day was fine and sunny with 1-2 ft seas and a light breeze. No surface oiling was observed. A total of 5 CTD casts were completed, as summarized below and shown in Figure 4.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM149 | 50 km SW | No signal | - | Two slight dips in the DO profile (0.5mg/L) ~50m high were evident at depths of 1150 and 1190m. |
| BM150 | 60 km SW | No signal | - | Two slight dips in the DO profile (0.5mg/L) ~50m high were evident at depths of 1100 and 1245m. |
| BM151 | 70 km SW | No signal | - | A slight depression (0.5mg/L) in the DO profile was evident between 1100 and 11500m. |
| BM152 | 95 km SW | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |
| BM153 | 90 km SW | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |

The days sampling showed no indication of subsurface oil at any of the five sites.  However, slight depressions in DO were located at depths where a subsurface plume has been previously located.  The absence of any indicators of oil in the LISST and fluorometry data provide weight to the idea that these DO dips are a residual signal of the natural breakdown of oil by resident bacterial populations.

Dissolved oxygen testing continued with a further 25 water samples collected and analyzed. A total of 65 samples have now been collected and analyzed on the cruise, 33 samples from deeper than 1000m, and 22 from shallower depths.  Figure 1 shows a simple plot of the in situ measurements collected from the Seabird sensor, and from samples returned to the surface and analyzed using automated titration methods. The two sets of measurements show a very high level of agreement (Table 1). There was also a high correlation between the handheld optical probe and the Seabird probe, but a poor correlation between the Seabird probe and the membrane probe (Table 1).

Table 1. Correlation of different measures of DO to the Seabird probe.

| Correlation values | Winkler (Hatch) | Optical Probe | Membrane Probe |
| --- | --- | --- | --- |
| Seabird | 0.969 | 0.907 | 0.795 |

Figure 1. Plot of in situ Seabird measurements and from samples returned to the surface and analyzed using automated Winkler (Hatch) titration methods. Data are sorted by depth.

Figure 2 shows the relationship between the two hand held probes.  It is obvious that there is a lot of variability in the measures related to use of the Extec membrane probe.

Figure 2. Plot of measurements from the two hand held probes. Data are sorted by depth.

Figure 3 is a scatter plot highlighting the variability of the membrane probe measures, and the relative consistency of the titrations and optical probe measurements, indicating a calibration difference rather than any variable measurement issues.

Figure 3. Scatter plot of measurements from the two hand held probes and the Winkler (Hatch) titrations.

All DO measures follow a profile typical of background values from the area and there are no indications of any conditions of concern about DO levels based on the current data. The data also indicate that the Seabird sensor is providing repeatable and meaningful results.  All data have been submitted with the daily deliverables.

VOC monitoring has been undertaken regularly throughout the day by Adam Kueker, Bureau Veritas Industrial Hygiene Specialist.  No VOCs were detected above background.

Rotox tests were started for samples BM149-153 (results due to be reported on 1 August 2010), and testing was completed for samples BM145-148.  No toxicity was apparent in any samples.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."

Figure 4. Location of Brooks McCall sampling stations, Cruise 15, 28 July-1 August 2010.