Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

August 06, 2010

Water samples were collected at six stations for particle size distribution measurements using the LISST-100X particle counter. A total of 43 duplicate LISST samples were analyzed. Samples at depths with elevated CDOM fluorescence levels and/or oxygen anomalies were selected for fluorescence intensity ratio measurements using a Quantech Life Sciences fixed wavelength fluorometer.

| Station | Latitude | Longitude |
| --- | --- | --- |
| BM162 | 28.247272 | -88.899780 |
| BM163 | 28.312755 | -88.970812 |
| BM164 | 28.249603 | -89.043393 |
| BM165 | 28.319215 | -89.107955 |
| BM166 | 28.385975 | -89.177977 |
| BM167 | 28.145580 | -89.419190 |

Small particle concentrations were <0.2 µL/L at all depths with the exception of the surface “bucket” samples (0.5 m) at Stations 162 (0.200µL/L) and 163 (0.210µL/L).  The CDOM in-situ fluorometry profiles from all stations did not show elevated fluorescence.

Fluorescence intensity ratios were measured at the surface along with depths where a slight depression was observed in the in-situ dissolved oxygen profile. The results of these analyses showed low fluorescence intensity ratios (<1.91).

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations BM162 to BM167.