30 July 2010 - Brooks McCall Status Report - Cruise 16 - Day 1

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

The primary purpose of this cruise was to complete Dispersant Monitoring and Assessment Directive – Addendum 4, with the collection and analysis of DO samples from a range of depths using automated titration methods.  The amperometric Winkler modified titration was undertaken by Research Associate George Berberino, University of Miami/NOAA, and NOAA Environmental Scientist Sean Kane.  The Winkler Hach modified titration was undertaken by Ed Morren, Senior Scientist with Shaw Environmental.  On board observing the testing were Dr Ken Lee, Senior Research Scientist, DFO Canada - and member of the JAG, and Kevin Larsen, EPA On Scene Coordinator, Region 7.

Sampling targeted the last known location of the rapidly degrading subsurface plume ~70-80km SW of the wellhead where the Ocean Veritas had detected very faint fluorescence and a small dip in DO levels.  This area was slightly closer to the wellhead and slightly further to the S and SW than the ASA SIMAP subsurface oil trajectory prediction.

Weather during the day was very hot and sunny with flat calm seas and virtually no breeze. No surface oiling was observed.  Because of the heat, the requirement for protective clothing (long pants) for sampling was relaxed and shorts were allowed. A total of 4 CTD casts were completed, as summarized below and shown in Figure 2.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM154 | 75 km SW of the wellhead | Very weak signal | 1215 | Two dips in the DO profile were evident at 1100m (~1.2mg/L) and 1260m and (0.2mg/L) and extending over ~100 vertical metres. |
| BM155 | 75 km SW of the wellhead | Very weak signal | 1220 | Two dips in the DO profile were evident at 1215m (~0.2mg/L) and 1180m and (~1.0mg/L) and extending over ~90 vertical metres. |
| BM156 | 75 km SW of the wellhead | Very weak signal | 1200 | One main dip in the DO profile evident at 1180m and (~1.0mg/L) and extending over ~90 vertical metres. |
| BM157 | 70 km SW | No signal | - | A depression in the DO profile was (~0.5mg/L) evident between 1150 and 1220m. |

Following the detection of the weak fluorescence signal and DO dip at BM154, it was decided to collect a total of three replicate casts at this location to maximize the collection of data relevant to the testing of DO levels as required under Addendum 4, and to also provide specific information on the short term temporal variability of the subsurface DO and fluorescence signals.

The CTD outputs of the three replicate casts (shown below), indicate a consistent signal, but slight variance in the strength of the DO dip, and in the depth the signal peaked at.  This is not unexpected and highlights the dynamic and evolving nature of the remaining subsurface indicators of dispersed oil.

A weak signal of slightly depressed DO was all that was detected at BM157, suggesting the site was at the edge of the residual plume.  The absence of any indicators of oil in the LISST and fluorometry data provide continued weight to the idea that these DO dips are a residual signal of the natural breakdown of oil by resident bacterial populations.

Twenty water samples were collected and analyzed using the modified Winkler titrations to measure dissolved oxygen, 16 from >1000m and 4 from <1000m - the vast majority collected between 1150m and 1250m.  The sampling ratio was altered to gather as much information possible from within the rapidly diminishing plume while it was present.  Sampling on subsequent days in the cruise will deliver the requisite 60:40 ratio specified in Addendum 4.  Data have been submitted with the daily deliverables and will be presented graphically once a larger data set is collected over the next 2 days. However, at this stage there appears to be good agreement between the two titration methods, and the Seabird sensor.

VOC monitoring has been undertaken regularly throughout the day by Josh Senter, Bureau Veritas Industrial Hygiene Specialist.  No VOCs were detected above background. The greatest concern today has been heat, which has been monitored and managed appropriately.

Rotox tests were started for samples BM154 and BM157 (results due to be reported on 5 August 2010), and testing results for sample BM153 (Cruise 15) were submitted.  No toxicity was apparent in any samples.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."