5 August 2010 - Brooks McCall Status Report - Cruise 16 - Day 2

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Day 2 was another beautiful day on the Gulf with clear sky, calm seas and virtually no breeze. Temperatures ranged from 99oF at 10am to106oF at 5pm.  No surface oiling was observed.

The days sampling focused on the continued collection and analysis of DO samples from a range of depths using automated titration methods.  Casts were made in a 5km grid from yesterday’s last sampling station to attempt to delineate the boundary of the residual plume in conjunction with the R/V Ferrel (see Figure over page). A total of 4 CTD casts were completed, as summarized below and shown in Figure 1.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM158 | 75 km SW of the wellhead | No signal | - | A sustained change in background DO between 1150 and 1300m (0.5mg/L), with the largest peak at 1170m. |
| BM159 | 70 km SW of the wellhead | Very weak | 1180 | Three dips in the DO profile were evident at 1150m, 1180m, and 1230m (~0.2-0.5mg/L) extending over ~100 vertical m. |
| BM160 | 65 km SW of the wellhead | No signal | - | Two dips in the DO profile evident at 1150m (~0.5mg/L) and 1190m (~0.2mg/L) extending over ~100 vertical metres. |
| BM161 | 65 km SW | No signal | - | A minor reduction in DO (~0.2mg/L) evident over 140m with samll peaks at 1140, 1220 and 1280m. |

All of the days casts showed a minor decrease in DO compared to background values at depths where the dispersed oil plume has been consistently located previously.  BM159 was the only site where a very small change in the background fluorometry signal was apparent, and this matched a corresponding dip in the DO.  The absence of any indicators of oil in the LISST and fluorometry data provide continued weight to the idea that these DO dips are a residual signal of the natural breakdown of oil by resident bacterial populations.

As for yesterday, the amperometric Winkler modified titration was undertaken by Research Associate George Berberino, University of Miami/NOAA, and NOAA Environmental Scientist Sean Kane.  The Winkler Hach modified titration was undertaken by Ed Morren, Senior Scientist with Shaw Environmental.  On board observing the testing were Dr Ken Lee, Senior Research Scientist, DFO Canada - and member of the JAG, and Kevin Larsen, EPA On Scene Coordinator, Region 7.

Twenty water samples were collected and analyzed using the modified Winkler titrations to measure dissolved oxygen, 14 from >1000m and 6 from <1000m - the majority collected between 1000m and 1300m.  The sampling ratio was altered to gather as much information possible from within the rapidly diminishing plume while it was present.  Over the first 2 days of the cruise 30 samples have now been collected from >1000m, and 10 from <1000m.  The remaining 10 samples required to be collected to deliver the requisite 60:40 ratio specified in Addendum 4 will be collected from 0-1000m tomorrow.

Data have been submitted with the daily deliverables and will be presented graphically once a larger data set is collected over the next 2 days. There is no place for submitting the Hach test results in the data spreadsheet so these have not been included.  Todays results continue to show high agreement between the two titration methods and the Seabird sensor.  In particular, DO from above, within and below the dips identified by the Seabird sensor were reproducible using all of the other measurement methods used onboard.  This provides compelling evidence that the Seabird sensors are returning accurate measures of DO, and have done so throughout the sampling program.

While in the field, direct measurements using the handheld YSI optical probe were also taken from directly within the Niskins after the samples had been drawn.  This showed samples in the Niskin were on average 0.5oC warmer and had a DO 0.2mg/l lower than those measured in the lab. 5-10 minutes after the samples were drawn. This is attributed to the combined influence of the CTD passing through warm 30oC surface waters then sitting on the sun on a hot deck, and the samples being drawn off into chilled sample containers prior to DO measurements.  Overall, the minor change recorded indicated that the handheld probes are an adequate cross check that the Seabird sensor has recorded values accurately from within the subsurface oil plume.

VOC monitoring has been undertaken regularly throughout the day by Josh Senter, Bureau Veritas Industrial Hygiene Specialist.  No VOCs were detected above background. Again, the greatest concern today has been heat, which has been monitored and managed appropriately.

Rotox tests were completed for samples BM154 and BM157. No toxicity was apparent in any samples. Rotox tests were started for samples BM158 through BM161 (results due to be reported on 6 August 2010).

The days sampling was able to delineate the edge of the residual plume identified yesterday, and sampling tomorrow will attempt to continue to delineate the plume further to the SW.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."