Sampling for Winkler oxygen analyses (Winklers), Final  8/17/2010

Oxygen analyses on discrete water samples are commonly done using the Winkler titration technique (Winklers).  These samples can be analyzed to high precision and accuracy of better then 0.4 umol/l (0.01 ml/l) at sea or, after the cruise, on shore provided proper sampling and storage protocols are exercised.  With the advent of precise in situ sensors on the CTD, Winklers are commonly taken to calibrate the oxygen sensor on the CTD and to check for artifacts on the sensor.

O2 sampling procedure

- Samples are taken in 125-ml Erlenmeyer flasks with ground glass stopper. Be sure that flask and stopper have the same number (they should be connected with a rubber leash).

- Sampling should be done as soon as the Niskin comes on deck since the water can lose/gain O2 when heating up.  Oxygen should be the first sample drawn from the Niskin.

- Rinse the flask twice with about ½ the bottle volume (40 to 60 ml)

- During the first rinse, flush all bubbles from the clear or opaque sampling tube (“noodle”), made of Tygon or silicone.  The bubbles are flushed by squeezing the tube at the bubbles, starting the flush at the Niskin’s drain cock.   Fill bottle from the bottom, avoiding entrainment of bubbles, and overflow approximately 2 volumes. Remove noodle while flowing such that the flask (including the expansion volume above the ground glass neck) are filled.

- Immediately after sampling add the chemicals “pickling reagent” in the following order from the re-pipettes:

1mL of MnCl2 (Manganous chloride) and

1 ml of NaOH/NaI

Lift the plunger up and then place the tip into the sample so that it extends at least 1 cm into       the water.  Then depress the plunger in a smooth steady motion to dispense the chemicals.

Do not mix pipettes for different reagents.

- Place stopper firmly on flask displacing excess seawater without trapping bubbles. Avoiding bubbles is critical. Tilting the stopper as it contacts the water surface can help.

- Shake bottle vigorously to mix the reagents. A brown precipitate (flocculant) will color the water until it settles to the bottom.

- Place bottle in plastic storage box. Fill tapered neck with fresh tap water to create a water seal around stopper. Place storage box in air conditioned space.  If samples are kept for more than 2 days, they should be periodically checked to replace any water that has evaporated.

- Fill out sample log sheet including date and time (GMT), Latitude and Longitude, station number, Niskin # Oxygen flask number, Oxygen sampling temperature, depth, temperature and salinity obtained from the CTD readout.

## O2 sample storage

Stored samples should be checked for water seal every 2 days or so. Fill neck with plastic squirt bottle if needed.  Samples should be stored in air-conditioned space. One set of copies of the sampling sheets should be included in the boxes. One set should be hand carried back.  Keep copies of the sampling sheets till it is confirmed that AOML has a copy. The samples are worthless without the data on the sampling sheet.

## Sampling locations

The primary purpose of the sampling is to calibrate an oxygen sensor which is installed on the CTD (CTD-O2). The sensor needs to be checked for offsets and for artifacts.  For offsets Winklers should be taken for a complete cast.  For CTD-O2 sensor artifacts Winklers should be taken if an oxygen dip is located in the 1100-1300 m depth range.

Duplicates: please take duplicates every fifth sample  (20 % of total samples) so we may assess sample integrity

Straw man sampling scheme

1. A full cast at the beginning of cruise, every 10th cast thereafter and the cast at the end of the cruise days including 20 % duplicate (1 out of 5).

2. Duplicate samples where the oxygen dip is observed.

Pertinent references

Carpenter, J. H. (1965), The Chesapeake Bay Institute technique for the Winkler  dissolved oxygen method, Limnol. Oceanogr., 10, 141-143.

Friederich, G. E., P. Sherman, and L. A. Codispoti (1984), A high precision automated Winkler titration system based on a HP-85 computer, a simple colorimeter and an inexpensive electromechanial buret, Bigelow Lab. Technical Report, 42, 24 pp.

Langdon, C. (2010), Determination of dissolved oxygen in seawater by Winkler titration using the amperometric technique, in GO-SHIP repeat hydrography manual: a collection of expert reports and guidelines, edited by B. M. Sloyan and C. Sabine, IOC/IOCCP, Paris.

Zhang, J.-Z., G. Berberian, and R. Wanninkhof (2002), Long-term storage of natural watersamples for dissolved oxygen determination, Water Research, 36/16, 4001-4004.