6 August 2010 - Brooks McCall Status Report - Cruise 16 - Day 3

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Day 3 sampling focused on collection and analysis of DO samples from a range of depths using automated titration methods, and tracking of the subsurface plume signals to the SW and WSW of the wellhead. A total of 6 CTD casts were completed, as summarized below and shown in Figure 1. No surface oiling was observed.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM162 | 75 km SW of the wellhead | No signal | - | A minor reduction in DO (~0.2mg/L) evident over 40m with a peak at 1145m. |
| BM163 | 77 km WSW of the wellhead | No signal | - | A reduction in DO (~0.5mg/L) evident over 70m between 1085m and the seabed. |
| BM164 | 85 km WSW of the wellhead | No signal | - | A reduction in DO (~0.2mg/L) evident over 50m between 1150m and the seabed. |
| BM165 | 87 km WSW | No signal | - | No reduction in DO compared to background. |
| BM166 | 90 km WSW | No signal | - | No reduction in DO compared to background. |
| BM167 | 125 km WSW | No signal | - | No reduction in DO compared to background. |

Three of the days 6 casts showed a minor decrease in DO compared to background values at depths where the dispersed oil plume has been consistently located previously.  There was no change apparent in the background fluorometry signal, and no indicators of oil in the LISST or fluorometry data.

Much of the days focus has been on compiling the DO data from the multiple measures undertaken on board.  Today, twelve water samples were collected and analyzed using the modified Winkler titrations to measure dissolved oxygen, 1 from >1000m and 11 from <1000m.  This was to achieve the 60:40 sampling ratio specified in Addendum 4, and included two additional samples.  These were added because one Winkler sample collected in the morning was unable to be analyzed due to the addition of the wrong reagent during lab preparation, and another as post-processing would have been required to extract a data point from the software which had a minor malfunction yesterday.  Because of these factors, two partial data rows (SW-20100805BM16-1, SW-20100806BM16-3) were excluded to provide a balanced paired analysis. In total, during this and the previous cruise, the Brooks McCall has now conducted 100 modified Winkler titrations, excluding lab duplicates and standards.  Lab standards showed the Winkler and Hach tests were capable of measuring to below 2 mg/L. The Ocean Veritas has completed 86 modified Winkler titrations.

Addendum 4, Additional Requirement 4 requests a correlation between the automated Winkler titration, and the probe measurements. Brooks McCall Cruise 16 paired sample correlation values are presented below:

| Correlation | Seabird | Hach | Optical | Membrane |
| --- | --- | --- | --- | --- |
| Winkler | 0.83 | 0.86 | 0.75 | 0.81 |
| Seabird |  | 0.96 | 0.92 | 0.88 |
| Hach |  |  | 0.91 | 0.93 |
| Optical |  |  |  | 0.74 |

These correlation values show a very good relationship exists between the Seabird CTD DO probe, and the other measures, particularly the Hach modified Winkler titration.  The larger combined data set of Cruise 15 and 16 has not been presented as it does not include University of Miami/NOAA Winkler results, but shows a very similar outcome.  The Ocean Veritas data also provide a very similar result.  This extensive combined data set provides compelling evidence that the Seabird sensors are returning accurate measures of DO.

This conclusion is further reinforced by the ability of the sensors to measure DO concentrations consistent with pre-spill conditions in the Gulf of Mexico in the upper part of a cast, then show a clear decrease in DO as it passes through dispersed subsurface oil, before returning to background values below the subsurface plume.  This pattern has then been consistently repeated the upcast.  The ability of the sensor to repeatedly return to background DO concentrations after passing through subsurface oil provides strong evidence that the CTD DO sensor is not being compromised.

Representative examples of the data from Cruise 16 are presented below. Sampling within the residual subsurface plume targeted dips in DO.  Station BM154 shows how all the measures used were able to measure the shift from background concentrations to a low point (1215m) in a DO depression accurately.  Station BM161 shows the consistent relationship with measures through the water column at 50m intervals.

Data have been submitted with the daily deliverables.  No regression analyses have been performed as the data already tell such a clear story that further analysis is not considered likely to alter the conclusion in any meaningful way.

Throughout this sampling effort on the Brooks McCall I would like to formally acknowledge the huge effort and contribution put in by Research Associate George Berberino (University of Miami/NOAA), Sean Kane (NOAA Environmental Scientist), and Ed Morren (Senior Scientist, Shaw Environmental).  Their technical expertise, professionalism, integrity and willingness to resolve this key question have been exemplary.  I would also like to acknowledge the input and feedback of the on board observers, Dr Ken Lee, Senior Research Scientist, DFO Canada - and member of the JAG, and Kevin Larsen, EPA On Scene Coordinator, Region 7.

VOC monitoring has been undertaken regularly throughout the day by Josh Senter, Bureau Veritas Industrial Hygiene Specialist.  No VOCs were detected above background. Again, the greatest concern today has been heat, which has been monitored and managed appropriately.

Rotox tests were completed for samples BM158 through BM161. No toxicity was apparent in any samples. Rotox tests were started for samples BM163 through BM166 (results due to be reported on 7 August 2010).

The days sampling struggled to find the residual plume identified yesterday.  Where the water depth shallowed to less than 1000m, the depth of the minor subsurface DO signal was measured in contact with the seabed and was sampled. There was no indication of any residual oil at concentrations detectable by fluorescence.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."