Readme_Submission of AOML_Winkler O2

December 4, 2010

These data are the O2 Winkler analyses made shipboard and at a shore-based laboratory by George Berberian of the oxygen group at the Atlantic Oceanographic and Meteorological Laboratory (AOML). Data analysis and quality control were performed by Rik Wanninkhof (AOML) and Geun-Ha Park (AOML/CIMAS).

It includes measurements on the following ships: the Nancy Foster, the Pisces, the Bigelow; the Brooks McCall and the Ocean Veritas covering a time period from July 1, 2010 to August 30, 2010.

All files are saved as Excel 4.0 worksheet.

All sample designations and CTD information were obtained from the processed CTD file at

http://data.nodc.noaa.gov/DeepwaterHorizon/Ship/

On or about November 24, 2010

Headers in the data files:

Vessel/Cruise:  designation of ship and cruise

Station: Station #

Niskin Bottle #:  position of the Niskin bottle on the Rosette

Date

Time

Pressure

Po Temp: Potential temperature as obtained from the processed CTD file

Temperature: temperature as obtained from the processed CTD file

Salinity: Salinity as obtained from the processed CTD file

CTD O2 (mg/L): O2 from CTD sensor (SBE43) in mg/l as obtained from the processed CTD file

CTD O2(ml/L):O2 from CTD sensor (SBE43) in mg/l as obtained from the processed CTD file

CTD O2(umol/l): O2 from CTD sensor (SBE43) in micromol/l as obtained from the processed CTD file

Winkler O2 (umol/l):O2 from Winkler analysis in micromol/l as obtained from AOML

Winkler O2duplicate: (if taken) duplicate analyses from same Niskin in micromol/l as obtained from AOML

Winkler O2 (ml/l): O2 from Winkler analysis in ml/l as obtained from AOML

Winkler O2duplicate:  (if taken) duplicate analyses from same Niskin in ml/l as obtained from AOML

QC_Winkler O2:  Quality control flag. A flag of 2 is assigned to a good sample.  A flag of 3   indicates a questionable Winkler analysis and a flag of 4 indicates a bad sample.  The quality flags are based on correspondence (or lack thereof) with the CTD/O2 sensor value.

Further information can be found in the report :

Oxygen Winkler titrations by NOAA/AOML in support of Deep Water Horizon spill monitoring

Rik Wanninkhof, NOAA/AOML

Geun-Ha Park, NOAA/AOML/CIMAS

George Berberian, NOAA/AOML/CIMAS