10 August 2010 - Brooks McCall Status Report - Cruise 17 - Day 1

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Day 1 sampling focused on tracking the subsurface plume signals to the SW of the wellhead to determine its spatial extent, with sampling locations determined based on Operation Clean Sweep Mission Guidance.  In addition, samples were analysed on-board for DO using automated titration methods. A total of 3 CTD casts were completed, as summarized below and shown in Figure 1. No surface oiling was observed.  Fewer casts than usual were completed because the deeper waters in this area increase cast deployment and retrieval times.  Conditions were overcast throughout the day and seas were 1-3 ft, increasing gradually throughout the day.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM169 | 95 km WSW | No signal | - | A reduction in DO (~0.5mg/L) evident over 80m with a peak at 1100m. |
| BM170 - T1, S5 | 100 km SW | No signal | - | No reduction in DO compared to background. |
| BM171 - T2, S5 | 115 km SW | No signal | - | No reduction in DO compared to background. |

Across all three sites there was no change apparent in the background fluorometry signal, and no indication of oil in the LISST or fluorometry data. As such, the sampling focus turned specifically to DO.

The first cast of the day was between Transects 1 and 2 near Station 3, an area where the Ferrel, Ocean Veritas and Brooks McCall have all previously identified minor decreases in DO compared to background values.  A clear DO signal (0.5mg/l) was located and multiple samples were collected at 10m intervals between 1070 and 1150 metres within the DO depression to see how closely the CTD seabird sensor tracked the DO signal in relation to the modified Winkler analyses.  Casts 2 and 3 were on the Mission Guidance grid (T1 S5 and T2 S5), and showed no DO depression.  Samples at these sites were collected from across the entire depth profile at 200m intervals for modified Winkler DO analyses.

A total of 32 samples were analyzed by both Hach and automated Winkler methods.  Results of the first 2 stations are available (see figures below) and, like the results of the previous 2 cruises, show very good agreement between the different measures.

Winkler data have been submitted with the daily deliverables but there is still no option for recording Hach data in this spreadsheet.  For these 2 stations the automated Winkler has a correlation of 0.94, and the Hach test a correlation of 0.98.

Rotox tests were started for samples BM169 through BM171 (results due to be reported on 11 Aug. 2010).

The days results again indicate that the residual plume signals are not being seen at depths greater than 1200m and that fluorometric signals indicative of hydrocarbons are becoming very difficult to find indicating ongoing rapid biodegradation of the dispersed oil.

Sampling will continue tomorrow on Transect 3 (S3, S2 and S1) if weather conditions allow.  Based on non-detection of the plume signal at stations T1 S4 and T2 S5, and feedback provided by Debbie French McCay that the residual plume was likely to be further to the west, we have decided not to sample outer stations of T1 and T2 (T1, S6 and S7 and T2, S6 and S7) at this time as we hope to encounter the DO dips while the Winkler testing capacity remains on-board.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."