10 August 2010 - Brooks McCall Status Report - Cruise 17 - Day 2

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Day 2 of Cruise 17 continued tracking the subsurface plume signals to the SW of the wellhead based on Operation Clean Sweep Mission Guidance.  In addition, samples were collected for on-board DO analysis using automated titration methods.  Despite the tropical storm warnings, conditions remained reasonable throughout the morning with 15 knot winds and 1-3 ft seas, increasing to 4ft with an underlying swell after midday.  No surface oiling was observed.  Because the weather was forecast to significantly worsen over the following 24 hours, sampling was limited to 3 casts in the early part of the day to allow a safe return to port along with the rest of the response fleet.

The 3 CTD casts completed today are summarized below and shown in Figure 1.  The casts allowed us to complete the majority of Transects 1-3 in conjunction with the R/V Ferrel and the R/V Ocean Veritas who have been working the area for the past few days.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM172 – T3, S3 | 110 km WSW | No signal | - | A reduction in DO (~0.75mg/L) evident over 100m between 1100-1200m, and with peaks at 1140m and 1190m. |
| BM173 – T3, S2 | 105 km SW | No signal | - | No reduction in DO compared to background. |
| BM174 – T3, S1 | 100 km SW | No signal | - | No reduction in DO compared to background. |

Across all three sites there was no change in the background fluorometry signal, and no indication of oil in the LISST or fluorometry data. A relatively strong DO signal (0.75mg/L below background) was returned on cast 1 (BM 172) at the depth the subsurface plume has been consistently located.  As such, intensive sampling was undertaken throughout this DO signal for comparison of the Seabird in situ CTD measurements, and those obtained from modified Winkler methods (Hach and University of Miami automated Winkler).  A total of 11 samples were analyzed by both Hach and automated Winkler methods today, making a total of 44 for the cruise. Of these, 32 were from depths greater than 1000m, and 12 from shallower depths. Results of station BM172 are presented below. Like previous results, there was very good agreement between the different measures, although at this particular site, the DO measures returned by the Hach and Winkler methods were consistently lower than the Seabird in situ measures and the optical probe.  Note, no values are presented for the membrane probe as the membrane fell off in the first samples and needed to be replaced and recalibrated before use.

DO data have been submitted with the daily deliverables.  For the current cruise, where 4 stations of additional DO measures were undertaken, the automated Winkler has a correlation of 0.87, and the Hach test a correlation of 0.96.  These are very consistent and repeatable results that show the in situ Seabird CTD sensor is accurately measuring DO.

The results also show that the DO signals indicating the location of the residual subsurface plume are present around 90-100km SW of the wellhead (e.g. ~N27 58.00 S-88 54.00 degrees minutes).  This corresponds to areas near Transects 1, 2 and 3, Station 2-3 as described in the Operation Clean Sweep Mission Guidance.  The results suggest sampling effort for the next few days (weather depending) may be best placed between Stations 1-4 on the grid layout and tracking in a SW direction from the wellhead.

Rotox tests will be completed for samples BM169 through BM171 tonight.  Rotox tests were started for sample BM172 (results due to be reported on 12 Aug. 2010).  Tests were not started for the later 2 casts because of rough sea conditions.

The days results again indicate that the residual plume signals are not being seen at depths greater than 1200m and that fluorometric signals indicative of hydrocarbons are becoming very difficult to find, indicating ongoing rapid biodegradation of the dispersed oil.  This rapid breakdown of the oil is consistent with what we now know about the biodegradation rate of the oil released from the wellhead.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."