22 August 2010 R/V Ferrel Status Report, Cruise 7 – Day 4

Compiled by Zach Weber, Chief Scientist

1.)	Site Conditions, Daily Activities and Sampling Locations

The Ferrel spent the day approximately 167 miles South West of the MC-252 wellhead conducting casts along transect 18 as directed in the Mission Guidance.  The Ferrel arrived to its first sample location at 0700 166 miles South West of the wellhead and sampled two locations:  Transect 18 Station 11 and Transect 18 Station 12.  Average weather conditions: Air Temperature 88 degrees Fahrenheit, seas 1-2ft, current 1kt out of the South East, winds 10-15kts out of the South West, and cloud coverage remained around 35% throughout the day.  The two CTD casts were undertaken to try to locate and sample remnants of the dispersed oil plume that may be around the 1150m depth but no evidence showing impact deeper than 1400m as indicated by a dip in dissolved oxygen levels and a rise in fluorescence in the water column. The cast (T18 S11) was conducted with no altimeter reading for depth to seabed.  The cast showed no visible peak in fluorescence or in dissolved oxygen.  The cast (T18 S12) was conducted with a reading of 1635m of depth to the sea bed.  The cast showed no visible peak in fluorescence or depression in dissolved oxygen.  Field analyses included dissolved oxygen, fluorescence, temperature, pH and salinity. Water samples were also collected for further chemical and microbiological analyses at three depths on each the two casts (at 100m, 1150m, and 1300m).  In addition, surface water samples were collected for microbiological analyses at both locations.

The results of the CTD casts were:

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BP-TN14-SS04 (T18 S11) | 166 miles SW | None | N/A | No DO depression. Total CTD cast depth 1500m. |
| BP-TN14-SS05 (T18 S12) | 167 miles SW | None | N/A | No DO depression. Total CTD cast depth 1608m. |

2.)	Air Quality

Not required.

3.)	Science Crew

| Name | First Name | Affiliation | Role |
| --- | --- | --- | --- |
| Weber | Zach | CTEH | Chief Scientist |
| Weiand | Corey | Entrix | Water Sampler |
| Pelling | Ray | Entrix | Water Sampler |
| Smith | Craig | C&C | Party Chief Team |
| Burke | Emily | C&C | Oceanography Team |
| McLeod | Alex | C&C | Oceanography Team |
| Tidwell | Hearn | SHAW | Microbiologist |
| Chastain | Roger | SCRIPPS | Microbiologist |
| Reid | Francine | LBNL | Microbiologist |
| Summers | Kaitlyn | NOAA | Data Manager |