26 August 2010 R/V Ferrel Status Report, Cruise 8 – Day 3

Compiled by Zach Weber, Chief Scientist

1.)	Site Conditions, Daily Activities and Sampling Locations

The Ferrel spent the day approximately 145 miles South West of the MC-252 wellhead conducting casts along transect 10 as directed in the Mission Guidance.  The Ferrel arrived to its first sample location at 0645 142 miles South West of the wellhead and sampled two locations:  Transect 10 Station 21 and Transect 10 Station 22.  Average weather conditions: Air Temperature 83 degrees Fahrenheit, seas 3-4ft, current .5kt out of South West, winds 15-20kts out of the South East, and cloud coverage remained around 90% with light rain at the beginning of the day.  The two CTD casts were undertaken to try to locate and sample remnants of the dispersed oil plume that may be around the 1150m depth but no evidence showing impact deeper than 1400m as indicated by a dip in dissolved oxygen levels and a rise in fluorescence in the water column.  The cast (T10 S21) was conducted in water exceeding the total recommended CTD cast depth.  The cast showed no visible peak in fluorescence or depression in dissolved oxygen. The cast (T10 S22) was conducted in water exceeding the total recommended CTD cast depth.  The cast showed no visible peak in fluorescence or depression in dissolved oxygen.  Field analyses included dissolved oxygen, fluorescence, temperature, pH and salinity. Water samples were also collected for further chemical and microbiological analyses at three depths on each of the two casts (at 1050m, 1150m, and 1250m at both casts).  In addition, surface water samples were collected for microbiological analyses at both locations.

The results of the CTD casts were:

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BP-TN15-SS06 (T10 S21) | 142 miles SW | None | N/A | No DO depression. Total CTD cast depth 1500m. |
| BP-TN15-SS07 (T10 S22) | 148 miles SW | None | N/A | No DO depression. Total CTD cast depth 1500m. |

2.)	Air Quality

Not required.

3.)	Science Crew

| Name | First Name | Affiliation | Role |
| --- | --- | --- | --- |
| Weber | Zach | CTEH | Chief Scientist |
| Weiand | Corey | Entrix | Water Sampler |
| Pelling | Ray | Entrix | Water Sampler |
| Smith | Craig | C&C | Party Chief Team |
| Burke | Emily | C&C | Oceanography Team |
| McLeod | Alex | C&C | Oceanography Team |
| Tidwell | Hearn | SHAW | Microbiologist |
| Reid | Francine | LBNL | Microbiologist |
| Summers | Kaitlyn | NOAA | Data Manager |