Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

August 24, 2010

Water samples were collected at four stations for particle size distribution measurements using the LISST-100X particle counter. A total of 44 LISST samples were analyzed, including duplicates. Samples at depths of elevated fluorescence or other significance were selected from the CTD trace for fluorescence intensity ratio measurements and analyzed using a Quantech Life Sciences fixed wavelength fluorometer.

Figure 1 presents the small droplet (∑ 2.5 - 60µm) particle size data and fluorescence intensity ratios for stations BM190 through BM193.  The station locations were:

| Station | Latitude | Longitude |
| --- | --- | --- |
| BM190 | N 28 16.7324 | E -88 55.5991 |
| BM191 | N 28 09.9564 | E -88 50.5362 |
| BM192 | N 28 03.2304 | E -88 45.5239 |
| BM193 | N 27 56.4522 | E -88 40.4732 |

All 4 stations showed higher small particle concentrations at the surface than at depth.

Small particle concentrations at all stations were uniformly low in the deeper water.  At station BM191 a very weak increase in the CTD fluorescence signal occurred at 1150 m, therefore the FIR was measured for this sample depth. The FIR for this sample was 1.90.  At the other three stations there was no increase in CTD fluorescence signal in deep water.

Fluorescence intensity ratios for the four stations were all less than 2.

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations BM190 to BM193.