25 August 2010 – Ocean Veritas Status Report – Cruise 16 – Day 1

Compiled by: Dan Pisegna, Fugro GEOS, Inc. (for BP)

Table 1 – Science Team Roster for 25 August 2010

| Name | Affiliation | Role |
| --- | --- | --- |
| Dan Pisegna | Fugro GEOS | Chief Scientist |
| Debbie Santavy | EPA | Observer |
| Claire McIntyre | DFO | Particle Analysis |
| Tom King | DFO | Particle Analysis |
| Shane Johnson | EM&A | Toxicology |
| Mark Deuger | Entrix | Sampling Technician |
| Connor Kobeski | Entrix | Sampling Technician |
| Christy Stephenson | NOAA | Data Management |
| Manuel Haenggi | Fugro GEOS | Oceanography Team |
| Robert Alexander | Fugro GEOS | Oceanography Team |
| Hannuman Bull | Fugro GEOS | Oceanography Team |
| Jeremy Peters | Fugro GEOS | Oceanography Team |
| Brian Moore | Fugro Chance | Navigation |

Today’s sampling plan, as directed by the Subsurface Monitoring Unit for 25 August 2010, was to start at Transect 2, Station 1 and work our way southeast along the transect.

Today’s weather conditions are summarized in Table 2, below.

Table 2 – Summary of the Weather for 25 AUG 2010

| Time | Wind | Sea | Weather |
| --- | --- | --- | --- |
| 07:00 | Variable < 5 knots | 1-2 feet | Partly Cloudy, Low 80’s |
| 12:00 | SE 10-15 knots | 1-2 feet | Partly Cloudy, Low 80’s |
| 17:00 | S 10-15 knots | 3 feet | Cloudy, Low 80’s |

Starting today, cast data will be collected using a Seabird 19 plus V2 CTD (equipped with a Seabird 43 dissolved oxygen sensor) and a Seabird 33 carousel deck unit.  This equipment replaced the borrowed Seabird 9 plus CTD (equipped with a Seabird 43 oxygen sensor) and Seabird 32 carousel deck unit that was returned to NOAA.  All other instrumentation remained the same.

The cast for station OV200 (T2, S1) was conducted approximately 90 kilometers south- southwest of the wellhead to a depth of 1186 meters.  There was no fluorescence signal, but two depressions in dissolved oxygen were observed, one at 1126 meters (~0.1 mg/l) and the other at 1169 meters (~0.1 mg/l).  We continued southeast along Transect 2 and conducted the cast for station OV201 (T2, S2) approximately 92 kilometers south-southwest of the wellhead to a depth of 1394 meters.  There was no fluorescence signal, but two depressions in dissolved oxygen were observed, one at 1041 meters (~0.25 mg/l) and the other at 1142 meters (~0.5 mg/l).  No sample was collected at 1041 meters due to the misfire of Niskin bottle #3 (bottom stopper did not close completely).  We continued southeast along Transect 2 and conducted the cast for station OV202 (T2, S3), approximately 98 kilometers south-southwest of the wellhead, to a depth of 1500 meters. There was no fluorescence signal, but a peak in dissolved oxygen was observed at 1079 meters (~0.25 mg/l) before a depression in dissolved oxygen at 1100 meters (~0.5 mg/l).  We continued southeast along Transect 2 and conducted a cast for station OV203 (T2, S4), approximately 106 kilometers south-southwest of the wellhead, to a depth of 1550 meters.  There was no fluorescence signal, but a depression in dissolved oxygen (~0.1 mg/l) was observed at 1143 meters.  A peak in dissolved oxygen (<0.1 mg/l) was observed at 1352 meters.  We also observed an oily substance on/inside the top stopper of Niskin bottle #1.  A sample was taken using a Teflon disk, stored in a glass jar, and will be taken to the Houma command center for further investigation.  Although the CTD carousel was deployed to the depth of 1567 meters (33 meters above the seafloor), the sample from bottle #1 was collected at a depth of 1550 meters (50 meters from the seafloor).  It was also noted that small oily smears were present on the outside of Niskin bottles #3 and #5.  An illustration of the oily substance in/on Niskin bottle #1 is shown in Figure 1, below.

Figure 1 – Oily Substance Discovered on Niskin Bottle #1, OV203, 25 August 2010

Our final cast of the day was conducted at station OV204 (T2, S5) approximately 113 kilometers south-southwest of the wellhead to a depth of 1678 meters.  There was no fluorescence signal and no depression in dissolved oxygen for station OV204.

The locations and results of the CTD casts conducted today are summarized in Table 3 and Figure 2, below.

Table 3 – Station Summary for 25 August 2010

| Station | Sampling Location and Position at Deployment | Fluorescence Signal | Signal Depth | Dissolved Oxygen Depression | Depression Depth |
| --- | --- | --- | --- | --- | --- |
| OV200 | T2,S1 Lat: 28.204352 Long: -89.053666 | No Signal | - | ~0.1 mg/l ~0.1 mg/l | 1126 m (1115 m – 1160 m) 1169 m (1160 m – 1175 m) |
| OV201 | T2, S2 Lat: 28.090496 Long: -88.968819 | No Signal | - | ~0.25 mg/l ~0.5 mg/l | 1041 m (1025 m – 1065 m) 1142 m (1110 m – 1161 m) |
| OV202 | T2, S3 Lat: 27.979074 Long: -88.884917 | No Signal | - | ~0.5 mg/l | 1100 m (1080 m – 1128 m) |
| OV203 | T2, S4 Lat: 27.866155 Long: -88.800626 | No Signal | - | ~0.1 mg/l | 1143 m (1125 m – 1170 m) |
| OV204 | T2, S5 Lat: 27.753480 Long: -88.717280 | No Signal | - | No Depression | - |

Figure 2 – Sampling Locations for 25 August 2010

Surface oil sheen was observed in Niskin bottle #1 (1550 meters) at station OV201.  This coincides with the oily substance that was discovered in/on the bottle.  No surface oil sheen was observed in any of the other samples collected today.  Comparisons of DO concentrations (CTD’s SBE 43 oxygen sensor versus the hand-held YSI ProODO optic probe) are included in today’s deliverables.  Although the hand-held probe reads consistently several tenths mg/l dissolved oxygen higher than the CTD’s oxygen sensor, the correlation between the two remains excellent.

At station OV200, we saw an increase in particle size from our typical background of 0.2 – 0.4 µl/l.  From the surface to 3 meters, we saw a background in particle size of approximately 1-2 µl/l.   From 3 meters to 50 meters, the particle size gradually decreased to our typical background of 0.2 – 0.4 µl/l.  For all other stations, a slight elevation (~0.5 µl/l) was observed in particle size background from the surface to 3 meters before gradually decreasing to our typical background (0.2 – 0.4 µl/l) at 50 meters (from 3 meters).  All of today’s stations were free of anomalies in fluorescence intensity.  LISST particle analysis and fluorescence intensity ratios are included in today’s deliverables.

Rotifer toxicity screening tests were started for the following stations:  OV200, OV201, OV203, OV204, and OV205.  The results for 25 AUG 2010 will be reported on 26 AUG 2010.  The stations and depths at which the screenings were conducted have been recorded in Table 4, below.

Table 4 - Rotoxkit M Analysis for 25 August 2010

| Station | Depth | Station | Depth | Station | Depth | Station | Depth | Station | Depth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OV200 | Control | OV201 | Control | OV202 | Control | OV203 | Control | OV204 | Control |
| OV200 | 1186 m | OV201 | 1394 m | OV202 | 1500 m | OV203 | 1550 m | OV204 | 1678 m |
| OV200 | 1169 m | OV201 | 1142 m | OV202 | 1100 m | OV203 | 1352 m | OV204 | 1300 m |
| OV200 | 800 m | OV201 | 1041 m | OV202 | 1079 m | OV203 | 1143 m | OV204 | 800 m |
| OV200 | 400 m | OV201 | 800 m | OV202 | 377 m | OV203 | 400 m | OV204 | 400 m |
| OV200 | 3 m | OV201 | 3 m | OV202 | 3 m | OV203 | 3 m | OV204 | 3 m |