26 August 2010 – Ocean Veritas Status Report – Cruise 16 – Day 2

Compiled by: Dan Pisegna, Fugro GEOS, Inc. (for BP)

Table 1 – Science Team Roster for 26 August 2010

| Name | Affiliation | Role |
| --- | --- | --- |
| Dan Pisegna | Fugro GEOS | Chief Scientist |
| Debbie Santavy | EPA | Observer |
| Claire McIntyre | DFO | Particle Analysis |
| Tom King | DFO | Particle Analysis |
| Shane Johnson | EM&A | Toxicology |
| Mark Deuger | Entrix | Sampling Technician |
| Connor Kobeski | Entrix | Sampling Technician |
| Christy Stephenson | NOAA | Data Management |
| Manuel Haenggi | Fugro GEOS | Oceanography Team |
| Robert Alexander | Fugro GEOS | Oceanography Team |
| Hannuman Bull | Fugro GEOS | Oceanography Team |
| Jeremy Peters | Fugro GEOS | Oceanography Team |
| Brian Moore | Fugro Chance | Navigation |

Today’s sampling plan, as directed by the Subsurface Monitoring Unit for 26 August 2010, was to start at Transect 3, Station 1 and work our way southeast along the transect.

Today’s weather conditions are summarized in Table 2, below.

Table 2 – Summary of the Weather for 26 AUG 2010

| Time | Wind | Sea | Weather |
| --- | --- | --- | --- |
| 07:00 | S 8-10 knots | 2-3 feet | Overcast, Low 80’s |
| 12:00 | S 10-15 knots | 3 feet | Overcast, Low 80’s |
| 13:15 | S 25-30 knots | 4-6 feet | Rain, Low 80’s |
| 17:00 | S 25-30 knots | 5-6 feet | Rain, Low 80’s |

The cast for station OV205 (T3, S1) was conducted approximately 104 kilometers southwest of the wellhead to a depth of 950 meters.  There was no fluorescence signal and no depression in dissolved oxygen for station OV205.  We continued southeast along Transect 3 and conducted the cast for station OV206 (T3, S2) approximately 107 kilometers southwest of the wellhead to a depth of 1260 meters.  There was no fluorescence signal and no depression in dissolved oxygen for station OV206. We continued southeast along Transect 3 and conducted the cast for station OV207 (T3, S3), approximately 112 kilometers southwest of the wellhead, to a depth of 1325 meters. There was no fluorescence signal, but a depression in dissolved oxygen was observed ranging from 1012 meters to1080 meters.  Two distinctive dips were observed within the depression, one at 1020 meters (~0.65 mg/l) and the other at 1062 meters (~0.35 mg/l).  The weather picked up (25-30 knot winds, 4-6 foot seas, and rain) at approximately 15:00 and all operations were put on hold.  At 17:00, operations were discontinued for the remainder of the day due to the safety concerns associated with the weather.

The locations and results of the CTD casts conducted today are summarized in Table 3 and Figure 1, below.

Table 3 – Station Summary for 26 August 2010

| Station | Sampling Location and Position at Deployment | Fluorescence Signal | Signal Depth | Dissolved Oxygen Depression | Depression Depth |
| --- | --- | --- | --- | --- | --- |
| OV205 | T3,S1 Lat: 28.129327 Long: -89.182705 | No Signal | - | No Depression | - |
| OV206 | T3, S2 Lat: 28.014826 Long: -89.097079 | No Signal | - | No Depression | - |
| OV207 | T3, S3 Lat: 27.903996 Long: -89.011608 | No Signal | - | ~0.65 mg/l ~0.35 mg/l | 1020 m (1012 m – 1040 m) 1062 m (1040 m – 1080 m) |

Figure 1 – Sampling Locations for 26 August 2010

No surface oil sheen was observed in any of the samples collected today.  Comparisons of DO concentrations (CTD’s SBE 43 oxygen sensor versus the hand-held YSI ProODO optic probe) are included in today’s deliverables.  Although the hand-held probe reads consistently several tenths mg/l dissolved oxygen higher than the CTD’s oxygen sensor, the correlation between the two remains excellent.

A slight elevation in small particle concentration was detected in samples collected at the surface to 3 meters for all of today’s stations.  All of today’s stations were free of anomalies in fluorescence intensity.  In fact, it was noted that the fluorescence intensities in the samples collected at the surface to 3 meters were slightly lower than what has been observed in the past weeks.  LISST particle analysis and fluorescence intensity ratios are included in today’s deliverables.

The results of yesterday’s (25 AUG 2010) rotifer toxicity screening tests noted 1 mortality in the control of OV200 and 1 mortality in the sample taken at 377 meters of OV202.  The results for 25 AUG 2010 are included in today’s deliverables.  Rotifer toxicity screening tests were started for the following stations:  OV205, OV206, and OV207.  The results for 26 AUG 2010 will be reported on 27 AUG 2010.  The stations and depths at which the screenings were conducted have been recorded in Table 4, below.

Table 4 - Rotoxkit M Analysis for 26 August 2010

| Station | Depth | Station | Depth | Station | Depth |
| --- | --- | --- | --- | --- | --- |
| OV205 | Control | OV206 | Control | OV207 | Control |
| OV205 | 950 m | OV206 | 1260 m | OV207 | 1325 m |
| OV205 | 900 m | OV206 | 1150 m | OV207 | 1150 m |
| OV205 | 700 m | OV206 | 800 m | OV207 | 1062 m |
| OV205 | 400 m | OV206 | 400 m | OV207 | 377 m |
| OV205 | 3 m | OV206 | 3 m | OV207 | 3 m |