Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

August 25, 2010

Water samples were collected at five stations for particle size distribution measurements using the LISST-100X particle counter.  A total of 83 LISST samples were analyzed, including duplicates.  Samples from the surface and 3m, plus those at depths of elevated fluorescence selected from the CTD trace, were analyzed for fluorescence intensity ratio measurements using a Quantech Life Sciences fixed wavelength fluorometer.

Figure 1 presents the small droplet (∑ 2.5 - 60µm) particle size data and fluorescence intensity ratios for stations OV200 through OV204.  The station locations were:

OV200: Lat= 28.204352	Long= -89.053666

OV201: Lat= 28.090496	Long= -88.968819

OV202: Lat= 28.979074         Long= -88.884917

OV203: Lat= 27.866155	Long= -88.800626

OV204: Lat= 27.753480	Long= -88.717280

High small particles concentrations were detected in surface (0.5 to 3m) samples at all stations compared to the deep water samples. However the deep water samples, at all stations, had consistently higher small particles concentrations then what was previously detected on August 19 & 20, 2010.  There was no evidence of a subsurface plume based on the CTD fluorometer trace at any of the five stations.

The fluorescence intensity ratios were between 0.70 and 1.36 for the five sampling stations. These values were lower than what was detected previously on August 19 & 20, 2010.

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations OV200 to OV204.