Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

August 26, 2010

Water samples were collected at three stations for particle size distribution measurements using the Sequoia LISST-100X particle counter.  A total of 51 LISST samples were analyzed, including duplicates.  Samples from the surface and 3m, plus those at depths of elevated fluorescence selected from the CTD trace, were analyzed for fluorescence intensity ratio measurements using a Quantech Life Sciences fixed wavelength fluorometer.

Figure 1 presents the small droplet (∑ 2.5 - 60µm) particle size data and fluorescence intensity ratios for stations OV205 through OV207.  The station locations were:

OV205: Lat= 28.129327	Long= -89.182705

OV206: Lat= 28.014826	Long= -89.097079

OV207: Lat= 27. 903996	Long= -89.011608

Slightly elevated small particles concentrations were detected in surface (0.5 to 3m) samples at stations OV205 and OV207. There was a slight elevation in small particles concentrations at 400m for Station OV205. However the deep water samples in most cases, at all stations, had consistently low small particles concentrations.  There was no evidence of a subsurface plume based on the CTD fluorometer trace at any of the three stations.

The fluorescence intensity ratios were between 0.73 and 1.21 for the three sampling stations.

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations OV205 to OV207.