Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

August 27, 2010

Water samples were collected at four stations for particle size distribution measurements using the Sequoia LISST-100X particle counter.  A total of 64 LISST samples were analyzed, including duplicates.  Samples from the surface and 3m, plus those at depths of elevated fluorescence selected from the CTD trace, were analyzed for fluorescence intensity ratio measurements using a Quantech Life Sciences fixed wavelength fluorometer.

Figure 1 presents the small droplet (∑ 2.5 - 60µm) particle size data and fluorescence intensity ratios for stations OV208 through OV211.  The station locations were:

OV208: Lat= 27.790336	Long= -88.926180

OV209: Lat= 27.678555 	Long= -88.842845

OV210: Lat= 27.577908	Long= -88.741369

OV211: Lat= 27.203929         Long=-89.046738

Small particles concentrations were low at all stations. There was no evidence of a subsurface plume based on the CTD fluorometer trace at any of the four stations.

The fluorescence intensity ratios were between 0.97 and 1.41 for the four sampling stations.

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations OV208 to OV211.