R/V Ronald H. Brown METADATA - 2003

Class of Data:  Surface ocean and atmospheric carbon dioxide concentrations

Dataset Identifier:  R/V Ronald H. Brown

One File:  RHB2003

Statement of how to cite dataset:

Ron Brown website: http://www.aoml.noaa.gov/ocd/gcc/rvbrown_data2003.php

These data are made freely available to the public and the scientific community

in the belief that their wide dissemination will lead to greater understanding

and new scientific insights.  The availability of these data does not

constitute publication of the data.  We rely on the ethics and integrity of

the user to assure that AOML receives fair credit for our work.  Please send

manuscripts using this data to AOML for review before they are submitted

for publication so we can insure that the quality and limitations of the data

are accurately represented.

Measurement platform identifier:  NOAA research vessel Ronald H. Brown (R104)

Cruise Information:

The Ron Brown conducted 9 major cruises in the Atlantic and eastern Pacific

Oceans for a total of 11 legs.

Project Information:

The system was operated by personnel from AOML or PMEL (Pacific Marine 	Environmental Laboratory) or by the Ron Brown’s Chief Survey Tech, Jonathan 	Shannahoff.  The work was sponsored by the Underway pCO2 on Ships project of the 	NOAA climate program.

Scientist responsible for technical quality of dataset:

Rik Wanninkhof

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Rik.Wanninkhof@noaa.gov

Contact person for this dataset:

Bob Castle

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Robert.Castle@noaa.gov

Timestamp for initial submission of dataset:  11/18/09

Timestamp for the most recent update of dataset:  11/18/09

Timestamp period the dataset refers to: 2/6/2003 – 11/21/2003

Geographic area the dataset refers to:

10 S to 65 N

115 W to 15 W

2003 Cruises:

RB200301 –	Western Boundary Time Series

Charleston, SC to Miami, FL

February 4, 2003 to February 15, 2003

Chief Scientist – Chris Meinen

Operator – Jonathan Shannahoff

RB200302 -	Puerto Rico Trench

Miami, FL to San Juan, Puerto Rico

February 19, 2003 to March 7, 2003

Chief Scientist – Uri ten Brink

Operator – Jonathan Shannahoff

RB200303 -	Kick’em Jenny Volcano

San Juan, Puerto Rico to Charleston, SC

March 10, 2003 to March 28, 2003

Chief Scientist – Haraldur Sigurdsson

Operator – Jonathan Shannahoff

RB200304T -	CO2/CLIVAR Transit

Charleston, SC to Reykjavik, Iceland

June 5, 2003 to June 14, 2003

Chief Scientist – Kevin Sullivan

Operator – Jonathan Shannahoff

RB200304A -	CO2/CLIVAR Leg A (WOCE A16N)

Reykjavik, Iceland to Funchal, Madeira

June 19, 2003 to July 10, 2003

Chief Scientist – John Bullister

Operator – Jonathan Shannahoff

RB200304B -	CO2/CLIVAR Leg B (WOCE A16N)

Funchal, Madeira, to Natal, Brazil

July 15, 2003 to August 10, 2003

Chief Scientist – John Bullister

Operator – Jonathan Shannahoff

RB200305 -	Puerto Rico Trench

St. Maarten, Netherlands Antilles to St. Petersburg, FL

August 28, 2003 to September 4, 2003

Chief Scientist – Uri ten Brink

Operator – Jonathan Shannahoff

RB200306 -	Harbor Branch Oceanographic Institution Bio-pharmaceuticals

St. Petersburg, FL to Panama City, FL

September 9, 2003 to September 19, 2003

Chief Scientist – John Reed

Operator – Jonathan Shannahoff

RB200307 -	Gulf of Mexico Deep Sea Habitats

Panama City, FL to Gulfport, MS

September 21, 2003 to October 1, 2003

Chief Scientist – George P. Schmahl

Operator – Jonathan Shannahoff

RB200308 -	U-166 Survey

Gulfport, MS to Pensacola, FL

October 6, 2003 to October 11, 2003

Chief Scientist – Robert Church

Operator – Jonathan Shannahoff

RB200309 -	TOGA/TAO

Balboa, Panama to Balboa, Panama

October 27, 2003 to November 21, 2003

Chief Scientist – Ben Morre

Operator – Jonathan Shannahoff

List of variables included in this dataset:

COLUMN	HEADER			EXPLANATION

1.		GROUP/SHIP:			AOML_Brown for all underway data from the Ron Brown.

2.		CRUISE_DESIGNATION:	Cruise ID (e.g., RBYYYYnn where RB = Ron Brown, YYYY

= the four digit year, and nn = the cruise number for

that year).

3.		JD_GMT:			Decimal year day.

4.		DATE_DDMMYYYY:		GMT date.  The date format has been changed to comply

with the IOCCP recommendations.

5.		TIME_HH:MM:SS:		GMT time.

6.		LAT_DEC_DEGREE:		Latitude in decimal degrees (negative values are in 							the southern hemisphere).

7.		LONG_DEC_DEGREE:		Longitude in decimal degrees (negative values are in 							the western hemisphere).

8.		xCO2W_PPM:			Mole fraction of CO2 (dry) in the equilibrator

headspace at equilibrator temperature (Teq) in parts

per million.

9.		xCO2A_PPM:			Mole fraction of CO2 in air in parts per million.

10.		PRES_EQUIL_hPa:		Barometric pressure in the lab in hectopascals (1

hectopascal = 1 millibar).

11.		PRES_SEALEVEL_hPa:	Barometric pressure corrected to sea level from the 							ship’s barometer in hectopascals (1 hectopascal = 1 							millibar).

12.		EqTEMP_C:			Temperature in equilibrator water in degrees

centigade.  Temperature in equilibrator measured with

a calibrated thermistor.

13.		SST(TSG)_C:			Temperature from the ship's thermosalinograph in

degrees centigrade.

14.		SAL(TSG)_PERMIL:		Salinity from the ship's thermosalinograph on the

Practical Salinity Scale.

15.		WATER_FLOW_L/MIN:		Water flow rate through the equilibrator in liters 							per minute.

16.		GAS_FLOW_IR_ML/MIN:	Gas flow through the sample cell of the Licor IR 							analyzer in milliliters per minute.

17.		TEMP_IR_C:			Temperature in the Licor sample cell in degrees 							centigrade.

18.		PRES_IR_hPa:		Barometric pressure in the lab in hectopascals (1

hectopascal = 1 millibar).  The Licor in this system 							does not include a pressure sensor so this field is 							the same as # 10 above.

19.		SHIP_HEADING_TRUE_DEGREE:	Ship’s heading in true degrees from the ship’s 								scientific computing system.

20.		SHIP_SPEED_KNOT:		Ship’s speed in knots from the ship’s scientific 							computing system.

21.		WIND_DIR_REL_DEGREE:	Relative wind direction in degrees from the ship’s 							scientific computing system.

22.		WIND_SPEED_REL_M/S:	Relative wind speed in meters per second from the 							ship’s scientific computing system.

23.		fCO2W@SST_uatm:		Fugacity of CO2 in sea water in microatmospheres

calculated as outlined below.

24.		QC_FLAG_WATER:		Quality control flag for fCO2W@SST measurement.  2 = 							good, 3 = questionable, 4 = bad.

25.		fCO2A_uATM:			Fugacity of CO2 in air in microatmospheres

calculated as outlined below.

24.		QC_FLAG_AIR:		Quality control flag for fCO2A measurement.  2 = 							good, 3 = questionable, 4 = bad.

27.		dfCO2_uATM:			Sea water fCO2 - air fCO2 in microatmospheres.  This

uses the average air value for the current hour.

28.		FLUORO_uG/l:		Measurement from the ship’s Turner 10AU fluorometer 							in micrograms per liter.

29.		WIND_SPEED_TRUE_M/S:	True wind speed in meters per second from the 								ship’s scientific computing system.

30.		WIND_DIR_TRUE_DEGREE:	True wind direction in degrees from the ship’s 								scientific computing system.

31.		AIR_TEMP_C:			Outside air temperature from the ship’s scientific 							computing system.

The following fields have been QC'ed by the CO2 group:

GROUP/SHIP

CRUISE_DESIGNATION

JD_GMT

DATE_DDMMYYYY

TIME_HH:MM:SS

LAT_DEC_DEGREE

LONG_DEC_DEGREE

xCO2W_PPM

xCO2A_PPM

EqTEMP_C

PRES_EQUIL_hPa

WATER_FLOW_L/MIN

GAS_FLOW_L/MIN

TEMP_IR_C

PRES_IR_hPa

fCO2W@SST_uatm

fCO2A_uATM

dfCO2_uatm

The following fields are from the ship's onboard systems and the quality of this

data cannot be verified:

SST(TSG)_C

Sal(TSG)_Permil

PRES_SEALEVEL_hPa

SHIP_HEADING_TRUE_DEGREE

SHIP_SPEED_KNOT

WIND_DIR_REL_DEGREE

WIND_SPEED_REL_M/S

FLUORO_uG/l

WIND_SPEED_TRUE_M/S

WIND_DIR_TRUE_DEGREE

AIR_TEMP_C

Narrative description of system design:

CO2 ANALYTICAL SYSTEM:

The concentration of carbon dioxide (CO2) in surface ocean water is determined

by measuring the concentration of CO2 in gas that is in contact with the water.

Surface water is pumped ~ 100 m through 7/8" Teflon tubing from an inlet

in the ship's bow to the equilibration chamber.  Water comes from the bow

intake ~4.2 m below the water line and the TSG is located close to the inlet.

When the SST is below about 20 oC, friction in the pipes and from the pump cause

heating and the Teq is higher than SST.  When the SST is higher than about 25 oC,

the ship’s air conditioning cools the water and the Teq is lower than SST.

The equilibration chamber has an enclosed volume of gas, or headspace, and a pool

of seawater that continuously overflows to a drain.  As the water flows through the

chamber, the dissolved gases (like CO2) partition between the water and the

headspace.  At equilibrium, the ratio of CO2 in the water and in the headspace is

influenced most by temperature, and that relationship is known.  By measuring

the concentration of CO2 in the headspace and the temperature in the chamber,

the partial pressure (or fugacity) of CO2 in the surface water can be calculated.

INSTRUMENT DESCRIPTION

The general principle of instrumental design can be found in Wanninkhof and Thoning

(1993), Ho et al. (1995), and Feely et al. (1999).  The concentration of CO2 in the

headspace gas is measured using the adsorption of infrared (IR) radiation, which

results from changes in the rotational and vibrational energy state of the CO2

molecule.  The LI-COR detector passes IR radiation through two 6" cells.  The

reference cell is flushed with a gas of known CO2 concentration.  The sample cell

is flushed with the headspace gas.  A vacuum-sealed, heated filament is the

broadband IR source.  The IR radiation alternates between the two cells via a

chopping shutter disc. An optical filter selects an adsorption band specific

for CO2 (4.26 micron) to reach the detector.  The solid state (lead selenide)

detector is kept at -12 degrees °C for excellent stability and low signal

noise (less than 0.2 ppm).

Several steps are taken to reduce interferences and to increase the accuracy

of the measurements.  After the equilibration chamber, the headspace travels

through a drying trap to remove water vapor.  During each analysis, the

headspace gas is compared to a reference gas of known concentration.  To

improve the accuracy of the measurements, three different gaseous standards

for CO2 are analyzed once an hour instead of the headspace gas.

Analyzer:  LI-COR 6251 (analog output) infrared (IR) analyzer.

Method of Analysis:  Differential analyses relative to the low standard.  Measures

dried equilibrator headspace gas.  Gas flow is stopped prior to IR readings.

Drying Method:  The equilibrator headspace sample gas first goes through a glass

condenser cooled to ~ 5 oC.  The sample and standard gases pass through a short

column of magnesium perchlorate before reaching the analyzer.

Equilibrator (setup, size, flows):  The equilibrator is based on a design by R.

Weiss and was fabricated from a plexiglass housing with ~8 L water reservoir and

~16 L gaseous headspace.  Water flow rate is ~11 L/min.  Headspace recirculation

rate is ~200 ml/min.

Additional sensors:

Thermistor mounted in the bottom of the equilibrator.

Setra Barometer Model 370

YSI Model 600R thermosalinograph with temperature, salinity, and dissolved

oxygen sensors.  This TSG is mounted in the Hydro lab sink near the

equilibrator and the two are teed off the uncontaminated seawater feed.

The dissolved oxygen measurements are not reported in the final data file.

Narrative statement identifying measurement method for each required parameter:

CALCULATIONS:

The mixing ratios of ambient air and equilibrated headspace air are calculated

by fitting a second-order polynomial through the hourly averaged millivolt

response of the detector versus mixing ratios of the standards.  Mixing ratios

of dried equilibrated headspace and air are converted to fugacity of CO2 in

surface seawater and water saturated air in order to determine the fCO2.

For ambient air and equilibrator headspace, the fCO2a (or fCO2eq) is calculated

assuming 100% water vapor content:

fCO2eq = xCO2eq(P-pH2O)exp(B11+2*d12)P/RT

where fCO2eq is the fugacity in the equilibrator, pH2O is the water vapor

pressure at the sea surface temperature, P is the atmospheric pressure (in atm),

T is the SST or equilibrator temperature (in K) and R is the ideal gas constant

(82.057 cm^3·atm·deg^-1·mol^-1).  The exponential term is the fugacity correction

where B11 is the second virial coefficient of pure CO2

B11 = -1636.75 + 12.0408T - 0.032795T^2 + 3.16528E-5 T^3

and d12 = 57.7 - 0.118 T is the correction for an air-CO2 mixture in units of

cm^3·mol^-1 (Weiss, 1974).

The calculation for the fugacity at SST involves a temperature correction term

for the increase of fCO2 due to heating of the water from passing through the

pump and through 5 cm ID PVC tubing within the ship.  The empirical temperature

correction from equilibrator temperature to SST is:

fCO2(SST) = fCO2(eq) /

Exp ((Teq-SST) * [0.03107 – 2.7851E-4 * Teq – 1.8391E-3 * ln(fco2eq * 1.0E-6)])

where SST is sea surface temperature and Teq is the equilibrator temperature in

degrees °C.

Sampling Cycle:

The system runs on an hourly cycle during which 3 standard gases, 3 air

samples from the bow tower and 8 surface water samples (from the

equilibrator head space) are analyzed on the following schedule:

Mins. after hour		Sample

4 				Low Standard

8 				Mid Standard

12 				High Standard

16.5				Water

21 				Water

25.5				Water

30				Water

34				Air

38				Air

42				Air

46.5				Water

51				Water

55.5				Water

60				Water

NOTES ON DATA:

Columns have a default value of –999.99 in case of instrument malfunction,

erroneous readings or missing data.  Furthermore, if a suspicious xCO2 value,

pressure or temperature value is encountered, the fCO2 is not calculated.

Analytical Instrument Manufacturer/Model:

The Ron Brown system (version 2.6) was built by Craig Neill in 1999.  The analyzer 	is a LI-COR 6251 (analog output) infrared analyzer.

Standard Gases and Reference Gas:  The three standard gases came from CMDL

in Boulder and are directly traceable to the WMO scale.  While individual data

points above the high standard gas concentration or below the low standard gas

concentration may not be accurate, the general trends should be indicative of the

seawater chemistry.

Description of any additional environmental control:

The system is located in the Hydro Lab of the Ron Brown.  The room is

air-conditioned with little temperature fluctuation.

Resolution of measurement:

The resolution of the instrument is better than 0.1 ppm.

Estimated overall uncertainty of measurement:

The xCO2eq measurements are believed accurate to 0.1 ppm.  The

fCO2@SST measurements are believed to be precise to 0.2 ppm.

List of calibration gases used:

The standards used during the 2003 field season were:

STANDARD		TANK #	CONCENTRATION	VENDOR

Low			CC71632	295.37		ESRL

Mid			CC91287	357.18		ESRL

High			CC71655	423.28		ESRL

Traceability to an internationally recognized scale (including date/place of last calibration made):

All standards are obtained from NOAA/CMDL, now called the Global

Monitoring Division of the Earth System Research Laboratory and are directly

traceable to WMO scale.

Uncertainty of assigned value of each calibration gas:

The uncertainty based on pre and post cruise calibrations is less

than 0.05 ppm.

Pressure/Temperature/Salinity:

For information about the ship’s thermosalinograph or other sensors, contact Chief 	Survey Tech Jonathan Shannahoff at jonathan.shannahoff@noaa.gov.

Units:

All xCO2 values are reported in parts per million (ppm) and fCO2 values

are reported in microatmospheres (uatm) assuming 100% humidity at the

equilibrator temperature.

Bibliography:

DOE (1994). Handbook of methods for the analysis of the various parameters of the

carbon dioxide system in sea water; version 2. DOE.

Feely, R. A., R. Wanninkhof, H. B. Milburn, C. E. Cosca, M. Stapp and P. P. Murphy

(1998). A new automated underway system for making high precision pCO2

measurements onboard research ships. Analytica Chim. Acta 377: 185-191.

Ho, D. T., R. Wanninkhof, J. Masters, R. A. Feely and C. E. Cosca (1997).

Measurement of underway fCO2 in the Eastern Equatorial Pacific on NOAA

ships BALDRIGE and DISCOVERER, NOAA data report ERL AOML-30, 52 pp.,

NTIS Springfield.

Wanninkhof, R. and K. Thoning (1993). Measurement of fugacity of CO2 in surface

water using continuous and discrete sampling methods. Mar. Chem. 44(2-4):

189-205.

Weiss, R. F. (1970). The solubility of nitrogen, oxygen and argon in water and

seawater. Deep-Sea Research 17: 721-735.

Weiss, R. F. (1974). Carbon dioxide in water and seawater: the solubility of

a non-ideal gas. Mar. Chem. 2: 203-215.

Weiss, R. F., R. A. Jahnke and C. D. Keeling (1982). Seasonal effects of

temperature and salinity on the partial pressure of CO2 in seawater.

Nature 300: 511-513.

Comments related to all 2003 data:

1.	xCO2 values outside the range of the standard gases (i.e. below the low standard or	above the high standard) are not as accurate as values within the range.  However,	the general trends should be indicative of the seawater chemistry.

2.	The standard gases for all cruises were 295.37 ppm, 	357.18 ppm, and 423.28 	ppm.

3.	Beginning with the second cruise (RB200302) additional data fields were reported.  		These data come from the ship’s various meteorological sensors and include wind 	speed and direction (both true and relative), air temperature, fluorometer, 	barometric pressure corrected to sea level, course over ground (COG), and speed 	over 	ground (SOG).  In addition, water and gas flow through the instrument are 	reported as well as QC flag fields for fCO2 water and air.  Flags are in the WOCE 	format with 2 = good, 3 = questionable, and 4 = bad.  No data was recorded for COG, 	SOG, and air temp during this year.  Beginning with cruise RB200306 and continuing 	until the end of the year, no data from the ship’s barometer was recorded.  All 	missing data has been set to –999.99.

Comments related to the individual legs:

RB200301:	1. The Licor was not turned on for the first 49 hours and no data was 				collected during this time period.

2. There was no water flow from Feb. 11 at 1720 to 1904 and from 2150 				on Feb. 11 to 0955 on Feb. 12.  All water values during these time 				periods were removed.

3. The salinity readings from the ship's TSG were bad from Feb. 14 at 				1804 to Feb. 15 at 1151 and were suspect for much of the rest of the 				cruise.  They have been replaced with the YSI salinity - 0.2 for the 				entire data set where 0.2 is the approximate offset between the two 				sensors.

RB200302:	1. The TSG temperature and salinity readings were "stuck" at a constant 			value on Feb. 20 from 18:38 to 18:52 and on Feb. 26 from 12:30 to 				15:15.  I did a linear interpolation from the last "non-stuck" value to 			the next "non-stuck" value.  The difference in fCO2 values should be 				much less than 1 hectopascal for these samples.

2. Some air values were removed because they were anomalously high or 				because of probable contamination by stack gas.  Some air values were 				slightly higher than the baseline of approximately 376 ppm and the high 			values could not be attributed to stack gas.  These values have been 				left in the data file and given a flag of 3 and are likely due to winds 			coming from the island of Puerto Rico.

RB200303:	1. The seawater flow was turned off three times: from March 21 at 20:40 			to March 22 at 19:40; from March 24 at 10:22 to March 24 at 18:52; and 				from March 25 at 17:36 to March 26 at 14:00.  No surface water 					measurements were included from these time periods.

2. From March 21 at 22:40 to March 22 at 15:40 all air values were 				removed due to probable contamination by stack gas.

RB200304T:	1. Seawater was turned off from June 7 @ 0950 to June 10 @ 2250 and 				from June 11 @ 1354 to June 12 @ 1720 (GMT) because no permission had 				been given to sample in Canadian and Danish waters respectively.

2. Early in the cruise air values that differed significantly from the 				baseline value of approximately 378 ppm were recorded.  I believe these 			are caused by air from the North American continent and are real, so I 				have left them in and given them a QC flag of 3.

RB200304A:	No problems of note.

RB200304B:	No problems of note.

RB200305:	1. The system was down from September 2 at 00:20 to September 3 at 				10:50 while passing through Cuban waters.

2. On September 1 and September 4, air values were measured that were 				above the normal baseline of approximately 372 ppm.  Most of these 				could not be attributed to stack gas contamination and have been left 				in the data file with a QC flag of 3.

RB200306:	1. The survey tech who normally operates the system was not present on 				this cruise.  This apparently had no effect as no significant problems 				were found in the data.

RB200307:	1. The survey tech who normally operates the system was not present on 				this cruise.  This apparently had no effect as no significant problems 				were found in the data.

2. The sea water flow was turned off from September 22 at 15:15 to 				September 23 at 13:28.  Water values in this period have been removed 				and given a QC flag of 9.  Air values have been retained.

RB200308:	1. The survey tech who normally operates the system was not present on 				this cruise.  This apparently had no effect as no significant problems 				were found in the data.

RB200309:	No problems of note.