30 August 2010 - Brooks McCall Status Report - Cruise 20 - Day 1

Compiled by: Michael Taylor – TDI Brooks International (for BP)

Day 1 of Cruise 20 continued tracking the subsurface plume signals as part of Operation Clean Sweep.  The day was spent working along Transect 9, ~240km SSW of the wellhead. Weather was good throughout the day with calming seas (2-4ft.).  No surface oiling was observed.  As with recent cruises, on-board DO analysis was undertaken using the Hach modified Winkler titration method.

The 2 CTD casts completed today are summarized in the table, and shown in the figure, below.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM194 – T9S9 | Bearing 199deg., 240 km | No signal | - | No deviations at depth from trace. No DO drop or fluorescence increase. Intervals sampled based on historical depths |
| BM195 – T9S10 | Bearing 217o, 78 km | Trace | - | No deviations at depth from trace. No DO drop or fluorescence increase. Intervals sampled based on historical depths |
|  |  |  | - |  |
|  |  |  | - |  |

There was no indication of oil in the fluorometry data at any of the sites sampled, including at BM195 (T9S9) and BM196 (T9S10) where no increase was observed in the background fluorometry signal.  The change was not sufficiently strong to be regarded a weak signal, our lowest category for rating plume strength.  There was no significant DO drop.

A total of 10 samples were analyzed by Hach modified Winkler methods today. DO data have been submitted with the daily deliverables.  The 60 Hach results from Cruise 20 had a correlation of 0.99 with the in situ Seabird DO sensor.  This demonstrates that the Seabird DO meter continues to provide a very reliable in situ measure of DO.

| Brooks McCall Science Team, Cruise 20 |  |
| --- | --- |
| Taylor, Michael | TDI Brooks : Chief Scientist |
| Morales, Anthony | NOAA :Data Manager |
| Susdorf, Gary | ENTRIX : Sample collection |
| Ascough, Rich | ENTRIX : Sample collection |
| Groen, Arie | Shaw: DO |
| Desphande, Ashok | NOAA: Observer |
| Schubauer-Berigan, Joe | EPA: Observer |

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."