01 September, 2010 - Brooks McCall Status Report - Cruise 20 - Day 2

Compiled by: Michael Taylor – TDI Brooks International (for BP)

Day 3 of Cruise 20 continued tracking the subsurface plume signals as part of Operation Clean Sweep.  The day was spent working along Transect 10, Transect 11 ~208 km of the wellhead. Weather was good throughout the day with ground swells (3-5ft.).  No surface oiling was observed.  As with recent cruises, on-board DO analysis was undertaken using the Hach modified Winkler titration method.

The 4 CTD casts completed today are summarized in the table, and shown in the figure, below.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM199– T11- S11 | Bearing 225 deg, 208 km | No signal | - | Moderate DO sag noted between approximately 1000m to 1090m with more pronounced DO sag noted at 1060m |
| BM200 – T11- S10 | Bearing 230 deg, 198 km | No signal | - | Slight DO sag noted between 1000m to 1100m with no distinct DO sags observed in this interval |
| BM 201 – T11- S9 | Bearing 232 deg, 193 km | No signal | - | DO sag noted between 800m and 880m with a slight deflection noted at 1030m |
| BM 202 – T10- S9 | Bearing 230 deg, 180 km | No signal | - | A slight DO decline noted within 840m to 910m interval. No prominent DO decreases noted within this interval |

There was no indication of oil in the fluorometry data at any of the sites sampled, including at BM199 (T11S11), BM 200 (T11S10), BM 201 (T11S9) and BM 202 (T10S9) where no increase was observed in the background fluorometry signal.  The change was not sufficiently strong to be regarded a weak signal, our lowest category for rating plume strength.

A total of 10 samples were analyzed by Hach modified Winkler methods today. DO data have been submitted with the daily deliverables.  The 60 Hach results from Cruise 20 had a correlation of 0.99 with the in situ Seabird DO sensor.  This demonstrates that the Seabird DO meter continues to provide a very reliable in situ measure of DO.

| Brooks McCall Science Team, Cruise 20 |  |
| --- | --- |
| Taylor, Michael | TDI Brooks : Chief Scientist |
| Morales, Anthony | NOAA :Data Manager |
| Susdorf, Gary | ENTRIX : Sample collection |
| Ascough, Rich | ENTRIX : Sample collection |
| Groen, Arie | Shaw: DO |
| Desphande, Ashok | NOAA: Observer |
| Schubauer-Berigan, Joe | EPA: Observer |

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."