03 September, 2010 - Brooks McCall Status Report - Cruise 21 - Day 2

Compiled by: Oscar Garcia – TDI Brooks International (for BP)

Day 2 of Cruise 21 continued tracking the subsurface plume signals as part of Operation Clean Sweep.  The day was spent transiting towards Transect 28 and working on the first two stations S07 and S08.. Weather was good throughout the day with ground swells (3-5ft.).  No surface oiling was observed.  As with recent cruises, on-board DO analysis was undertaken using the Hach modified Winkler titration method.

The 4 CTD casts completed today are summarized in the table, and shown in the figure, below.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM203– T28- S07 | Bearing 247 deg, 433 km | No signal | - | No distinct DO sags observed in this station |
| BM203– T28- S08 | Bearing 246 deg, 434 km | No signal | - | No distinct DO sags observed in this station |

There was no indication of oil in the fluorometry data at any of the sites sampled, including at BM203 (T28S07), and BM 204 (T28S08) where no increase was observed in the background fluorometry signal.  The change was not sufficiently strong to be regarded a weak signal, our lowest category for rating plume strength.

A total of 8 samples were analyzed by Hach modified Winkler methods today. DO data have been submitted with the daily deliverables.

| Brooks McCall Science Team, Cruise 21 |  |
| --- | --- |
| Garcia, Oscar | TDI Brooks : Chief Scientist |
| Morales, Anthony | NOAA :Data Manager |
| Susdorf, Gary | ENTRIX : Sample collection |
| Ascough, Rich | ENTRIX : Sample collection |
| Groen, Arie | Shaw: DO |
| Desphande, Ashok | NOAA: Observer |
| Schubauer-Berigan, Joe | EPA: Observer |

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."