05 September, 2010 - Brooks McCall Status Report - Cruise 21 - Day 3

Compiled by: Oscar Garcia – TDI Brooks International (for BP)

Day 4 of Cruise 21 continued tracking the subsurface plume signals as part of Operation Clean Sweep.  The day was spent working on four stations in transect T28 (S12, S13, S14).  Weather was rough all day. Ground swells (3-5ft.).  No surface oiling was observed.  As with recent cruises, on-board DO analysis was undertaken using the Hach modified Winkler titration method. Due weather conditions personnel onboard was sea-sick.

The 3 CTD casts completed today are summarized in the table, and shown in the figure, below.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM209– T28- S12 | Bearing 241 deg, 445 km | No signal | 1150 | Distinctive DO sags observed in this station |
| BM210– T28- S13 | Bearing 240 deg, 448 km | No signal | 1150 | Distinctive DO sags observed in this station |
| BM211– T28- S14 | Bearing 239 deg, 450 km | No signal | - | No distinct DO sags observed in this station |

There was no indication of oil in the fluorometry data at any of the sites sampled, including at BM209 (T28S12),  BM 210 (T28S13), and BM211 (T28S14) where no increase was observed in the background fluorometry signal.  However distinctive DO anomaly levels where observed on BM209 (T28S12) and  BM 210 (T28S13) at 1150m water depth.

A total of 8 samples were analyzed by Hach modified Winkler methods today. DO data have been submitted with the daily deliverables.

| Brooks McCall Science Team, Cruise 20 |  |
| --- | --- |
| Garcia, Oscar | TDI Brooks : Chief Scientist |
| Morales, Anthony | NOAA :Data Manager |
| Susdorf, Gary | ENTRIX : Sample collection |
| Ascough, Rich | ENTRIX : Sample collection |
| Groen, Arie | Shaw: DO |
| Desphande, Ashok | NOAA: Observer |
| Schubauer-Berigan, Joe | EPA: Observer |

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."