GEK FORMAT  (from Accession 8900142 submission)

Positions       Usage

---------       -----------------------------------------------

1 - 2          Country Code  ( US NODC)

3 - 4          Ship Code     (Japan Ocean Data Center ship code )

5 - 6          Latitude-degrees

7 - 9          Latitude-minutes  (xx.x)

10          Hemisphere ( N or S)

11 - 13          Longitude-degrees

14 - 16          Longitude-minutes  (xx.x)

17          Hemisphere ( E or W)

18 - 20          Marsden Square

21 - 26          Date   (YYMMDD) GMT

27 - 29          Hour (xx.x) GMT

30 - 34          Station Number

35 - 38          Depth (meters)

39 - 41          Current Direction  (degrees)

42 - 43          Current Velocity (x.x) knots

44 - 46          Surface Temperature (xx.x) Degrees  C.

47 - 48          Wind Drection -degrees

49 - 50          Wind Speed/Wind Force

51 - 54          Station Number

55 - 59          Blank

60          Drift  Code: blank = Drifter;  1 = GEK

61          Blank

62          Project Name  Code (See Table)

63 - 66          North Current Component  (63 = + or -) knots

( Blank or "+" = North; "-"  = South)

67 - 70          East Current Component  (67 = + or -) knots

( Blank or "+" = East; "-"  = West)

71 - 76          Reference ID Number

77 - 80          Consecutive Number

81 - 84          Square Number (Geographic Area)

81 - 82  = One-degree square

83  = 30-minute square

84 = 15-minute square

_________________________________________________________________________

Project Name Table:

D = DNP (Declared National Program)

C = CSK (Cooperative Study of the Kurishio Current)

K = KER

W = WESTPAC (Western Pacfic Study)

F = FGGE (First Garp Global Experiment)

B = Biomass

___________________________________________________________________________