The data on this disk are in the following format, following specifications

in the NODC User's Guide:

1         2         3         4         5         6

123456789012345678901234567890123456789012345678901234567890123456789

X QDDMMXNPDDDMMXEPYYMMDDHHMMP                         XXXXXXXXYYYY

X 740204N1136521W188071600  3                         3EZG5  1   1

1         1

7         8         9         0         1

01234567890123456789012345678901234567890

14 0532FF              PPPP DDDDTTTTDDDDTTTTDDDDTTTT...

14 053204                15    41620  101589  261569...

01   01   ALWAYS X

02   02   ALWAYS BLANK

03   03   QUADRANT

04   10   LATITUDE (D=Degrees, M=Minutes, X=tenths, N=North or South,

P=Precision: 1 => nearest minute or better

11   18   LONGITUDE (D, M, X, P as above; E=East or West)

19   24   DATE (Y=Year, M=Month, D=Day)

24   29   TIME (H=Hour, M=Minute, P=Precision: 3 => nearest hour)

30   54   (Left blank for use by NODC)

55   62   ORIGINATOR'S CRUISE NUMBER (callsign + sequential cruise number)

63   66   CONSECUTIVE NUMBER

67   72   (Left blank -- for paper chart data only)

73   73   INSTRUMENT TYPE (1 => Shipboard XBT)

74   74   GRID CODE (4 => 450 meter probe reporting in meters & deg. C.)

75   75   HIT BOTTOM (blank => no bottom contact)

76   77   DIGITIZATION METHOD (05 => direct digital output)

78   79   DIGITIZATION INTERVAL (32=> variable -- statistically determined)

80   81   TREATMENT AND STORAGE (02-06 => data compression with fit within

02=0.05; 03=0.1; 04=0.2; 05=0.3; 06=0.7 degrees C.)

82   85   BOTTOM DEPTH (blank => unknown)

86   95   (Left blank -- for MBT data only)

96   99   NUMBER OF DATA PAIRS (right justified in field)

100  100   BLANK

101  104   DEPTH IN WHOLE METERS

105  108   TEMPERATURE IN HUNDREDTHS OF A DEGREE CENTIGRADE

(Depth and temperature pairs fill out the rest of the record

ending with a carriage return-linefeed pair (ASCII 13 & 10))

Any questions, requests for alterations, complaints, notification of errors,

etc. should be addressed to Norman Hall, NODC SW Liaison Office.

Phone (619) 456-7110

FTS  893-7110