NODC ACCESSION NUMBER:  9300076

Draft Data Report 2

Tier 2 Permittee Monitoring:

Expendable Current Profiler  (XCP)

Measurements

at the 106-Mile Site

December 1990 through June 1991

EPA Contract No. 68-C8-0105

September 3, 1991

Submitted to

Environmental Protection Agency

Office of Wetlands, Oceans and Watersheds

Prepared by

Battelle Ocean Sciences

397 Washington Street

Duxbury, MA  02332

(617) 934-0571

This is minimal documentation extracted by NODC from this report

to accompany the data.

Data parameters are, in order: XCP #, depth (meters), temperature

(degrees celsius), u velocity component, v velocity component and

velocity error.

###########################################

XCP DATA SUMMARY

###########################################

XCP No.   Data Rvcd   Date Deployed  Serial No.  Data Quality

-------   ---------   -------------  ----------  --------------

30         12/17/90    12/01/90      89061054    poor (25 %)

31         12/26/90    12/07/90      89061019    poor (45 %)

32         12/26/90    01/15/91      90021007    poor (0 %)

33         01/28/91    01/19/91      89101067    fair (70 %)

34         02/01/91    01/26/91      89101056    good (94 %)

35         02/11/91    02/02/91      89101039    no data (*)

36         02/20/91    02/07/91      89091123    poor ( 23 %0

37         03/06/91    02/18/91      89091082    poor (23 %)

38         03/21/91    03/13/91      90121067    no data (**)

39a        05/01/91    04/12/91      90121118    fair (63 %)

40a        05/01/91    04/19/91      90111020    fair (69 %)

41a        05/06/91    04/25/91      90121115    good (95 %)

42a        05/05/91    05/02/91      90121112    good (96 %) (***)

43a        05/20/91    05/07/91      90121113    good (97 %)

44a        05/20/91    05/13/91      90121110    good (98 %)

45a        06/10/91    05/19/91      90121061    probe failure

46a        06/10/91    05/20/91      90121066    good (90 %)

47a        06/10/91    06/01/91      91051026    poor (1 %) (****)

-------------------------------------------------------

Note:  *   XCP No. 32 was processed using Velerr value of 10 cm/s.

**   XCP No. 35 and XCP No. 38 tapes contained no data.

***   XCP No. 42a profile has no data below 1150 m.

****   XCP No. 47a did not contain sufficient valid data points

to provide useful profiles.

----------------------------------------------------------

----------------------------------------------------------

XCP #        Date Deployed  Longitude   Latitude       Serial No.

(deg W)     (deg N)

::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

30         12/01/90     072 01.85'   38 59.13'      89061054

31         12/07/90     072 02.00'   38 59.80'      89061019

32         01/15/91     072 02.90'   39 00.00'      90021007

33         01/19/91     072 01.90'   38 57.50'      89101067

34         01/26/91     072 01.84'   38 59.93'      89101056

35         02/02/91     072 02.97'   38 59.79'      89101039

36         02/07/91     072 02.00'   39 00.00'      89291123

37         02/18/91     072 02.21'   38 59.45'      89091082

38         03/13/91     072 01.94'   39 00.00'      90121067

39a        04/12/91     072 03.90'   39 00.00'      90121118

40a        04/19/91     072 03.90'   39 00.00'      90111020

41a        04/25/91     072 04.00'   39 00.00'      90121115

42a        05/02/91     072 03.80'   39 59.90'      90121112

43a        05/07/91     072 03.90'   39 00.00'      90121113

44a        05/13/91     072 04.10'   39 00.10'      90121110

45a        05/19/91     072 04.00'   39 00.00'      90121061 *

46a        05/20/91     072 04.10'   39 00.00'      90121066

47a        06/01/91     072 03.90'   39 00.00'      91051026

::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

Note: XCPs suffixed by the letter "a" designate air-launched

probes.

*  Probe failure on XCP # 45a

----------------------------------------------------------------------

1.0  INTRODUCTION

This data report presents a summary of results for expendable current

profiler (XCP) measurements made at the 106-Mile Deepwater Municipal

Sludge Dump Site (106-Mile Site) during the period fro December 1990

through June 1991.   A previous data report (Battelle, 1991) presents a

summary of XCP measurement results for the period March through November

1990.  Ocean disposal of sewage sludge has been conducted at the 106-Mile

Site since 1986.  Monitoring activities of disposal operations, sludge

fate, and resultant effects have been performed under the Environmental

Protection Agency (EPA) Monitoring Plan for the 106-Mile Site (Battelle,

1988a,b).  In 1988, the Congress of the United States passed the Ocean

Dumping Ban Act (ODBA).  Under the ODBA, EPA, The National Oceanic and

Atmospheric Administration (NOAA), and the United States Coast Guard

(USCG) have developed a joint Monitoring, Research and Surveillance Plan

(joint Monitoring Plan) for the 106-Mile Site (EPA,1990). Under the the

joint Monitoring Plan, EPA is responsible for studies designed to evaluate

the farfield-fate (Tier III) of sewage sluge disposed at the 106-Mile

Site.  EPA also utilizes data produced by other agencies for some of the

data-interpretation tasks.

The 106-Mile Site farfield-fate studies focus on determining the transport

and fate of the sewage sludge, using a variety of physical oceanographic

techniques.  These include measuring vertical current shears with XCPs,

taking vertical temperature profiles with expendable bathythermographs

(XBT), determining currents with moored current-meter arrays (Eulerian

measurements), using satellite-tracked drifting buoys to determine water-

mass movement (Lagrangian measurements), and measuring se-surface

temperatures (SST) with imagery from satellite-based sensors.  Under the

Joint Monitoring Plan, each agency, as well as the permitees using the

106-Mile Site for sludge disposal are responsible for acquiring data via

these various oceanographic techniques.  This data report summarizes the

results of XCP measurements made at the 106-Mile Site during the period

from December 1990 through June 1991.  The final XCP deployment under the

presently defined study program for the 106-Mile Site occurred on June 1,

1991.   Comprehensive analyses and syntheses of results from the various

measurement programs will be addressed in reports prepared under separate

work assignments.

The XCP measurements reported here have been performed under Work

Assignments 1-07 and 2-07 to the EPA Office of Marine and Estuarine

Protection (in April 1991, the Office of Marine and Estuarine Protection

became part of the new EPA Office of Wetlands, Oceans, and Watersheds).

These work assignments also include XBT measurements made at the 106-Mile

Site; the results for the XBT measurements are summarized in separate

reports.

2.0  XCP DEPLOYMENT SUMMARY

Under the permits issued for sludge disposal at the 106-Mile Site, the

sewage authorities holding the permits (permittees) were required to

deploy an XCP at the Site on an approximate weekly basis .  The permittees

were required to complete a deployment log (Figure 1) for each deployment

and to send the resultant data tape and log sheet to Batttelle for

processing and archiving and analysis.

The XCP system utilized by the permittees is manufactured by Sippican, Inc.

(Marion, Massachusetts).  The XCP system comprises an XCP probe and surface

float with radio transmitter, launched from the sludge barge, and a

receiver/recorder unit that was aboard the barge tow vessel.  An antenna

aboard the tow vessel was connected via coaxial cable cable to the receiver

/recorder unit.  Shortly after entering the water during launch (approx.

40 seconds) the XCP probe separates from the free-floating surface buoy

and descends at a known rate through the water column.

Each XCP probe contains sensors for measuring water temperature and

orthogonal electrical potentials proportional to the current-velocity

changes with depth (current shears).  Magnetic sensors are also incorp-

orated into the probes, so that the orientation of the relative current

velocity can be determined relative to the earth's magnetic field.

The sensor measurements are telemetered to the surface buoy via thin,

coated wires (dual magnet wire), which are dereeled from spools located

in both the probe and the surface buoy.  This dual-spool arrangement

permits the probe to descend smoothly, with the rate of descent being

governed by the probe's predetermined terminal velocity through the water

column.  The depth of the probe at any instant is determined via a quadratic

equation dependent on the elapsed time of descent.  Sensor data were

telemetered from the surface buoy via a radio frequency (RF) signal to

the receiving antenna located on the tow vessel.  The tow vessel carried

an antenna, radio receiver, and a tape recorder unit used to acquire a

cassette tape copy of the sensor data from the XCP probe.   The data tape

and deployment log sheet were then mailed to Battelle after the tow vessel

had returned to port.

The training of permittee technicians in the installation and operation

of the XCP system was performed by Clearwater Consultants (Newton, Mass.).

A single XCP shipboard system (antenna/receiver/recorder) was provide for

the XCP program, necessitating that the system be transportable since

different tow vessels were utilized by the permittees.  Four of the tow

vessels used by the permittees were outfitted with permanent Yagi antennas

in late spring 1990, and three other vessels were outfitted with permanent

Yagi antennas in July 1990.   Tow vessels without permanent Yagi antennas

used a portable whip antenna that always accompanied the shipboard

receiver/recorder unit.   The XCP probes are self-contained units, and were

carried aboard the sludge barges, as needed, for launching at the 106-Mile

site.

Note than in April 1991 it was decided by EPA Region II to end the program

of XCP deployment from the permittee barges, and to begin weekly XCP

deployments via aircraft overflights of the 106-Mile site.   XCPs were

launched from the overflying aircraft utilizing a small drogue parachute

attached to XCP probe/surface buoy package.  Upon entering the water,

this small parachute detached from the deployment package, and the XCP

probe began its normal detachment process from the surface buoy leading

to the descent of the probe through the water column to obtain the XCP

profile.   After launching the XCP package, the deployment aircraft then

circled the launch point to receive and record the RF signal from the

XCP surface buoy.   This airlaunch XCP program was initiated in an attempt

to eliminate intermittent noise spiking problems that were contaminating

some of the XCP data profiles (discussed in Section 4.0).  The deployment

aircraft was configured with an antenna and an XCP receiver/recorder unit.

The aircraft-launched XCP program continued until June 1, 1991, when the

XCP deployment program was formally ended.   The XCP deployments performed

during the period December 1990 through June 1991 are summarized in

Table 1.   XCP deployments which were accomplished via aircraft overflights

are suffixed by the letter "a" in the XCP No.

3.0 XCP DATA PROCESSING

Upon receipt at Battelle's Duxbury, Massachusetts, facility, the XCP data

tapes and accompanying deployment log sheet were logged in by Battelle

personnel prior to processing.   A cassette-tape player and Sippican

model MK 10 digital interface were utilized with a PC to translate and

process the XCP data.   Software provided by Sippican was installed on

the PC for processing the data.  Following digitization and transfer of the

data to the PC, a header record was created by using information provided

on the accompanying deployment log sheet, such as the date, time, and

position of deployment, the tow-vessel name, cruise number, XCP ID

number, and additional information related to the operation of the XCP

probe.   The header record was merged with digital XCP data file and was

retained with the data file through all subsequent processing and data

archiving.

Processing software was then utilized to convert the raw digital XCP data

into nine variables: depth, temperature, east and north components of

velocity, and five technical parameters related to probe performance.

In addition, the header record was expanded to include the parameters

used during the postprocessing function to create the processed data

file.  The expanded header record and processed data variables were then

used as the archived data file.