#DOCUMENTATION_FILE_NAME:Natsushima_CTD_ddf \

#NODC_ACCESSION_NUMBER: \

#NODC_REFERENCE_NUMBER: \

#NODC_DOCUMENTATION_DATE:March 31, 1994 \

#NODC_DOCUMENTOR:Brenda Humphries, Harry Iredale \

#OTHER_DOCUMENTORS: Brigitte Baeuerle of TOGA COARE Internatinal Program

Office and the principal inventigator provided most inforamtion. \

#DISTRIBUTION_RESTRICTION:none \

#DATE_RECEIVED: March 17, 1994 \

#SUBMISSION_MEDIUM:ftp \

#SUBMITTOR_NAME:Kentaro Ando \

#SUBMITTOR_INSTITUTION:JAMSTEC,Tropical Ocean Climate Study Group

Ocean Research Department

#SUBMITTER_STREET_ADDRESS:2-15, Natsushima \

#SUBMITTER_CITY:Yokosuka, Kanagawa 237 \

#SUBMITTER_COUNTRY:JAPAN \

#SUMITTER_TELEPHONE_NO:(81) 468 66 3811 \

#SUBMITTER_INTERNET:anouk@mstkid.jamstec.go.jp \

#SUBMITTER_EMAIL:jamstec.res (Omnet) \

#COLLECTION_INFORMATION:

The R/V Natsushima participated in the TOGA COARE Intensive Observing

Period from 01 Feb 93 - 18 Feb 93 (Pohnpei to Rabaul) and from

20 Feb 93 - 07 Mar 93 (Rabaul to Guam).  During the first leg, the

vessel maintained a relatively stationary position at 0 156E and

tended moorings at 2N 156E, 0 154E and 2S 156E.  During the second leg,

moorings at 0 147E, 2N 147E and 5N 147E were tended.

CTD observations were performed every 6 hours while on station (O 156E)

and every 1 degree of latitude along the ship's track.  A total of 60

casts was made.

Filename           Date        Time   Latitude   Longitude

N9301C01_1M_V7.DAT 04 FEB 1993 08:10  01 39.80 S 155 52.80 E

N9301C02_1M_V7.DAT 05 FEB 1993 00:30  00 00.13 S 156 00.43 E

N9301C03_1M_V7.DAT 05 FEB 1993 07:15  00 00.01 N 156 00.00 E

N9301C04_1M_V7.DAT 05 FEB 1993 11:39  00 00.37 S 156 00.14 E

N9301C05_1M_V7.DAT 05 FEB 1993 17:32  00 00.08 S 155 59.80 E

N9301C06_1M_V7.DAT 05 FEB 1993 23:10  00 00.08 S 156 00.17 E

N9301C07_1M_V7.DAT 06 FEB 1993 05:31  00 00.07 N 156 00.21 E

N9301C08_1M_V7.DAT 06 FEB 1993 11:33  00 00.12 S 156 00.10 E

N9301C09_1M_V7.DAT 06 FEB 1993 17:25  00 00.05 N 155 59.94 E

N9301C10_1M_V7.DAT 06 FEB 1993 23:32  00 00.06 S 156 00.02 E

N9301C11_1M_V7.DAT 07 FEB 1993 05:25  00 00.03 S 156 00.09 E

N9301C12_1M_V7.DAT 07 FEB 1993 11:29  00 00.11 S 156 00.06 E

N9301C13_1M_V7.DAT 07 FEB 1993 17:21  00 00.20 N 156 00.05 E

N9301C14_1M_V7.DAT 07 FEB 1993 23:00  00 00.02 S 155 56.14 E

N9301C15_1M_V7.DAT 08 FEB 1993 05:31  00 00.01 S 155 56.09 E

N9301C16_1M_V7.DAT 08 FEB 1993 11:54  00 00.00 N 155 55.99 E

N9301C17_1M_V7.DAT 08 FEB 1993 17:41  00 00.10 N 155 59.04 E

N9301C18_1M_V7.DAT 08 FEB 1993 23:26  00 00.07 N 155 59.03 E

N9301C19_1M_V7.DAT 09 FEB 1993 05:25  00 00.00 N 155 59.00 E

N9301C20_1M_V7.DAT 09 FEB 1993 11:22  00 00.00 S 155 58.85 E

N9301C21_1M_V7.DAT 09 FEB 1993 17:25  00 00.11 N 155 58.87 E

N9301C22_1M_V7.DAT 09 FEB 1993 23:28  00 00.05 N 155 58.94 E

N9301C23_1M_V7.DAT 10 FEB 1993 05:32  00 00.00 N 155 59.32 E

N9301C24_1M_V7.DAT 10 FEB 1993 11:26  00 00.01 S 155 59.01 E

N9301C25_1M_V7.DAT 10 FEB 1993 17:24  00 00.03 N 155 59.13 E

N9301C26_1M_V7.DAT 10 FEB 1993 22:55  00 00.05 N 155 58.89 E

N9301C27_1M_V7.DAT 11 FEB 1993 05:25  00 00.09 S 155 58.89 E

N9301C28_1M_V7.DAT 11 FEB 1993 11:26  00 00.30 N 155 58.82 E

N9301C29_1M_V7.DAT 11 FEB 1993 17:25  00 00.20 N 155 58.95 E

N9301C30_1M_V7.DAT 11 FEB 1993 23:25  00 00.02 N 155 58.79 E

N9301C31_1M_V7.DAT 12 FEB 1993 05:23  00 00.00 N 155 58.93 E

N9301C32_1M_V7.DAT 12 FEB 1993 11:26  00 00.02 N 155 59.03 E

N9301C33_1M_V7.DAT 12 FEB 1993 17:25  00 00.01 N 155 58.87 E

N9301C34_1M_V7.DAT 12 FEB 1993 23:25  00 00.10 N 155 58.87 E

N9301C35_1M_V7.DAT 13 FEB 1993 05:26  00 00.10 N 155 58.78 E

N9301C36_1M_V7.DAT 13 FEB 1993 11:30  00 00.06 N 155 59.05 E

N9301C37_1M_V7.DAT 13 FEB 1993 17:21  00 00.04 N 155 58.97 E

N9301C38_1M_V7.DAT 13 FEB 1993 23:26  00 00.05 S 155 58.80 E

N9301C39_1M_V7.DAT 14 FEB 1993 05:05  00 00.08 N 155 59.12 E

N9301C40_1M_V7.DAT 15 FEB 1993 00:05  01 59.92 N 155 56.94 E

N9301C41_1M_V7.DAT 15 FEB 1993 23:19  00 01.40 S 154 06.77 E

N9301C42_1M_V7.DAT 16 FEB 1993 22:42  01 59.86 S 156 00.05 E

N9301C43_1M_V7.DAT 17 FEB 1993 05:52  03 00.05 S 155 28.21 E

N9301C44_1M_V7.DAT 17 FEB 1993 12:40  03 59.93 S 154 54.25 E

N9301C45_1M_V7.DAT 17 FEB 1993 21:08  04 59.90 S 153 59.95 E

N9301C46_1M_V7.DAT 18 FEB 1993 02:14  05 00.10 S 153 10.88 E

N9301C47_1M_V7.DAT 22 FEB 1993 18:45  03 59.98 S 145 59.88 E

N9301C48_1M_V7.DAT 23 FEB 1993 01:01  03 00.11 S 146 00.08 E

N9301C49_1M_V7.DAT 23 FEB 1993 09:56  01 45.00 S 145 42.12 E

N9301C50_1M_V7.DAT 26 FEB 1993 13:13  01 00.07 S 147 00.09 E

N9301C51_1M_V7.DAT 26 FEB 1993 17:04  00 30.13 S 146 59.95 E

N9301C52_1M_V7.DAT 26 FEB 1993 20:54  00 00.11 S 147 00.01 E

N9301C53_1M_V7.DAT 27 FEB 1993 00:50  00 29.85 N 146 59.86 E

N9301C54_1M_V7.DAT 27 FEB 1993 04:40  00 59.92 N 146 59.93 E

N9301C55_1M_V7.DAT 27 FEB 1993 11:51  02 00.13 N 146 59.90 E

N9301C56_1M_V7.DAT 27 FEB 1993 18:54  02 59.93 N 146 59.88 E

N9301C57_1M_V7.DAT 28 FEB 1993 02:09  03 59.95 N 146 59.97 E

N9301C58_1M_V7.DAT 28 FEB 1993 09:15  04 59.97 N 147 00.10 E

N9301C59_1M_V7.DAT 28 FEB 1993 16:04  05 59.88 N 147 00.01 E

N9301C60_1M_V7.DAT 28 FEB 1993 22:44  07 00.04 N 146 59.95 E

\

#PRINCIPAL_INVESTIGATOR_NAME:Kentaro Ando \

#PI_INSTITUTION:JAMSTEC, Tropical Ocean Research Group

Ocean Research Department \

#PI_STREET-ADDRESS:2-15, Natsushima \

#PI_CITY:Yokosuka, Kanagawa 237 \

#PI_COUNTRY:JAPAN \

#PI_TELEPHONE_NO:(81) 468 66 3811 \

#PI_INTERNET:andouk@mstkid.jamstec.go.jp \

#PI_EMAIL:jamstec.res (Omnet) \

#PROJECT:TOGA COARE - JAPACS (Japanese Pacific Climate Study) \

#FUNDING AGENCY:Japan Science and Technology Agency \

#GRANT/CONTRACT-NO: \

#PLATFORM_TYPE:research vessel \

#PLATFORM_NAME:R/V Natsushima \

#COLLECTION_METHODS:

CTD profiles were made down to a depth of 1,000 meters, with the CTD

being operated from the left deck of the R/V Natsushima.  The data

were collected at 24 Hz and averaged to 6 Hz.  Data were recorded on

an IBM computer (IBM/Think Pad) using Seasoft software version 3,

provided by SBE.

On some stations, ten Nansen bottles were attached to the wire

to sample water for comparison with the CTD data. \

#ANALYSIS_METHODS:

The CTD sensors were calibrated before and after the cruise by SBE.

The drift of the sensors was small.  The data were averaged into 1 meter

intervals, rejecting noise.  The data set contains noisy salinity data

in the surface layer (0-5 meters).  Salinity and temperature CTD data were

checked with data obtained from bottle samples. Their precision is

within 0.01 psu and 0.01 degree.  Oxygen data were not checked and

are the original values from CTD/DO.  \

#INSTRUMENTS:SBE9/17 Sea-Bird Electronics, Seattle WA

#PUBLICATIONS:Annual report of Japanese Pacific Climate Study edited by Japan Science

and Technology Agency in 1994 (in preparation) \

#ASSOCIATED_DATASETS: \

#ASSOCIATED_VERSIONS: \

##DATA_SET_INFORMATION: \

#DATA_SET_NAME:Natsushima CTD \

#DATA_SET_VOLUME:  4179150 \

#SOURCE_COMPUTER:Digital VAX \

#SOURCE_COMPUTER_OPERATING_SYSTEM:VMS \

#SOURCE_LANGUAGE: \

#COMPUTER_CODE:ASCII \

#ORIGINATOR_DATASET_IDENTIFIER:N9301 TOGA COARE \

#DATA_DATETS: 1 Nov 92 - 28 Nov 93 \

#LEFT_GEOGRAPHIC_UPPER_BOUND: 10N 140E \

#RIGHT_GEOGRAPHIC_LOWER_BOUND: 10S 180 \

#GEOGRAPHIC_REGION: Equatorial Pacific, north of Australia,

TOGA COARE Large Scale Domain, Western Pacific warm pool

#DATA_TYPE: CTD data, 60 profiles \

#PARAMETERS_DEPENDENT:

temperature, salinity, potential temp., density, potential density,

dynamic height, dissolved oxygen

#PARAMETERS_INDEPENDENT:  date, latitude, longitude,  depth

#FORMAT_DESCRIPTION:

ALL DATA ARE IN ASCII:

depth (m)

temp (C)

salinity (PSU)

potential temperature (C)

density (kg/m3)

potential density (kg/m3)

dynamic height (m2/s2)

Dissolved Oxygen (ml/l)

#PARAMETER_1:depth \

#UNITS_1:meters w/decimal \

#RESOLUTION_1:10-0 \

#PARAMETER_2:temperature \

#UNITS_2:degrees \

#SCALE_2: Centigrade \

#RESOLUTION:10-3 \

#PARAMETER_3: salinity \

#UNITS_3: PSU \

#RESOLUTION_3: 10-3 \

#PARAMETER_4: potential temperature \

#UNITS_4:degrees \

#SCALE_4:Centigrade \

#RESOLUTION_4:10-3 \

#PARAMETER_5: density \

#UNITS_5: kg/m3 \

#RESOLUTION_5:10-4 \

#PARAMETER_6:potential density \

#UNITS: kg/m3 \

#RESOLUTION: 10-4 \

#PARAMETER_7:dynamic height \

#UNITS_7: m2/s2 \

#RESOLUTION_7:10-3 \

#PARAMETER_8:dissolved oxygen \

#UNITS_8:ml/l \

#RESOLUTION_8:10-2 \

#FORMAT_PUBLICATION: \

#SUBMITTOR_DOCUMENTATION: \

#SAMPLE_DATA:

27 FEB 1993 04:40  00 59.92 N 146 59.93 E

depth  temp.   sal.  p-tem density pot-den  dy-ht    DO

(m)     (C)   (PSU)   (C)  (Kg/m3) (Kg/m3) (m2/s2) (ml/l)

1. 28.838 33.935 28.838 21.3251 21.3209  0.065   4.39

2. 28.830 33.852 28.830 21.2697 21.2614  0.131   3.90

3. 28.846 33.831 28.845 21.2529 21.2404  0.197   5.37

4. 28.844 34.224 28.843 21.5529 21.5361  0.260   4.42

5. 28.842 34.174 28.841 21.5203 21.4993  0.323   4.38

6. 28.844 34.203 28.843 21.5457 21.5205  0.386   4.25

7. 28.846 34.209 28.844 21.5538 21.5243  0.449   4.57

8. 28.847 34.209 28.845 21.5577 21.5242  0.513   4.62

9. 28.840 34.210 28.838 21.5651 21.5273  0.576   4.56

10. 28.827 34.213 28.825 21.5759 21.5339  0.639   4.56

11. 28.844 34.208 28.841 21.5709 21.5247  0.702   4.63

12. 28.817 34.217 28.814 21.5909 21.5405  0.765   4.43

13. 28.834 34.210 28.831 21.5842 21.5296  0.828   4.66

14. 28.829 34.214 28.826 21.5933 21.5344  0.891   4.42

15. 28.818 34.217 28.814 21.6034 21.5404  0.954   4.42

16. 28.786 34.225 28.782 21.6242 21.5570  1.017   4.42

17. 28.786 34.224 28.782 21.6279 21.5563  1.080   4.44

18. 28.778 34.227 28.774 21.6370 21.5613  1.143   4.43

\