Date: Wed, 22 Mar 1995 11:06:49 -0500

To: phadsell@nodc.noaa.gov

From: jsherman@nemo (jeff sherman)

Subject: ALACE description

Cc: jsherman, hall

To Phil Hadsell and Norm Hall,

DESCRIPTION OF THE ALACE PROFILER FOR NODC ARCHIVING

BRIEF INSTRUMENT DESCRIPTION

The ALACE (Autonomous LAgrangian Circulation Explorer) is a subsurface

drifter, periodically rising to the surface to relay data to ARGOS.

Instrument location is then obtained from ARGOS.  An ALACE profiler

collects data on ascent and relays a compressed data set to ARGOS.  The

amount of time spent at its neutrally-buoyant depth, and then at the

surface, is variable, dependent upon the deployment site and the main

scientific objective of the ALACE.  Profiling ALACEs generally complete a

cycle every 8-10 days, spending 24 hours at the surface transmitting to

ARGOS.

The ALACE's transmitted temperature profile has the following resolution:

1) T is resolved to ~0.01 deg. C, with accuracy of 0.02 deg. C.

2) Depth resolution is variable, anywhere from 5-50 m (see below).

Pressure sensor accuracy is ~10 m.  Maximum depth is variable between

instruments and also between profiles for the same instrument.

3) Profile location is obtained from the ARGOS fix, with an accuracy

of 2 km.

DEPTH-AVERAGING

A profile is taken on ascent, with T sampled every 3 s while ALACE is

ascending at 10-15 cm/s.  The data is depth-averaged and compressed for

transmission through ARGOS.  The degree of averaging increases with depth.

For instance, a  profile might start out with 5 m resolution from 0-200 m

depth, switching to 10 m resolution from 200-800 m, followed by 20 m

resolution to 1200 m.

SENSOR DESCRIPTION

TEMPERATURE:  A thermistor bead from YSI (YSI-44016) is used, having a

typical drift of <0.01 deg. C over 1 year, increasing to 0.02 deg. C over 8

years.

Laboratory calibration of thermistors typically have less than 0.002 deg. C

standard deviation.  The largest error comes from the 12-bit a/d (Linear

Technology's LTC1290) which resolves T to 0.01-0.02 deg C, with better

resolution obtained at low temperatures.

PRESSURE: A Paines 210-38 with 0-3000 PSI range is used.  Quoted accuracy

from thermal drift, non-linearity, hysteresis, and repeatability is ~10

dBar.  Observations of a few units in laboratory testing suggest an

accuracy of ~5 dBar.

OTHER PROFILING ALACEs:  At this time, there are a few CTD ALACEs.

Sampling and data format are the same as the temperature profiler, with the

addition of conductivity also being transmitted.  The units have not been

thoroughly tested for long-term accuracy, and thus their data sets are not

ready for release.

SUBSURFACE DRIFT RATE

The difference in locations between profiles includes the subsurface drift

and the 24 hour surface drift in one complete ALACE cycle.  Therefore the

NODC data set itself could be used to estimate deep currents, but accuracy

would be limited by the unkown surface drift.  Dr. Davis uses a fairly

sophisticated algorithm to extrapolate the surface drift from the set of

ARGOS fixes for the 24 hours ALACE is transmitting.

I believe Russ Davis's intent is to provide temperature profiles in the

same format as the NODC XBT data set, so people can readily use the ALACE

data without requiring any further programming.

If you have any further questions, let me know.

Jeff Sherman

jtsherman@ucsd.edu

(619) 534-9863