meta data

file t45merge.doc

project=JGOFS Arabian Sea

ship=Thomas Thompson

cruise=TN045 (Process cruise 2)

PI's Dr. Lou Codispoti

Dr. Frank Millero

Dr. Edward Peltzer

Dr. Dennis Hansell

Dr. Hugh Ducklow

----------------------------------------------------------------

Dr. Lou Codispoti, Old Dominion Univ.

Cast specific comments, quality assessment, analytical methods as prepared

by L. Codispoti

FOR THE HYDROGRAPHIC BOTTLE DATA

L.A. Codispoti (lou@ccpo.odu.edu)

Old Dominion University, November 1995

General Comments:

This "readme" file pertains to the salinity, dissolved oxygen, and

nutrient data taken from sampling bottles with the hydrographic

rosette that was typically equipped with 24 10-liter Niskin type

bottles during RV T.G. Thompson cruise TN045. This cruise was the

second JGOFS Arabian Sea Process Leg and took place during March-April

1995.  Dr. John Marra of the Lamont Doherty Earth Observatory

(marra@ldeo.columbia.edu) was the chief scientist. DATA TAKEN WITH THE

CLEAN ROSETTE USED FOR OBTAINING PRIMARY PRODUCTION AND OTHER TYPES OF

BIOLOGICAL SAMPLES ARE NOT INCLUDED IN THIS REPORT.

Some questionable data are not included in this report. These data are

still retained in files at Old Dominion University, and are available

upon request.

No units are given for salinity in this report because the most recent

definitions of salinity define it as a dimensionless number.  To

accommodate every preference, Winkler oxygen values are reported in

ml/l, micromolar and micromoles per kg.  The latter values can only be

calculated with a knowledge of the oxygen sample temperatures when the

samples were drawn. These "draw temperatures" are not reported here,

but can be obtained by contacting lou@ccpo.odu.edu.  Nutrient values

are reported in micromolar. They can be converted to micromoles per

kg, by combining lab. temperature on the Thompson (approx. 23.5 deg C)

and the salinity of the sample to compute density and then dividing

the value in micromolar by this number.

Methods:

In general, the methods employed for the bottle salinity, Winkler

dissolved oxygen, and nutrient analyses did not differ significantly

from those described in the JGOFS protocols that were distributed in

June, 1994. Minor differences included the following: 1) Sea Bird CTD

systems and bottle carousels were employed (SBE- 9+ underwater units,

SBE-11 deck units, SBE-32 carousels).  These units represent a newer

generation of equipment than the units described in the JGOFS

protocols.  2) The weights of the salts used for primary standards for

dissolved oxygen and nutrients were not adjusted to an "in vacuo"

basis as suggested in the protocols. It is unlikely that this

departure from procedure would cause significant errors. Our

calculations suggest that the maximum differences arising from our

decision to not correct to an "in vacuo" basis would range from 0.02%

(oxygen standards) to 0.06%(ammonium standards).  3) The protocols

give one a choice of adjusting nutrient methods so that calibration

curves are strictly linear, or opting for more response and taking

into account non-linearities.  We choose the latter method.  4) No

corrections were made for "carryover" between nutrient samples run on

the Technicon Autoanalyzer. Data from this cruise, suggest that

carryover effects in our nutrient analyses are generally less than ~2%

of the concentration difference between adjacent samples. Examination

of cases where more than one sample was taken from a depth at which

there was a significant increase in nutrient concentrations will help

the user determine the carryover effect for many individual casts. 5)

Calibration and re-calibration of volumetric ware were not exactly as

described in the JGOFS protocols, but this was largely compensated for

by comparing independent standards diluted with independent volumetric

ware, and by re-calibration of some of the volumetric ware after

cruises TN045 and TN050. DATA FOR THIS CRUISE (TN045) HAVE BEEN

CORRECTED FOR ERRORS IN THE PIPETS BY MULTIPLYING THE NITRATE AND

PHOSPHATE VALUES BY 0.998, THE SILICATE VALUES BY 0.999, THE NITRITE

VALUES BY 1.004 AND THE AMMONIUM VALUES BY 1.003. 6) Duplicate oxygen

samples were not drawn from every Niskin or Go-Flo bottle, but there

were several comparisons of bottles tripped at the same depth.  7)

Azide was added to the Winkler oxygen pickling reagents to destroy

nitrite that can be present in relatively high concentrations in the

Arabian Sea.

Pressure:

There was a change in pressure sensors during this cruise:

Station 04500101 --- Station 04501003 used SeaBird Pressure Sensor 34901

Station 04501101 --- Station 04503002 used SeaBird Pressure Sensor 43434

It turns out that Sensor 43434 had a pressure hysteresis problem. This problem

appears to be linear.  In order to correct the upcast pressures, the following

method was used:

1.  A nominal "surface" pressure was computed for the CTD using the

mean surface pressure for the previous cruise which was

2.2 db +/- 1.0 db.  (If we do this for the first 10 stations

on TN045, the comparible value was 2.3 +/- 1.0 db).

We assume that this is the nominal surface pressure reading

from the pressure sensor when the CTD package is just

below the surface of the water.

2.  Also, the pressure offset for the deepest station (04501311)

was 7.4 meters and the station depth was 4300 meters. When we

look at the pressure offsets for all of the stations, they

linearily increase as a function of maximun station depth, with

a slope of .0017 (for example for station 04501311 the offset

was -7.2 meters).

3.  This correction was applied in the following manner:

Corrected Pres =

(Max Pres for Station - Original Bottle Pres) * .017

+ Original Bottle Pres

4.  Using this method, the accuracy of the corrected pressure is on the

order of +/- 1 db.

John M. Morrison

Dept of Marine, Earth and Atmospheric Sciences

North Carolina State University

1125 Jordan Hall --- Box 8208

Raleigh, NC 27695 - 8208

Phone:  (919) 515-7449

Fax:    (919) 515-7802

Email:  John_Morrison@NCSU.EDU

Temperature:

The temperature data associated with each bottle depth were taken by

the CTD system during the bottle tripping process.  Consult the

companion CTD data report for this cruise to learn more about the CTD

system.

Sampling:

The samples in this report were taken from 10 liter Niskin

bottles.

Because there is little or no lag time between triggering a bottle and

bottle closure with the new SeaBird rosette systems, bottles were

generally held at the sampling depth for at least 30 seconds before

tripping or until the deck read-outs stabilized if this took more than

30 seconds.

NOTE THAT THE MID-POINT OF THE SAMPLING BOTTLES WAS ONE METER ABOVE

THE CTD SENSORS.  THE DATA HAVE NOT BEEN CORRECTED FOR THIS ONE METER

OR 1.1 DECIBAR DIFFERENCE BETWEEN CTD SENSOR AND SAMPLING BOTTLE

POSITIONS.

Salinity:

Salinities were run on almost every bottle sample with new vials of

standard sea-water used before and at the end of every run (12-36

samples). Agreement between bottle salinities and the recently

calibrated sensors on the Sea Bird CTD systems was usually better than

0.01 except in regions of strong gradients. More information on the

quality of the salinity data are given in the companion CTD report.

Both the CTD salinity data at the time of bottle tripping and the

salinities run on the Niskin bottle samples with an Autosal salinometer

are reported here.

Dissolved oxygen:

The Winkler dissolved oxygen set-up was built and supplied by the

SIO/ODF group.  This system is computer controlled and detects the

end-point photometrically.  Temperature of the thiosulfate and

standard solutions is automatically monitored by this system.  Checks

on cruises TN039 and TN043 between independent standards prepared with

independent volumetric ware gave agreement of +-0.02 per cent. The

linearity of the "Dosimat" automatic buret was also checked during

cruise TN043.

Nutrients:

Note that the terminology used to describe nutrients has become

somewhat loose over the years and that silicate=silicic acid, and

phospate=reactive phosphorus.

Nutrient analyses were performed on a 5-channel Technicon II AA

system that was modified and provided by the SIO/ODF group.

In assessing the nutrient standard comparisons outlined below,

note that the full-scale ranges for nutrients were as follows:

Ammonium  =0 to   5  micromolar

Nitrate   =0 to  45  "

Nitrite   =0 to   5  "

Phosphate =0 to   3.6"

Silicate  =0 to 180  "

These ranges were arrived at after an Internet pole of PI's and

cover the full depth concentration range for the Arabian Sea.

On the set-up and calibration cruise (TN039), the SIO/ODF nitrate and

nitrite standards and standards from the National Institute of

Oceanography in India (provided by S.W.A.  Naqvi) were compared with

the following results:

NIO Nitrate Std.= 22.6 micromolar; SIO/ODF=22.5 micromolar

NIO Nitrite Std.=  2.42 micromolar; SIO/ODF = 2.50 micromolar

As can be inferred from the above, the nitrate plus nitrite

values were almost identical in the mixed standards;

25.02 (NIO) vs 25.00 (SIO)micromolar.

On TN039 Lou Codispoti prepared independent primary nitrate, nitrite,

silicate and phosphate standards for comparison with SIO/ODF primary

standards, and made dilutions using glassware entirely independent of

the SIO/ODF glassware.

The results were as follows:

Codispoti           SIO/ODF

Nitrate   26.96 micromolar    26.85 micromolar

Nitrite    2.90 "              2.86 "

Silicate  86.4  "             85.8  "

Phosphate  2.36 "              2.36  "

All of the above results are within plus or minus 0.5% of the full

scale values, and with the exception of nitrite, the rest are within

plus or minus 0.2% of the full scale values. On TN043, the volumetric

equipment used for making routine nitrate and phosphate standards was

checked against volumetric ware calibrated by LAC. The average of the

results agreed to within +-0.1% of the full scale value for phosphate

and +-0.2% of the full scale value for nitrate.

Because nitrite values in the suboxic waters of the Arabian Sea can

attain values of approximately 5 micromolar, we kept track of the

efficiency of the Cd column that reduces nitrate to nitrite in the

nitrate analysis.  The efficiencies were all greater than 98.8% and

frequently close to 100%.  Therefore, no corrections have been made

for any errors in nitrate arising from deviations in cadmium column

efficiency. NOTE THAT THE FULL-SCALE NITRITE RANGE FOR THIS CRUISE WAS

5 MICROMOLAR AND THAT SOME CONCENTRATIONS EXCEEDED THIS VALUE.  IN

THESE CASES, THE SAMPLES EITHER HAD TO BE DILUTED OR THE VOLTAGE RANGE

CHANGED ON THE RECORDER.  THESE MANIPULATIONS TEND TO DEGRADE THE

ACCURACY OF NITRITE VALUES IN EXCESS OF 4.5 TO 5 MICROMOLAR.

The ammonium results are the least precise of all the nutrient

results.  On TN039, three primary standards were compared with

agreement of about plus or minus three per cent of the full-scale

value. These standards may have agreed within the precision of the

method, but we found a significant salinity effect on the ammonium

results that might explain some of these differences since the

salinities of the comparison standards varied a bit.  Experiments on

this first JGOFS Arabian Sea process study cruise (TN043) suggest that

the ammonium signal decreases by approximately 3.5% for a salinity

increase of 1.00.  Comparisons of an independent standard compared by

LAC with the SIO standard on this cruise (TN043) when corrected for

salinity differences between the standards agreed to ~ + -0.1% of the

full-scale value.  The largest absolute difference was 0.025

micromolar and the average difference was 0.013 micromolar for six

comparisons between 1-3 micromolar. Thus, the average difference

between these two independent standards was + -0.006 micromolar. These

results tend to confirm the need to take salinity differences between

samples and standards into account when calculating the final ammonium

concentrations. THE AMMONIUM VALUES IN THIS REPORT HAVE BEEN CORRECTED

FOR THIS EFFECT. ON THIS CRUISE THE SALINITY EFFECT CORRECTION IS A

3.2% DECREASE IN SIGNAL FOR A SALINITY INCREASE OF 1.00. On this

cruise, the salinity of the working standards used to calibrate the

ammonium method was 34.39.

-----------------------------------------------------------------

Dr. Frank Millero, RSMAS, Univ. of Miami (TCO2, TALK, pH)

This file briefly discribes the methods that were used to make the carbon

dioxide system measurements in the Arabian Sea on the JGOFS Process #2 cruise

by the University of Miami, CO2 group.  More detailed sampling and analytical

methods descriptions can be found in the references given below.

Discrete Total Carbon Dioxide measurements:

The total inorganic carbon dioxide (TCO2) in a volume of seawater was

determined coulometrically after acidification with 8.5 % H3PO4.  The system

has been used in earlier studies by Goyet and Hacker (1992) and it is similar

to the SOMMA (Single-Operator Multiparameter Metabolic Analyzer), contructed by

Johnson et al. (1987). The system is composed of five units: a DICE (Dissolved

Inorganic Carbon Extractor) which controlls the movement and delivery of acid

and sample to the stripper, a coulometer (UIC Inc., model 5011), a CO2 free N2

generator (Balston, model 74-5021), a personal computer and a printer. Nitrogen

gas from the Balston generator is split into two streams, one for pneumatic

controll of sample and acid movements, and the other for the carrier gas for

the CO2 stripped from the seawater sample. The sample is acidified (with 1 to

1.5 mls of 8.5 % phosphoric acid) and the carbon dioxide is extracted with N2

and introduced into the coulometric cell where the CO2 reacts quantitatively

with ethanolamine producing hydroxyethylcarbamic acid.  Hydroxyethylcarbamic

acid is titrated by electrochemically generated hydroxide ion.  The number of

electrons utilized in generating the titrant is proportional to the amount of

inorganic carbon in the original sample.

The life time of the coulometer cell is about twelve hours, after which the

cell solutions need to be changed.  In addition to the cell solutions, the

water trap was changed (Gelman, 0.2 (m PTFE ACRODISC). Changing the cell

solution requires about 30 minutes.  After which about three hours are need

for the system to stabalize, determine new blank values, and confirm the

calibration with analyses of CRM's.   If the CRM value was not reproduced with

in 2 (mol/kg TCO2, the system was recalibrated with Na2CO3 standards.  A single

measurement takes about 25 minutes, and a 24 bottle station cast can be

completed in eight hours.

Calibration of the system:

The electrical calibration of the coulometer is not perfectly accurate and the

current efficiency of the electrode processes occurring in the coulometer cell

has been shown to vary from 100 %.  Consequently, the system was calibrated

using aqueous solutions of sodium carbonate which were treated as if they were

samples. The consistency of the calibration was checked for each cell solution

using  the Certified Reference Material (Dr. Andrew Dickson, Marine Physical

Laboratory, La Jolla, California).

Alkalinity Titration system:

The alkalinity titration system is similar to the one used in our earlier

studies (Millero et al., 1993).  The titration systems used to determined TA

consisted of a titrator (Metrohm, model 665 Dosimat) and a pH meter (Orion,

model 720A) that is controlled by a personal computer.  The temperature of

both the acid titrant in a water jacketed burette and the seawater sample in a

water jacketed cell were controlled to a constant temperature of 25  0.1C with

a constant temperature bath (Neslab, model RTE 221).  The plexiglass water

jacketed cells used during the cruise were similar to that used by Bradshaw et

al. (1988) except a larger volume (about 200 cm3) was used to increase the

precision.  Each cell had a fill and drain valve which increased the

reproducibility of the volume of sample contained in the cell.  A LabWindows-C

program was used to run the titration, record the volume of the added acid and

the emf of the electrodes using RS-232 communication interfaces.  Seawater

samples were titrated  by adding HCl to exceed  the carbonic acid end point.

During a typical titration the emf readings are recorded after the readings

become stable ( 0.05 mV),and then a volume of acid is added to change the

voltage to a pre-assigned increment (13 mV).  In contrast to the delivery of a

fixed volume increment of acid, this method gives an even distribution of data

points in the range of  rapid increase in the emf near the endpoint.  A full

titration (25 points) takes about 20 minutes.  Using two systems a 24 bottle

station cast can be completed in 4 hours.

Selected References:

DOE (1994) Handbook of methods for the analysis of the various parameters of

the carbon dioxide system in sea water, version 2, A. G. Dickson & C. Goyet,

eds.

Goyet, C. and Hacker, S.D. (1992)  Procedure for calibration of a coulometric

system used for total inorganic carbon measurements. Marine Chemistry 38,

37-51

Millero, F. J., Zhang, J., Lee, K., and Campbell, D. M. (1993) Titration

alkalinity of seawater.  Mar. Chem., 44:153-165.

---------------------------------------------------------------------

Dr. Edward Peltzer, Woods Hole Oceanographic Inst, & Dr. Dennis Hansell,

Bermuda Biological Station for Research

Methods reported in:

Peltzer, Edward T. (1993).  Shipboard determination of total organic carbon by

a high temperature combustion/direct injection technique.  U.S. Joint Global

Ocean Flux Study - Equatorial Pacific Protocols, 1993, section 21A.

The EqPac methods were followed explicity on the Arabian Sea Process cruises

with two exceptions:

1. For Process cruises 1, 2 and 4, the WHOI custom built HTC/DI-TOC

analyzer was used.  For Process cruises 6 and 7, the BBSR custom

built HTC/DI-TOC analyzer was used.

2. Low carbon water (LCW) prepared using a commercial Milli-Q UV/TOC (R)

system was used to measure the instrument blank as opposed to the

carbon free distilled water (CFDW) that was used in EqPac.  For all

legs, the LCW was assigned a background TOC value of 0 uMC.

---------------------------------------------------------------------

Dr. Dennis Hansell, Bermuda Biological Station for Research

Total organic nitrogen data

Shore based analysts: Paula Hansell and Catherine Adams

Methods reported in:

Hansell, D.A. and T,Y. Waterhouse. 1997. Controls on the

distribution of organic carbon and nitogen in the eastern Pacific

Ocean, Deep Sea Research, 44: 843-857.

Notes:

1. Samples were taken in 60 ml. polyethylene bottles and stored

frozen until analyis at BBSR.

2. Samples were oxidized by UV light for at least 24 hours to

ensure full oxidation of refractory organics such as urea.

_____________________________________________________________

Dr. Hugh Ducklow, VIMS, College of William & Mary

Particulate carbon and nitrogen

Notes on the Sampling Methodology, by Hugh Ducklow

Samples were harvested from hydrocasts into Milli-Q and sample-rinsed

2-liter polycarbonate bottles, and concentrated over low vacuum

filtration onto precombusted GF/F filters. Volumes collected were

adjusted according to depth, expected carbon concentration and filter

color (generally 1-4 liters). Filters were removed from the manifold

before they dried and were stored folded in half in plastic envelopes

in a dessicator. The filters were returned to the USA on dry ice, and

finally shipped to BBSR on dry ice for analysis.

Samples were analyzed at Bermuda Biological Station for Research

following the BATS protocols.  NB: Filters were acidified so the

carbon analysis is for particulate organic carbon not total particulate

carbon.