Meta data files JGOFS Arabian Sea Cruise TTN050

Dr. Lou Codispoti (Physical and nutrient parameters)

Dennis Hansell  (TOC and TON)

-------------------------------------------------------------------------

L.A. Codispoti (lou@ccpo.odu.edu)

Old Dominion University, June 1996

General Comments:

This "readme" file pertains to the salinity, dissolved oxygen, and

nutrient data taken from sampling bottles with the hydrographic

rosette that was equipped with 24 10-liter Niskin type bottles during

RV T.G. Thompson cruise TN050. This cruise was the fourth JGOFS

Arabian Sea Process Leg and took place during Aug.-Sept. 1995.

Prof. Sharon L. Smith of the University of Miami

(ssmith@rsmas.miami.edu) was the chief scientist.

NOTE THAT MULTIPLE CASTS WERE TAKEN AT MOST STATIONS AND THAT, IN SOME

CASES, (WHEN FOLLOWING THE PRIMARY PRODUCTIVITY DROGUE, FOR EXAMPLE)

THE GEOGRAPHIC POSITIONS OF CASTS AT THE SAME STATION CAN VARY BY

MORE THAN 5 MILES. FOR EXAMPLE, CAST 9 AT STATION 7 (TN05000709) WAS

MORE THAN 10 MILES AWAY FROM THE "STANDARD" POSITION. TN05002109 WAS

MORE THAN 5 MI "OFF" AND TN002609 AND TN002610 WERE ABOUT 10 MI "OFF".

Some questionable data are not included in this report. These data are

still retained in files at Old Dominion University and are available

upon request.

No units are given for salinity in this report because the most recent

definitions of salinity define it as a dimensionless number.  To

accommodate every preference, Winkler oxygen values are reported in

ml/l, micromolar and micromoles per kg.  The latter values can only be

calculated with a knowledge of the oxygen sample temperatures when the

samples were drawn. These "draw temperatures" are not reported here,

but can be obtained by contacting lou@ccpo.odu.edu.  Nutrient values

are reported in micromolar. They can be converted to micromoles per

kg, by combining laboratory temperature on the Thompson (approx. 24.5

deg C during this leg) and the salinity of the sample to compute

density and then dividing the value in micromolar by this number.

Methods:

In general, the methods employed for the bottle salinity, Winkler

dissolved oxygen, and nutrient analyses did not differ significantly

from those described in the JGOFS protocols that were distributed in

June, 1994. Minor differences included the following: 1) Sea Bird CTD

systems and bottle carousels were employed (SBE-9+ underwater units,

SBE-11 deck units, SBE-32 carousels).  These units represent a newer

generation of equipment than the units described in the JGOFS

protocols.  2) The weights of the salts used for primary standards for

dissolved oxygen and nutrients were not adjusted to an "in vacuo"

basis as suggested in the protocols. It is unlikely that this

departure from procedure would cause significant errors. Our

calculations suggest that the maximum differences arising from our

decision to not correct to an "in vacuo" basis would range from 0.02%

(oxygen standards) to 0.06%(ammonium standards).  3) The protocols

give one a choice of adjusting nutrient methods so that calibration

curves are strictly linear, or opting for more response and taking

into account non-linearities.  We choose the latter method.  4) No

corrections were made for "carryover" between nutrient samples run on

the Technicon Autoanalyzer. Data from this cruise, suggest that

carryover effects in our nutrient analyses are generally less than ~2%

of the concentration difference between adjacent samples. Examination

of cases where more than one sample was taken from a depth at which

there was a significant increase in nutrient concentrations will help

the user determine the carryover effect for many individual casts. 5)

Calibration and re-calibration of volumetric ware were not exactly as

described in the JGOFS protocols, but this was largely compensated for

by comparing independent standards diluted with independent volumetric

ware, and by re-calibration of some of the volumetric ware after

cruises TN045 and TN050. DATA FOR THIS CRUISE (TN050) HAVE BEEN

CORRECTED FOR ERRORS IN THE PIPETS BY MULTIPLYING THE SILICATE VALUES

BY 0.999 AND THE PHOSPHATE AND NITRATE VALUES BY 0.998. 6) Duplicate

oxygen samples were not drawn from every Niskin or Go-Flo bottle, but

there were several comparisons of bottles tripped at the same depth.

7) Azide was added to the Winkler oxygen pickling reagents to destroy

nitrite that can be present in relatively high concentrations in the

Arabian Sea. ON LEGS PRIOR TO THIS ONE, OXYGEN STANDARDIZATIONS WERE RUN

USING REAGENTS THAT DID NOT CONTAIN AZIDE, BUT DISCUSSIONS AND TESTS

SUGGESTED THAT IT WOULD BE BETTER TO STANDARDIZE WITH AZIDE, DESPITE

SOME CONFUSION IN THE LITERATURE ON THIS MATTER.  CONSEQUENTLY, WE

SWITCHED PROCEDURES BEGINNING WITH THIS LEG (TN050) AND USED REAGENTS

CONTAINING AZIDE TO STANDARDIZE.  OUR TESTS SUGGEST THAT THE MAXIMUM

CHANGE IN OXYGEN CONCENTRATIONS ARISING FROM THIS CHANGE WOULD OCCUR

AT THE HIGHEST OXYGEN CONCENTRATIONS AND BE < ~0.01 ML/L.

Temperature:

The temperature data associated with each bottle depth were taken by

the CTD system during the bottle tripping process.  Consult the

companion CTD data report for this cruise to learn more about the CTD

system.

Sampling:

The samples in this report were taken from 10 liter Niskin

bottles.

Because there is little or no lag time between triggering a bottle and

bottle closure with the new SeaBird rosette systems, bottles were

generally held at the sampling depth for at least 30 seconds before

tripping or until the deck read-outs stabilized if this took more than

30 seconds.

NOTE THAT THE MID-POINT OF THE SAMPLING BOTTLES WAS ONE METER ABOVE

THE CTD SENSORS.  THE DATA HAVE NOT BEEN CORRECTED FOR THIS ONE METER

OR 1.1 DECIBAR DIFFERENCE BETWEEN CTD SENSOR AND SAMPLING BOTTLE

POSITIONS.

Salinity:

Salinities were determined with Guildline Autosal salinometers. New

vials of standard sea-water were used to standardize before and at the

end of every run. Agreement between bottle salinities and the recently

calibrated sensors on the Sea Bird CTD systems was usually better than

0.01 (except in regions of strong gradients) before post-cruise data

processing which employs the bottle salinities to correct the CTD

salinities. More information on the quality of the salinity data are

given in the companion CTD report.  Both the CTD salinity data at the

time of bottle tripping and the salinities run on the Niskin bottle

samples with an Autosal salinometer are reported here.

Dissolved oxygen:

The Winkler dissolved oxygen set-up was built and supplied by the

SIO/ODF group.  This system is computer controlled and detects the

end-point photometrically.  Temperature of the thiosulfate and

standard solutions is automatically monitored by this system.  Checks

on cruises TN039 and TN043 between independent standards prepared with

independent volumetric ware gave agreement of +-0.02 per cent. A

similar check made during TN054 suggested agreement of better than

+-0.15 per cent. The linearity of the "Dosimat" automatic buret was

also checked during cruises TN043 and TN054 with good results.

Nutrients:

Note that the terminology used to describe nutrients has become

somewhat loose over the years and that silicate=silicic acid, and

phospate=reactive phosphorus.

Nutrient analyses were performed on a 5-channel Technicon II AA

system that was modified and provided by the SIO/ODF group.

In assessing the nutrient standard comparisons outlined below,

note that the full-scale ranges for nutrients were as follows:

Ammonium  =0 to   5  micromolar

Nitrate   =0 to  45  "

Nitrite   =0 to   5  "

Phosphate =0 to   3.6"

Silicate  =0 to 180  "

These ranges were arrived at after an Internet poll of PI's and

cover the full depth concentration range for the Arabian Sea.

On the set-up and calibration cruise (TN039), the SIO/ODF nitrate and

nitrite standards and standards from the National Institute of

Oceanography in India (provided by S.W.A.  Naqvi) were compared with

the following results:

NIO Nitrate Std.= 22.6 micromolar; SIO/ODF=22.5 micromolar

NIO Nitrite Std.=  2.42 micromolar; SIO/ODF = 2.50 micromolar

As can be inferred from the above, the nitrate plus nitrite

values were almost identical in the mixed standards;

25.02 (NIO) vs 25.00 (SIO)micromolar.

On TN039, Lou Codispoti prepared independent primary nitrate, nitrite,

silicate and phosphate standards for comparison with SIO/ODF primary

standards and made dilutions using glassware entirely independent of

the SIO/ODF glassware.

The results were as follows:

Codispoti           SIO/ODF

Nitrate   26.96 micromolar    26.85 micromolar

Nitrite    2.90 "              2.86 "

Silicate  86.4  "             85.8  "

Phosphate  2.36 "              2.36  "

On TN043, the volumetric equipment used for making routine nitrate and

phosphate standards was checked against volumetric ware calibrated by

LAC. The average difference between these comparsions of mid-range

standards was + or - 0.2% for phosphate and + or -0.4% for nitrate.

Because nitrite values in the suboxic waters of the Arabian Sea can

attain values of approximately 5 micromolar, we kept track of the

efficiency of the Cd column that reduces nitrate to nitrite in the

nitrate analysis.  The efficiencies were all greater than 99% during

this cruise, so no corrections have been made for any errors in

nitrate arising from deviations in cadmium column efficiency. NOTE

THAT THE FULL-SCALE NITRITE RANGE FOR THIS CRUISE WAS 5 MICROMOLAR UP

TO STATION TN05001708 WHEN THE SYSTEM WAS "RE-PLUMBED" TO COVER THE

FULL NITRITE RANGE (~0.0-7.0 MICROMOLAR). BEFORE TN05001708

CONCENTRATIONS THAT EXCEEDED 5 MICROMOLAR EITHER HAD TO BE DILUTED OR

THE VOLTAGE RANGE CHANGED ON THE RECORDER.  THESE MANIPULATIONS TEND

TO DEGRADE THE ACCURACY OF NITRITE VALUES IN EXCESS OF 4.5 TO 5

MICROMOLAR.

The ammonium results are the least precise of all the nutrient

results.  On TN039, three primary standards were compared with

agreement of about plus or minus three per cent of the full-scale

value. These standards may have agreed within the precision of the

method, but we found a significant salinity effect on the ammonium

results that might explain some of these differences since the

salinities of the comparison standards varied a bit.  Experiments on

this first JGOFS Arabian Sea process study cruise (TN043) suggested

that the ammonium signal decreases by approximately 3.5% for a

salinity increase of 1.00.  Comparisons of an independent standard

compared by LAC with the SIO standard on this cruise (TN043) when

corrected for salinity differences between the standards agreed to ~ +

-0.1% of the full-scale value.  The largest absolute difference was

0.025 micromolar and the average difference was 0.013 micromolar for

six comparisons between 1-3 micromolar. Thus, the average difference

between these two independent standards was + -0.006 micromolar. These

results tend to confirm the need to take salinity differences between

samples and standards into account when calculating the final ammonium

concentrations. THE AMMONIUM VALUES IN THIS REPORT HAVE BEEN CORRECTED

FOR THIS EFFECT. ON THIS CRUISE THE SALINITY EFFECT CORRECTION IS A

2.9% DECREASE IN SIGNAL FOR A SALINITY INCREASE OF 1.00. On this

cruise, the salinity of the working standards used to calibrate the

ammonium method was ~35.06 for stations TN001-TN013 and ~34.89 for the

remainder of the stations.

Final Corrections

performed at US JGOFS DMO October 28, 1996

From: Lou Codispoti

Subject: Final Corrections

Dear JGOFS investigator,

This is to inform you that some minor corrections were made to the

JGOFS Arabian Sea nutrient data after the 1996 meeting in New

Hampshire.  It is doubtful that any of these corrections will alter any

of your hypotheses, but they may have some significance in documenting

changes over long periods of time, and in WOCE-style investigations.

The need for these corrections arose, as a consequence of post-cruise

re-calibration of pipets, and because we did not properly account for

the nutrient content of our low nutrient sea-water.

The corrections have already been applied to the US JGOFS Arabian Sea

nutrient data collected with the hydrographic rosette, by personnel at

WHOI who maintain the US JGOFS data. I list them here for your

information.

1) All ammonium and nitrite data from Thompson leg TN053 were

multiplied by 0.997.

2) All ammonium and nitrite data from ALL US JGOFS Arabian Sea cruises

(TN039, 043, 045, 049, 050, 053 and 054) were multiplied by 0.995.

3) ALL phosphate data from ALL US JGOFS Arabian Sea cruises (TN039,

043, 045, 049, 050, 053 and 054) were multiplied  by 0.992.

Corrections 2) and 3) should probably be applied to the ONR data as

well, but I will leave this up to Burt Jones, as I am not exactly sure

how he did his standardizations.  Basically, corrections  2) and 3)

arise from a failure to take into account dilution of the Low Nutrient

Sea Water signal by standards in cases where the LNSW contains

appreciable quantities of nutrients.  The above corrections are pretty

minor within the scheme of nutrient analyses, but we should always

attempt to eliminate such systematic errors.

George and Chris have made these corrections on the US JGOFS Arabian Sea

nutrient data collected from the hydrographic rosette. Trace metal

rosette data and experimental data have not yet been corrected.

Cheers, Lou Codispoti

---------------------------------+------------------------------------------+

Total organic carbon profile data (TOC)

Dennis Hansell

Bermuda Biological Station for Research

Laboratory analyst:   Rachel Parsons (BBSR)

Method reported in:

Hansell, D.A., C.A. Carlson, N. Bates and A. Poisson.

Horizontal and vertical removal of organic carbon in the

equatorial Pacific Ocean: a mass balance assessment.  Deep-Sea

Research II (in press).

Notes:

1. For Process cruises 1, 2 and 4, the WHOI custom built HTC/DI-TOC

analyzer was used.  For Process cruises 5, 6 and 7, the BBSR custom

built HTC/DI-TOC analyzer was used.

2. Low carbon water (LCW) prepared using a commercial Milli-Q UV/TOC (R)

system was used to measure the instrument blank as opposed to the

carbon free distilled water (CFDW) that was used in EqPac.  For all

legs, the LCW was assigned a background TOC value of 0 uMC.

Total organic nitrogen data  (TON)

Shore based analysts: Paula Hansell and Catherine Adams

Methods reported in:

Hansell, D.A. and T,Y. Waterhouse. 1997. Controls on the

distribution of organic carbon and nitogen in the eastern Pacific

Ocean, Deep Sea Research, 44: 843-857.

Notes:

1. Samples were taken in 60 ml. polyethylene bottles and stored

frozen until analyis at BBSR.

2. Samples were oxidized by UV light for at least 24 hours to

ensure full oxidation of refractory organics such as urea.

----------------------------------------------------------------------