MASTER SAMPLE FILE METADATA

August 25, 1997

PROGRAM:	1990 Reference Site Survey.

SOURCE:	Southern California Coastal Water Research Project (SCCWRP)

CONTACT:	Dr. Mary Bergen

PHONE:	(714) 894-2222,  (714) 894-9699 (FAX)

NUMBER OF STATIONS SAMPLED:	20.  Seven of the 13 sites sampled in the 1985 Reference Site Survey were re-sampled.  At each area samples were taken at 30, 60, and 150 meters.  The 30 m station at area R61 off La Jolla was abandoned due to kelp.

SAMPLING PERIOD:	Samples were taken between 30 July and 7 August 1990.

NUMBER OF VARIABLES MEASURED:	3:  Latitude and Longitude (to 0.000 01 degrees) and depth (meters).

SAMPLING EQUIPMENT USED:	A 0.1 meter-square, modified chain-rigged Van Veen grab sampler was used to obtain sediment samples.

ANALYTICAL TECHNIQUES:	Navigation was accomplished using LORAN.  Coordinates were later converted using Global Positioning System (GPS) and were based upon North American Datum 27 (NAD 27).  Depth was measured using a fathometer.

DATA SET VERIFICATION:	Data set verified by SCCWRP.

GENERAL COMMENTS:	The purpose of the 1990 Reference Site Survey was to provide reference information that could be used for comparison to data collected from areas affected by wastewater discharges.  The 1990 survey was designed to update the information reported in the 1977 60-Meter Control Survey, and the 1985 Reference Site Survey.

This data set includes the location and date of sampling for all samples for benthic analysis.  Each record has the following fields (variables):  Sample ID, Agency, Project, Study, Data Type, Sample Type, Date, Time, Site, Latitude, Longitude, Elevation/Depth, Max Depth, and Units.

Sample ID is the station identification.  A five character code from R13-30 to R71-150.  The first three characters before the dash refer to the site sampled.  The numbers after the dash refer to the depth at that site.  R13-30 = Site R13, depth 30 meters.  Twenty stations are identified.

Agency indicates the organization responsible for the data:  SCCWRP = Southern California Coastal Water Research Project

Project identifies the reason for data generation:  R = Reference Site Survey.

Study describes that this was a summer survey.  90-Summer = survey that was conducted during the summer of 1990.

Data type identifies the data file.  The code BENIF = Benthic infauna data.

Sample type is a code that indicates whether it is an analytical or control sample.  FS = Field sample.

Date (MM/DD/YY)  Benthic grab samples were taken between 30 July and 7 August 1990.

Time (hhmm) is not available.

Site identifies the areas sampled. In 1990, only seven of the original 34 areas between Point Conception and the Mexican Border were re-sampled.  The 30 meter station at area R61 off La Jolla was abandoned due to kelp.

List of Areas			# Stations Sampled

Point Hueneme			3

Trancas Canyon			3

Laguna Beach				3

San Clemente				3

Sorrento				3

La Jolla				2

Imperial Beach			3

Latitude (degrees North) was the recorded position at the time of sampling.

Longitude (degrees West) was the recorded position at the time of sampling.

Elevation/depth was at the time of sampling.  Depths ranged from 30-150 meters.

Max Depth was the maximum depth at the station.  It is equal to the Elevation/depth since the samples were taken from the bottom.

Units is a field to describe how depth was measured.  M = meters.

Specifics regarding sampling, analysis and QA/QC are available from SCCWRP upon request.

QUALITY ASSURANCE AND QUALITY CONTROL EVALUATION:	A minimum of 10% of each sample was re-sorted to determine if organisms were missed in the original sort.  If sorting efficiency was less than 95%, then the entire sample was resorted.

This data should be used with the understanding that there have been changes in taxonomy since 1990.  The taxonomy was accurate for its time; all samples were identified by specifically trained scientists who were experts in their taxonomic group.  However, since 1990 taxonomic changes have occurred, including changes in nomenclature, description of new species and refinements in understanding variation in species characteristics.  Species names have not been standardized to conform with the earlier 1977 60-Meter Control Survey or the 1985 Reference Site Survey.

FIELD AND LABORATORY TECHNIQUES:	Macrobenthic infauna were collected from the sediment samples by sieving through a 1.0 mm mesh screen.  The animals and debris were fixed in 10% borax buffered formalin in seawater.  Upon return to the laboratory, the samples were transferred to 70% ethanol for preservation, sorted into major taxonomic groups (annelids, molluscs, arthropods, ophiuroids, miscellaneous echinoderms, and "other phyla"), identified to the lowest taxon practical and enumerated.

DATA USE AND COMPARABILITY:	The data are applicable for locating sampling stations used in the 1990 Reference Site Survey and can be compared to the data collected from the 1977 60-Meter Control Survey and the 1985 Reference Site Survey.