Benthic Infaunal Abundance Metadata

April 2, 1996

Program:  Southern California Bight Pilot Project (SCBPP)

Source:  Southern California Coastal Water Research Project (SCCWRP)

Contact:  Mary Bergen

Phone:  (714) 894-2222, -9699 (FAX)

Number Of Stations Sampled:  251

Sampling Period:  13 July 1994 -- 22 August 1994

Number Of Variables Measured:  1: infaunal abundance.  Infaunal biomass appears in a separate file.

Sampling Equipment Used:  Sediment for infaunal analysis was collected with a modified, chain-rigged 0.1m2 Van Veen grab and seived through 1.00 mm mesh.

Analytical Techniques:  Infaunal samples were distributed to the City of Los Angeles Hyperion Treatment Plant-Environmental Monitoring Division, the Sanitation Districts of Los Angeles County, the City of San Diego Metropolitan Wastewater Department and MEC Analytical Systems, Inc. for sorting and identification of species.

Data Set Verification:  Data set verified by biologists from City of Los Angeles Hyperion Treatment Plant-Environmental Monitoring Division, the Sanitation Districts of Los Angeles County, the City of San Diego Metropolitan Wastewater Department, MEC Analytical Systems, Inc. and SCCWRP.

General Comments:

This data set includes number of indivduals per grab for each taxon identified at each station.  Each record has the following fields (variables): LogNumber, Species, Abundance, Unidentified, Include, LabCode, and Comments.  The log number is a 10 digit code in the following form:

Digit

1     -     P for Pilot Project

2 & 3   -     Agency collecting the data

4 & 5   -     Type of data (BE = Benthic Infauna)

6 to 9  -     Station number

10    -     Replicate number.

Species is the species binomen or higher level identification.

Abundance is reported as number per grab.

The field "Unidentified" was designed to identify taxa that were too damaged or too small to be identified to species.  The field is null because the codes were not consistently used by all the participating laboratories.

The field "Include" was designed to identify taxa that should be included in the analysis, despite not being a species level identification.  The field is null because the codes were not consistently used by all the participating laboratories.

Lab Code identifies the laboratory that sorted and identified the sample (HY = Hyperion Treatment Plant, LA = County Sanitation Districts of Los Angeles County, ME = MEC Analytical Systems, Inc. and SD = City of San Diego).

Comments is a field used for any unique notations.

Station locations can be found in the Benthic Master Sample File.  Descriptive data on sediment color and texture and depth of penetration of the grab are available from SCCWRP on request.

Field And Laboratory Techniques:  Sediment for infaunal analysis were collected with a modified 0.1m2 Van Veen grab.  After it was determined that the surface of the sediment was undisturbed and the penetration depth was at least 5 cm, the overlying water was drained off and the sediment was seived through 1.00 mm mesh.  Material retained on the screen was transferred to a sample jar.  After 30 minutes of exposure to relaxant (MgSO4), the sample was fixed in 10% buffered formalin.  After 3-14 days samples were transferred into 70% ethanol.  The samples were then sorted under a dissecting microscope and the specimens were identified and counted.

Quality Assurance And Quality Control Evaluation:

To determine sorter performance, a minumum of 10% of all material processed was resorted.  If sorting efficiency was determined to be below 95%, samples were re-sorted.  To determine accuracy of species identifications and counts, 10% of the samples were randomly re-distributed among the laboratories for re-identification.  Accuracy was maintained at a minimum of 90%.

To maintain consistency among the laboratories, monthly meetings, under the auspices of the Southern California Association of Marine Invertebrate Taxonomists, were held to discuss problem species.  After the data was compiled, the taxonomists conducted a synoptic review of the species lists to determine if identifcations were consistent among the laboratories.  On the basis of the review, some identifications were changed to a higher taxonomic level in order to achieve taxonomic consistency.

Data Use And Comparability:  The data are appropriate for analysis of general near shore (10-200 meters) benthic conditions in the Southern California Bight in the summer 1994.  The data are appropriate for use with other data collected during the SCBPP.

The sample design was based on the random grid sampling design used in EPA's Environmental Monitoring and Assessment Program.  The data were collected to estimate the amount of area with a particular characteristic (e.g., the amount of area with dissolved oxygen less then 5 mg/l).  Since the grid design affects the calculation of variance, care should be taken in making statistical comparisons between this data and data collected in fixed-station or stratified random sampling.  See the SCBPP Overview file for more information.