CTDMAST.DOC

CTD Master sample File Metadata

APRIL 26, 1996

Program:  Southern California Bight Pilot Project (SCBPP)

Source:  Southern California Coastal Water Research Project (SCCWRP)

Contact:  Dario Diehl

Phone:  (714) 894-2222

Number Of Stations Sampled:  264 stations were designated for sampling.  Two stations were abandoned due to exceedence of maximum target depth (>220m) and another because of placement within Mexican waters.

Sampling Period:  12 July 1994 -- 27 August 1994

Number Of Variables Measured:  3: Latitude and Longitude (to 0.00001 degrees) plus depth (meters).  Other variables exist including weather conditions and sampling identifiers and qualifiers.

Sampling Equipment Used:  The survey used either a Seabird SBE-9 or SBE-25 conductivity, temperature, and depth (CTD) unit to obtain water column profiles.  Each unit had a SBE-3 temperature probe, SBE-4 conductivity probe, Sensor-Metrics or Parascientific Digiquartz pressure gauge, SBE-13 oxygen probe, SBE pH probe, Sea-Tech 25 transmissometer, and either internal (RAM-Random Access Memory) or external (cable to deck box interface) recording with data being downloaded to a portable computer.

Analytical Techniques:  Navigation was accomplished using either Loran-C or Global Positioning System (GPS).  Coordinates were based upon North American Datum 83 (NAD 83).  Depth was measured using a fathometer.

Data Set Verification:  The data set verified by the cruise leader aboard each sampling vessel and by SCCWRP.

General Comments:  The data set includes the station sampled, station location, sampling date, and the agency taking the measurement as well as weather and sea conditions recorded at the time of sampling.  Each record has the following fields (variables): Station, LogNumber, Date, Agency, Vessel, Depth, ArriveTime, DepartTime, Weather, WeatherCode, WindSpeed, WindDir, SwellHt, SwellPeriod, CTDMethod, BeginCastTime, BeginLatitude, BeginLongitude, BottomTime, BottomLatitude, BottomLongitude, FailureCode, StationAccept, StationAcceptComments, and Comments.

Station is a 4 digit code non-continuous from 0009 to 2011.  A total os 264 stations are identified.

LogNumber is a 10 digit code in the following form:

Digit

1     -     P for Pilot Project

2 & 3   -     Agency collecting the data

4 & 5   -     Type of data (WQ = Water Quality)

6 to 9  -     Station number

10    -     Replicate number.

Date (MM-DD-YY)

Agency indicates which of the 5 groups was responsible for sampling at that station:

HY = City of Los Angeles Hyperion Treatment Plant-Environmental Monitoring Division

LA = County Sanitation Districts of Los Angeles County

OC = County Sanitation Districts of Orange County

SD = City of San Diego Metropolitan Wastewater Department)

SC = Southern California Coastal Water Research Project (contracted to MEC Analytical Systems, Inc.).

Vessel indicates the name of the boat used to sample the station.

Depth (meters) was at the time of sampling, not corrected for tides.  Sampling depths ranged from 10 to 220 meters (blank = not sampled, -1 = not recorded).

ArriveTime is the time (HH:MM, 24 hour clock) of arrival at the indicated station.

DepartTime is the time (HH:MM, 24 hour clock) of departure from the indicated station.

Weather is text describing conditions present during sampling at station.

WeatherCode is a one digit code with the following format:

0 = Clear (no cloud at any level)

1 = Partly cloudy (scattered or broken)

2 = Continuous layers of clouds

3 = Sand storm, dust storm, or blowing snow

4 = Fog, thick dust or haze.

5 = Drizzle

6 = Rain

7 = Snow, or rain and snow mixed

8 = Showers

9 = Thunderstorms

WindSpeed (miles per hour, mph) at the time of sampling.

WindDir is wind direction (degrees from magnetic North) at the time of sampling.

SwellHt is the swell height (feet) at the time of sampling.

SwellPeriod is the time (seconds) between maximum swell heights.

CTDMethod is the type of data recording on the CTD; RAM = internal and REAL TIME = external.

BeginCastTime is the time (HH:MM, 24 hour clock) the CTD began to be lowered to the bottom of the station.

BeginLatitude is the recorded latitude when the CTD began to be lowered to the bottom of the station.

BeginLongitude is the recorded longitude when the CTD began to be lowered to the bottom of the station.

BottomTime is the time (HH:MM, 24 hour clock) the CTD reached the bottom of the station.

BottomLatitude is the recorded latitude when the CTD reached the bottom of the station.

BottomLongitude is the recorded longitude when the CTD reached the bottom of the station.

FailureCode describes whether a sample was obtained with the following formats:

No = sample taken

Yes = no sample taken

StationAccept is a one character code which describes if the station was acceptable for use in estimating percent areas.  Acceptability is based upon distance from the prescribed sampling location and follows the format.

Y = Yes, this station was acceptable (260 total), within 1.2 times the radius of a circle with an area equal to the area represented by the sample.

N = No, this station was unacceptable (3 total) due to either the FailCode = -1 or the station location was greater than 2 times the radius of a circle with an area equal to the area represented by the sample.

M = Maybe, this station was marginally acceptable (1 total); the station location was greater than 1.2, but less than 2 time the radius of a circle with an area equal to the area represented by the sample.

StationAcceptComments is a field for unique StationAccept notations.

Comments is field used for any unique notations.

Station locations can be found in the CTD Master Sample File (CTD.MTR).  Complete One meter interpolated data for the downcast is in file ONEMETER.DAT.  Raw data for the completed stations are in file CTD.RAW.  Specifics regarding sampling, analysis and QA/QC are available from SCCWRP on request.

Quality Assurance And Quality Control Evaluation:  The SCBPP Quality Assurance Project Plan and Field Manual lists minimum ongoing QA/QC requirements for CTD sampling.  Each agency was responsible for minimum equipment and training requirements.  Unacceptable CTD casts based upon graphical appearance were immediately excluded and resampled.  Greater than 98% of the stations designated (261 out of 264 stations)for the study were completed.

All QA/QC requirements were met for this data set.

Each organization participated in a intercalibration exercise at SCCWRP to evaluate the precision, accuracy, and comparability of the CTD's used during the survey.  In addition, each group did a pre-survey instrument calibration (within 24 hours of field sampling) to standards described in the Southern California Bight Pilot Project Field Manual and a post-survey instrument calibration to evaluate sensor drift.

In the field, the first cast of the day was graphically displayed to evaluate proper sensor performance.  Thereafter, casts were periodically evaluated to see if sensors maintained their performance.  After the survey was complete, the SCBPP CTD group checked station data integrity by verifying that the decent rate and depth criteria were met for each station.  Downcast data were processed with a computer to identify outlying data points.  If the CTD group determined equipment performance was the cause of the outlier, the data point was flagged in the data set.

Field And Laboratory Techniques:  Water quality measurements were made by four public agencies (City of Los Angeles Hyperion Treatment Plant-Environmental Monitoring Division, County Sanitation Districts of Los Angeles County, County Sanitation Districts of Orange County, and City of San Diego Metropolitan Wastewater Department) and one private contractor (MEC Analytical Systems, Inc.).

Stations were located either by Loran-C or GPS.  Once on station, data forms were filled out regarding the station.  Before beginning the downcast, the CTD was placed in the water for a minimum of three minutes for the first station of the day and 90 seconds for all additional stations to thermally equilibrate and purge instrument tubing.  The unit was raised to the surface (top of cage) and then lowered at a decent rate not to exceed one meter per second to within two meters or five percent of the bottom depth.  The data was either downloaded to the computer directly or stored in internal RAM and downloaded to the computer sometime during the day.

Data Use And Comparability:  The data are applicable for locating water quality sampling stations used in the SCBPP and provide information on weather conditions during sampling.  These data are appropriate for use with other data collected during the SCBPP.

See the SCBPP overview file for more information about the sampling design.