SEDIMENT CHEMISTRY METADATA

EARLY RELEASE, VERSION 1 - CHECK FOR FURTHER UPDATES

July 18, 1996

PROGRAM:  Southern California Bight Pilot Project (SCBPP)

SOURCE:  Southern California Coastal Water Research Project (SCCWRP)

CONTACT:  Mary Bergen

PHONE:  (714) 894-2222, -9699 (FAX)

NUMBER OF STATIONS REPORTED:  249

SAMPLING PERIOD:  2 July 1994 -- 22 August 1994

NUMBER OF VARIABLES MEASURED:  20: 14 Inorganic (Aluminum, Arsenic, Antimony, Beryllium, Cadmium, Chromium, Copper, Iron, Lead, Mercury, Nickel, Selenium, Silver, Zinc), and 6 Organic (op- and pp-DDT, op- and pp-DDE, op- and pp-DDD).

SAMPLING EQUIPMENT USED:  The survey used a 0.1 meter-square, modified chain-rigged Van Veen grab sampler to obtain sediment samples.  Sediments were placed into pre-cleaned glass jars and kept frozen until analyzed.

ANALYTICAL TECHNIQUES:  Laboratory analyses for DDT's were conducted by four public agencies and one contract lab (See Table 1).  The methods were performance-based so there was no standard protocol used throughout the study.  However, each laboratory typically follows EPA guidelines (i.e. Methods for the analysis of solid waste, SW-846).  For organics, samples of marine sediment were solvent extracted and analyzed by gas chromatography.  Laboratory analysis of metals was conducted by the Trace Element Research Laboratory at Texas A&M Universi[KS1]ty which used a modification of the EPA Method 200 Series.  For metals, samples of marine sediment were digested using heated Nitric and Hydrochloric Acids, then analyzed using ICP/MS (Ag, Al, Be, Cd, Cr, Cu, Fe, Ni, Pb, Sb, and Zn), Graphite Furnace AA (As and Se), or Cold Vapor AA (Hg).DATA SET VERIFICATION:  Data set verified by laboratory chemists and QA officers at each of the agencies and by SCCWRP.

GENERAL COMMENTS:  This data set includes isomers and metabolites of DDT as ng/g and 14 different metals as ug/g (all on dry weight basis).  Each record has the following fields (variables): Log Number, Parameter Code, QA batch, Lab Rep, Qualifier, Result (dry weight basis), Units, True Value, MDL, Analysis Date, QA Code, Lab Code, Comments.

The Log Number is a 10 digit code in the following form:

Digit

______

1     -  P for SCBPP sample

Q for Quality Assurance sample

2 to 3-  Agency collecting the sample

4 to 5-  Type of data (OR = Organics, MT = Metals)

6 to 9-  Station number

0000 if instrument blank

CRM0 if certified reference material

LCM0 if laboratory control material

MS00 if matrix spike

10    -  Sampling Replicate number (0 or 1)

Cumulative batch number if QA Sample.

Parameter code represents the analyte of interest.

Lab rep is a two or three digit code with the following key:

S1  -  Original sample analysis

S2  -  Duplicate sample analysis

B1  -  Instrument Blank analysis

Bx  -  Replicate blank analyses, x = replicate number

BS1 -  Blank spike analysis

BSx -  Blank spike replicate analysis, x = replicate number

R1  -  CRM or LCM analysis

Rx  -  Replicate CRM or LCM analyses, x = replicate number

M1  -  Matrix spike analysis

M2  -  Matrix spike duplicate analysis

D1  -  Interlaboratory Split sample analysis

Dx  -  Interlaboratory Split sample replicate analysis

QA Batch is an alphanumeric code which links SCBPP samples to QA samples.

Qualifier is a one digit code with the following key;

[blank]-  not qualified

ND     -  not detected

<      -  Estimated quantity below reported detection limit

Result is the quantified value for the sample (blank if ND).

Units for each record (ug/dry g for inorganics, ng/dry g for organics).

True Value is only applicable for matrix or blank spiked samples (Lab Code = Mx, BSx) or reference materials (Lab Code = Rx).  Otherwise, this field is blank.

MDL is the method detection limit.

Analysis date (MM-DD-YY).

QA Code is a one character data qualifier with the following key:

[blank] - not qualified,

Q - Only Station 964 was qualified since it was sampled out of range (See Master Sample File for details).

Lab code identifies laboratory doing the analysis.  See Table 1 for a list of codes.

Comments is a field used for any unique notations.

Station locations can be found in the Master Sample Files.  Complete raw data and specifics regarding sampling, analysis and QA/QC are available from SCCWRP upon request.

FIELD AND LABORATORY TECHNIQUES:  Surficial sediments (top 2 cm) from each grab were collected using a stainless steel or Teflon coated scoop and placed in pre-cleaned glass jars.  Jars were kept on ice until transported to the laboratory whereupon each sample was frozen and kept at -20 degrees C until analyzed. Sediment samples were collected by four public agencies (City of Los Angeles Hyperion Treatment Plant-Environmental Monitoring Division, Sanitation Districts of Los Angeles County, Sanitation Districts of Orange County, and City of San Diego Metropolitan Wastewater Department) and one private contractor (MEC Analytical Systems, Inc.)

QUALITY ASSURANCE AND QUALITY CONTROL EVALUATION: The SCBPP used performance-based quality assurance objectives for sediment chemistry.  Each laboratory was to have demonstrated initial capability and ongoing performance.  The SCBPP Quality Assurance Project Plan lists minimum ongoing QA/QC requirements for sediment chemistry analysis.  Included in this data set are QA/QC documentation for:

Laboratory blanks - minimum 1 per batch, < 3 x MDL

Matrix Spike(s) - minimum 1 per batch; 50 - 120% recovery for 80% of the organic analytes or 70 - 120% recovery for 80% of the inorganic analytes; relative difference of duplicate organic or inorganic analysis < 30%.

Reference Materials (CRM or LCM) -	minimum 1 per batch; within 30% of certified value for organic analytes > 10 x MDL or within 20% of certified value for inorganic analytes > 10 x MDL; < 30% percent difference of organic or inorganic analytes > 10 x MDL for all batches combined.  Certified Reference Material (CRM) for organics was NIST Standard Reference Material 1941a.  Certified Reference Material (CRM) for inorganics was National Research Council of Canada MESS-2.  Each laboratory provided its own LCM.

Duplicate samples - no applicable requirement for organics.  Relative percent difference for inorganics < 20%.

DATA USE AND COMPARABILITY:  The data are applicable for analysis of benthic conditions (10-200 meters depth)in the Southern California Bight.  These data represent the general summer condition for 1994 and are appropriate for use with other data collected during the SCBPP.

The sample design is based on the random grid sampling design used in EPA's Environmental Monitoring and Assessment Program.  The data were collected to estimate the amount of area within a particular characteristic (e.g., Copper < 270 ug/dry g).  Since the grid design affects the calculation of variance, care should be taken in making statistical comparisons between this data and data collected in fixed-station or stratified random sampling.  See the SCBPP overview file for more information.

Although 249 Stations were analyzed, only 248 are applicable to the SCBPP statistical design.  Station 964 was sampled out of range and has been qualified for use in area statistical calculations.  It has met all chemistry QA/QC objectives and is valid data for uses other than the SCBPP statistical design.

TABLE 1.  List of participating laboratories.

Code     Laboratory

HY       Hyperion Treatment Plant, City of Los Angeles

LA       County Sanitation Districts of Los Angeles County

PA       Pace Analytical, Inc.

OC       County Sanitation Districts of Orange County

SC       SCCWRP

SD       City of San Diego, Metropolitan Wastewater Department

TX       Trace Element Research Laboratory, Texas A&M University

[KS1]