GRAIN SIZE ANALYSIS METADATA

April 4, 1996

PROGRAM:  Southern California Bight Pilot Project (SCBPP)

SOURCE:  Southern California Coastal Water Research Project (SCCWRP)

CONTACT:  Mary Bergen

PHONE:  (714) 894-2222, (714) 894-9699 (FAX)

NUMBER OF STATIONS SAMPLED:  251 stations were sampled for infauna, but insufficient sample was available for sediment chemistry or grain size analysis at Stations 937, 1131, and 1916.  Three additional stations (1791, 1839, and 2001) were sampled, but were too coarse for the grain size analytical techniques used herein.

SAMPLING PERIOD:  2 July 1994 -- 22 August 1994

NUMBER OF VARIABLES MEASURED:  1: Grain Size (Particle Size).  Data are reported in 11 different phi size classes.

SAMPLING EQUIPMENT USED:  The survey used a modified, chain-rigged Van Veen grab sampler to obtain benthic samples.

ANALYTICAL TECHNIQUES:  Grain Size (particle size) was measured using a Horiba Model LA-900 Laser Analyzer.  All laboratory analyses were conducted by the City of San Diego.  An aliquot of homogenized sample was introduced to the instrument sample reservoir.  The sample was then ultra-sonically dispersed and circulated through the measuring cell which determines the varying particle sizes by measuring scattered (refracted and reflected) laser light.  Seventy-four size classes of particles between 1 and 1,000 microns were measured and then appropriately summed into one phi intervals.  The instrument cannot measure particle sizes greater than 1.0195 mm.  Therefore, these data cannot be used to quantify percent fine-grained material (i.e. < 63 microns) for an entire sample.  That is, the percent of particles less than 63 microns are not equivalent to the percent silt+clay.  This instrument quantifies the number of particles in a given size class.  Consequently, the units are reported as percent of occurrence and not percent by mass.

DATA SET VERIFICATION:  Data set verified by the City of San Diego and SCCWRP.

GENERAL COMMENTS:  This data set includes percent of particles in each of 10 phi sizes between 0 and 10 phi.  Each record has the following fields (variables): Log Number, Parameter Code, QA Batch, Lab Rep, Qualifier, Value (as % of particles in sample), Analysis Date, QA Code, Lab Code, Comments.

The log number is a 10 digit code in the following form:

Digit

1       -     P for SCBPP sample

2 to 3  -     Agency collecting the sample

4 to 5  -     Type of data (GS = Grain Size)

6 to 9  -     SCBPP Station Number

10      -     Sample Replicate Number (0=sample rep 1)

Parameter code represents the phi size of interest.  Eleven different phi sizes are reported and correspond to instrument analysis and particle diameter as follows:

Parameter   Channels       Diameter             Phi

Code        Summed         Microns)             Size

>0          not measured   greater than 890.1   not analyzed

>1          70-74          517.2 - 890.1        0 - 1

>2          65-69          262.3 - 451.5        1 - 2

>3          60-64          133.1 - 229.0        2 - 3

>4          55-59          67.7 - 116.2         3 - 4

<4          50-54          34.4 - 58.9          4 - 5

<5          45-49          17.4 - 29.9          5 - 6

<6          40-44          8.8 - 15.2           6 - 7

<7          35-39          4.5 - 7.7            7 - 8

<8          30-34          2.3 - 3.9            8 - 9

<9          24-29          1.0 - 2.0            9 - 10

Lab rep is a one digit code with the following key:

1     -     Original sample analysis

2     -     Duplicate sample analysis (26 total)

QA Batch is a code which links SCBPP samples to QA samples.  (35 different batch numbers).

Qualifier is a two digit code with the following key:

[blank] -     not qualified

NA      -     Not Analyzed, refers to PHI >0

Value is the percent of particles per sample (blank if NA) and can range from 0 to 100.0 percent.

Analysis date (MM-DD-YY).

QA Code is null for this data set.

Lab code identifies the analytical laboratory (SD=City of San Diego).

Comments is a field used for any unique notations.

Station locations can be found in the Master Sample Files.  Complete raw data and specifics regarding sampling, analysis and QA/QC are available from SCCWRP upon request.

QUALITY ASSURANCE AND QUALITY CONTROL EVALUATION:  The SCBPP used performance-based quality assurance objectives for sediment chemistry.  Each laboratory was to have demonstrated initial capability and ongoing performance.  The SCBPP Quality Assurance Project Plan lists minimum ongoing QA/QC requirements for Grain Size Analysis.  Included in this data set are QA/QC documentation for:

Duplicate sample analysis - minimum 1 per 25 samples, relative difference (RPD) < 20%

The quality of this data set is very high.  The RPD from duplicate sample analysis was greater than 20% in 11 of 2706 records (9 samples).  In nine records, the value field was extremely low (less than 1% of the particle distribution for the entire sample) which amplifies the RPD.  The remaining three records (from Stations 943 and 1096) comprised 8 and 4% of the entire sample, respectively.

FIELD AND LABORATORY TECHNIQUES:  Surficial sediments (top 2 cm) from each grab were collected using a stainless steel or Teflon coated scoop and placed in pre-cleaned glass jars.  Jars were kept on ice until transported to the laboratory whereupon each sample was and kept at 4 degrees C until analyzed. Sediment samples were collected by four public agencies (City of Los Angeles Hyperion Treatment Plant-Environmental Monitoring Division, Sanitation Districts of Los Angeles County, Sanitation Districts of Orange County, and City of San Diego Metropolitan Wastewater Department) and one private contractor (MEC Analytical Systems, Inc.).

DATA USE AND COMPARABILITY:  The data are applicable for analysis of benthic conditions (10-200 meters depth)in the Southern California Bight.  These data represent general summer condition for 1994 and are appropriate for use with other data collected during the SCBPP.

The sample design is based on the random grid sampling design used in EPA's Environmental Monitoring and Assessment Program.  The data were collected to estimate the amount of area within a particular characteristic (e.g., Grain Size phi <4).  Since the grid design affects the calculation of variance, care should be taken in making statistical comparisons between this data and data collected in fixed-station or stratified random sampling.  See the SCBPP overview file for more information.