TOTAL ORGANIC CARBON/TOTAL NITROGEN (TOC/TN) METADATA

April 4, 1996

PROGRAM:  Southern California Bight Pilot Project (SCBPP)

SOURCE:  Southern California Coastal Water Research Project (SCCWRP)

CONTACT:  Mary Bergen

PHONE:  (714) 894-2222, (714) 894-9699 (FAX)

NUMBER OF STATIONS SAMPLED:  249

SAMPLING PERIOD:  2 July 1994 -- 22 August 1994

NUMBER OF VARIABLES MEASURED:  2: Total Organic Carbon (TOC) and Total Nitrogen (TN)

SAMPLING EQUIPMENT USED:  The survey used a modified, chain-rigged Van Veen grab sampler to obtain benthic samples (0.1 meter square surface area).

ANALYTICAL TECHNIQUES:  TOC and TN were analyzed using a Carlo-Erba Model 1108 CHN Elemental Analyzer equipped with an AS/23 Autosampler. All laboratory analyses were conducted by SCCWRP.

DATA SET VERIFICATION:  Data set verified by chemists and biologists at SCCWRP.

GENERAL COMMENTS:  This data set includes Total Organic Carbon and Total Nitrogen as percent dry weight.  Each record has the following fields (variables): Log Number, Parameter Code, Lab Rep, QA batch, Qualifier, Result (dry weight basis), Units, True Value, MDL, Analysis Date, QA Code, Lab Code, Comments.

The log number is a 10 digit code in the following form:

------------------------------------------------------------

Digit  -  Code

------------------------------------------------------------

1      -  P for SCBPP sample, Q for Quality Assurance sample

2 to 3 -  Agency collecting the sample

4 to 5 -  Type of data (TO = TOC/TN analysis)

6 to 9 -  Station number if SCBPP sample, 0000 if instrument blank, CRM0 if certified reference material

10     -  Sample Replicate number.

------------------------------------------------------------

Parameter code represents the analyte of interest (either TOC or TN).

Lab rep is a two digit code with the following key:

S1   -   Original sample analysis

S2   -   Duplicate sample analysis (once per 15 samples)

B1   -   Instrument Blank analysis

Bx   -   Replicate instrument blank analyses

R1   -   CRM analysis

Rx   -   Replicate CRM analyses

QA Batch is a one digit code which links SCBPP samples to QA samples.

Qualifier is a one digit code with the following key;

[blank]-   not qualified

nd     -   not detected

Result is the quantified value for the sample (blank if nd).

Units for this data base are all percent (%, dry weight basis).

True Value is only applicable for CRM samples, otherwise this field is blank.

MDL is the method detection limit.

Analysis date (MM-DD-YY).

Lab code identifies laboratory doing the analysis (SC for SCCWRP).

Comments is a field used for any unique notations.

Station locations can be found in the Master Sample Files.  Complete raw data and specifics regarding sampling, analysis and QA/QC are available from SCCWRP upon request.

FIELD AND LABORATORY TECHNIQUES:  Surficial sediments (top 2 cm) from each grab were collected using a stainless steel or Teflon coated scoop and placed in pre-cleaned glass jars.  Jars were kept on ice until transported to the laboratory whereupon each sample was frozen and kept at -20 degrees C until analyzed. Sediment samples were collected by four public agencies (City of Los Angeles Hyperion Treatment Plant-Environmental Monitoring Division, Sanitation Districts of Los Angeles County, Sanitation Districts of Orange County, and City of San Diego Metropolitan Wastewater Department) and one private contractor (MEC Analytical Systems, Inc.)

QUALITY ASSURANCE AND QUALITY CONTROL EVALUATION:  The SCBPP used performance-based quality assurance objectives for sediment chemistry.  Each laboratory was to have demonstrated initial capability and ongoing performance.  The SCBPP Quality Assurance Project Plan lists minimum ongoing QA/QC requirements for TOC/TN analysis.  Included in this data set are QA/QC documentation for:

Laboratory blanks - minimum 1 per batch, < 3 x MDL

CRM - minimum 1 per batch, < 15% of certified value

Duplicate samples - minimum 1 per batch, relative difference < 30%

Acetanilide (71.09% C and 10.36% N was used as initial and continued calibration standard.  Acetanilide and cyclohexanone (51.79% C and 20.14% N) were used as QC check standards.  Certified Reference Material (CRM) standard was PACS-1 (3.69% C, National Research Council).  All QA/QC requirements were met for this data set.

DATA USE AND COMPARABILITY:  The data are applicable for analysis of benthic conditions (10-200 meters depth)in the Southern California Bight.  These data represent general summer condition for 1994 and are appropriate for use with other data collected during the SCBPP.

The sample design is based on the random grid sampling design used in EPA's Environmental Monitoring and Assessment Program.  The data were collected to estimate the amount of area within a particular characteristic (e.g., TOC greater than 1%).  Since the grid design affects the calculation of variance, care should be taken in making statistical comparisons between this data and data collected in fixed-station or stratified random sampling.  See the SCBPP overview file for more information.