Sediment Toxicity Bioassay Water Quality Metadata

September 13, 1996

Program:  Southern California Bight Pilot Project (SCBPP)

Source:  Southern California Coastal Water Research Project (SCCWRP)

Contact:  Steven Bay

Phone:  (714) 894-2222,  894-9699 (FAX)

Number of Stations Sampled:  72

Sampling Period:  13 July 1994 -- 22 August 1994

Number Of Variables Measured:  7:  Data for pH, dissolved oxygen, salinity, temperature, total sulfides,

total ammonia and un-ionized ammonia in interstitial water samples from toxicity testing and overlying

water from whole sediment toxicity tests.  Bioassay data appears in the file TOX_DATA.TXT.

Sampling Equipment Used:  Sediment for toxicity sampling was collected with a modified, chain-rigged

0.1m2 Van Veen grab.

Analytical Techniques:  Interstitial water toxicity testing was performed at SCCWRP using embryos of the

purple sea urchin, Strongylocentrotus purpuratus, following methods of the EPAs West Coast Manual.

Whole sediment testing was performed at SCCWRP and SAIC in Rhode Island.  Whole sediment tests used

the amphipod, Ampelisca abdita, by the methods in ASTM.

Data Set Verification:  Data set verified by toxicologists from SCCWRP.

General Comments:

This data set includes minimum and maximum values for water quality parameters measured

during the performance of toxicity tests for the SCBPP.  Each record has the following fields (variables):

Log Number, QA Batch, Lab Code, Parameter, Dilution, Minimum and Maximum.  The log number is a 10

digit code in the following form:

Digit

1                   -                           P for Pilot Project

2 & 3               -                           Agency collecting the sediment

4 & 5               -                           Type of data (ST = Sediment toxicity)

6 to 9               -                           Station number

10                 -                            Replicate number.

For agency collecting sediment, EC = Control sediment collected by SCCWRPs amphipod supplier; SA =

Control sediment collected by SAIC in Rhode Island;  SC= Centrifugation blank created in the SCCWRP

laboratory;  HY= Hyperion Treatment Plant;  LA= County Sanitation Districts of Los Angeles County;

ME= MEC Analytical Systems, Inc. and SD= City of San Diego.  The replicate digit will be 0 in all cases

except when a sample has been split between SCCWRP and SAIC for toxicity testing.  In those cases, the

replicate value is 0 for the sample tested by SCCWRP and 1 for the sample tested by SAIC.

QA Batch is the internal experiment designation by either SCCWRP or SAIC.

Lab Code is the two letter code for the laboratory that performed the bioassay.  Either SC for SCCWRP or

NG for SAIC in Rhode Island.

Parameter is the water quality parameter measured.  PH = pH; DO = dissolved oxygen in milligrams/liter;

SAL = Salinity in grams/kilogram; TEMP = temperature in degrees centrigrade;  NH3T = total ammonia in

milligrams/liter;  STP = total sulfides in milligrams/liter;  NH3U= un-ionized ammonia in milligrams/liter.

Dilution is percentage of sample that was interstitial water.  Null indicates 100% sample.

Station locations can be found in the Benthic Master Sample File, except for the following three control

sites.  All log numbers having the EC agency code, except station 3002, were a negative control station

where the amphipods were collected in Rhode Island (lat:41.443889, long:71.456944).  Station 3002 was a

positive control site for the amphipod test.  This station was located in Wickford Harbor, Rhode Island

(lat:41.582222,long:71.425).  All stations with the SA agency code were for the negative control station

used by SAIC for there amphipod tests.  This station is located in Long Island Sound (lat:41.1325,

long:72.87833).  Descriptive data on sediment color and texture and depth of penetration of the grab for

SCBPP stations are available from SCCWRP on request.

Field and Laboratory Techniques:  Measurements of pH were made using an Orion model 91-157 pH

Triode electrode attached to a Orion Model 290A portable meter.  Dissolved oxygen was measured with an

Orion Model 810 portable meter/electrode.  Conductivity was measured with an Orion model 124

meter/probe.  The conductivity measurements were converted to salinity mathematically.  Temperature was

continuously monitored during the experiments using electronic thermometers.  Total sulfides were

measured through a calorimetric method.  Total ammonia was measured using an Orion model 95-12

specific ion electrode attached to a Corning model 150 benchtop meter.  Un-ionized ammonia was

calculated by the methods of Hampson (1977).

Quality Assurance And Quality Control Evaluation:  Calibrations were performed on all water quality

measurement apparatus on the days they were used.  Duplicate measurements were made on a subset of

samples to verify instrument reproducibility and stability.

Data Use and Comparability:  The data are appropriate for determining water quality conditions during the

toxicity tests.  The data are appropriate for comparing conditions between exposures and between the

laboratories that conducted the bioassays.  Since the data were not collected in the field, they should not be

used for determining water quality conditions in the field.

For the interstitial water data, there was an interference in toxicity data caused by ammonia.

Ammonia levels were found to have increased greatly during storage of the sediments and led to many

samples appearing to be more toxic than would have been expected if no ammonia was present.  The

toxicity data must therefore be viewed in light of the water quality data.