Sediment Toxicity Bioassay Metadata

August 28, 1996

Program:  Southern California Bight Pilot Project (SCBPP)

Source:  Southern California Coastal Water Research Project (SCCWRP)

Contact:  Steven Bay

Phone:  (714) 894-2222,  894-9699 (FAX)

Number of Stations Sampled:  72

Sampling Period:  13 July 1994 -- 22 August 1994

Number Of Variables Measured:  2:  Interstitial water toxicity measured as number of normal embryos and

whole sediment toxicity measured as number of surviving amphipods.  Water quality data from the

exposures appears in the file TOX_WQ.TXT.  Since the interstitial water tests were affected by

interference from ammonia, data from interstitial water tests should not be used without consideration of

this factor.  See "Data Use and Comparability" for more information.

Sampling Equipment Used:  Sediment for toxicity sampling was collected with a modified, chain-rigged

0.1m2 Van Veen grab.

Analytical Techniques:  Interstitial water toxicity testing was performed at SCCWRP using embryos of the

purple sea urchin, Strongylocentrotus purpuratus, following methods of the EPAs West Coast Manual.

Whole sediment testing was performed at SCCWRP and SAIC in Rhode Island.  Whole sediment tests used

the amphipod, Ampelisca abdita, by the methods in ASTM.

Data Set Verification:  Data set verified by toxicologists from SCCWRP.

General Comments:

This data set includes number of embryos counted and number normal for interstitial water tests

and the starting number of amphipods and number surviving for the whole sediment tests.  Each record has

the following fields (variables):  Log Number, QA Batch, Species, Phase, Dilution, ControlCode, Endpoint,

Lab Rep, Start Count, Value, QA Code, and Lab Code.  The log number is a 10 digit code in the following

form:

Digit

1             -            P for Pilot Project

2 & 3           -            Agency collecting the sediment

4 & 5           -            Type of data (ST = Sediment toxicity)

6 to 9          -            Station number

10            -            Replicate number

For agency collecting sediment, EC = Control sediment collected by SCCWRPs amphipod supplier; SA =

Control sediment collected by SAIC in Rhode Island;  SC= Centrifugation blank created in the SCCWRP

laboratory;  HY= Hyperion Treatment Plant;  LA= County Sanitation Districts of Los Angeles County;

ME= MEC Analytical Systems, Inc. and SD= City of San Diego.  The replicate digit will be 0 in all cases

except when a sample has been split between SCCWRP and SAIC for toxicity testing.  In those cases, the

replicate value is 0 for the sample tested by SCCWRP and 1 for the sample tested by SAIC.

QA Batch is the internal experiment designation by either SCCWRP or SAIC.

Species is for test species, either SP for Strongylocentrotus purpuratus or AB for Ampelisca abdita.

Phase is for phase of sediment tested, either SP for solid phase (whole sediment) or WP for water phase

(interstitial water).

Dilution is percentage of sample that was interstitial water.  Null indicates 100% sample.  All whole

sediment tests were 100% whole sediment.

ControlCode is the log number of the control sample associated with the particular record.

Endpoint is the measured endpoint for the record, either NN for number normal in embryo tests or NA for

number alive in amphipod tests.

Lab Rep is the replicate number for the particular record.

Start Count is either the total number of embryos counted for the urchin test or the number of amphipods

added to the beaker at the start of a whole sediment test.

Value is either the number of normal embryos for the urchin test or the number of surviving amphipods for

that test.

QA Code is a letter designation for deviations from the QA parameters set forth for the project.  More than

one letter may be present for any given record:

A       All QA criteria met

C       Reduced number of replicates

D       Control performance criteria not met

E       Sample stored greater than 30 days

J       Minor deviation in test conditions

N       Endpoint not evaluated for this record

Lab Code is the 5 letter code for the laboratory that performed the bioassay.  Either CASCC for SCCWRP

or RISAI for SAIC in Rhode Island.

Station locations can be found in the Benthic Master Sample File, except for the following three control

sites.  All log numbers having the EC agency code, except station 3002, were a negative control station

where the amphipods were collected in Rhode Island (lat:41.443889, long:71.456944).  Station 3002 was a

postitive control site for the amphipod test.  This station was located in Wickford Harbor, Rhode Island

(lat:41.582222,long:71.425).  All stations with the SA agency code were for the negative control station

used by SAIC for there amphipod tests.  This station is located in Long Island Sound (lat:41.1325,

long:72.87833).  Descriptive data on sediment color and texture and depth of penetration of the grab for

SCBPP stations are available from SCCWRP on request.

Field and Laboratory Techniques:  Sediment for toxicity analysis was collected with a modified 0.1m2 Van

Veen grab.  A minimum of three liters of surface (top 2 cm) sediment was collected from each station.

Sediment was stored in one liter polyethylene jars at 3-5  C for up to four weeks.  For the amphipod tests,

sediment was homogenized by hand then passed through 1 mm screen for samples tested at SCCWRP or 2

mm screen for samples tested at SAIC.  A 2 cm layer of sediment was added to quart-sized canning jars

with 600 ml of 30 g/kg seawater on top.  Five replicates from each station were tested.  Twenty amphipods

were added the next day and the test was carried out for 10 days at 20  C, with survival as the endpoint.

Interstitial water was obtained by centrifuging approximately 400 ml of homogenized, unscreened

sediment at 3,000 x g for 30 min.  Samples with pH less than 7.5 were adjusted to 7.6-8.0.  Three

concentrations of interstitial water (100, 50 and 25%) were prepared if sufficient sample volume was

available.  Three replicates of each concentration were tested.  Purple sea urchin embryos were added to

the interstitial water and allowed to develop for 72 h at 15  C.  Embryo samples were then preserved and

examined microscopically for normal development.

Quality Assurance And Quality Control Evaluation:  Most of the tests met the QA criteria set forth in the

SCBPP Quality Assurance Plan.  One amphipod survival test failed to meet the criteria and retesting of

these samples resulted in several samples being stored greater than 30 days and having less than 5

replicates.  An amphipod test was not performed for station 245 due to not enough sample volume.  Some

concentrations of interstitial water were not done because of insufficient sample volume.  Many of the

interstitial water samples have reduced numbers of replicates.  Reduced replication resulted when the lower

concentrations exhibited complete toxicity (0% normal).  When lower concentrations exhibited complete

toxicity, only one replicate of the higher concentration was examined microscopically to confirm toxicity.

Reference toxicant tests for both the urchin and amphipod bioassays indicated that the animals

were within expected sensitivity ranges for all experiments.  For 16 of the stations, splits of the whole

sediment were made and amphipod tests were performed by both SCCWRP and SAIC.  Both sets of data

appear in the file and there is good agreement between the results.

Data Use and Comparability:  The data are appropriate for the comparison between stations at the time of

sampling.  The data are appropriate for the use with other data collected during the SCBPP.

For comparison of data between different bioassays within the SCBPP toxicity data set, the data

should be normalized to the control values.

For the interstitial water data, there was an interference by ammonia.  Ammonia levels were found

to have increased greatly during storage of the sediments and led to many samples appearing to be more

toxic than would have been expected if no ammonia was present.  SCCWRP has processed the data to

remove the ammonia interference.  After processing, each station was designated as toxic, non-toxic or

indeterminate.  That information appears in the file TOX_TOX.TXT.