STATION MASTER SAMPLE FILE METADATA

May 14, 1996

PROGRAM:  Southern California Bight Pilot Project (SCBPP)

SOURCE: Southern California Coastal Water Research Project (SCCWRP)

CONTACT:  Dr. M. James Allen

PHONE: (714) 894-2222

NUMBER OF STATIONS SAMPLED: 140 stations were designated for sampling.  Seventeen stations were abandoned because of unsuitable bottom conditions (i.e. rocky and or irregular bottoms).  Four stations were abandoned because they were too deep, three stations because the net did not fish and one because it was south of the U.S.-Mexico border.  A total of 114 stations were sampled.

SAMPLING PERIOD:   12 July 1994 -- 22 August 1994

NUMBER OF VARIABLES MEASURED:  3:  Latitude and Longitude ( to 0.00001 degrees) plus depth (meters).  Other variables exist including weather conditions and sampling identifiers and qualifiers.  See GENERAL COMMENTS for a list of other information recorded.

SAMPLING EQUIPMENT USED:  The survey used a semiballoon otter trawl with the following net dimensions:  7.6-m headrope (25 ft); 8.8-m footrope (29 ft); 3.8-cm (1.5 in) body mesh; and a 1.3-cm cod-end mesh (0.5 in).  The nets had 22.9-m (75 ft) long bridles made of 1.0-1.6 cm (3/8 to 5/8 in) diameter rope.  The otter boards (doors) had a width of 76.2 cm (30 in), height of 50.8 cm (20 in), and a suggested weight of 15.9 kg (35 lb).  The amount of wire used varied from 54 to 1500 m (16-457 ft).

ANALYTICAL TECHNIQUES:  Navigation was accomplished using Global Positioning System (GPS).  Coordinates were based upon North American Datum 83 (NAD 83).  Depth was measured using a fathometer.

DATA SET VERIFICATION:  Data sets were verified by the cruise leader aboard each sampling vessel and by Dr. M. James Allen and Shelly Moore at SCCWRP.

GENERAL COMMENTS:  This data set includes location, date, time of collection of samples, organization collecting the sample, and weather and sea conditions at the time of sampling.  Each record has the following fields (variables): Station,  Lognumber, Date, Depth, Agency, Vessel, Weather, WindSpeed, WindDirection, SwellHeight, SwellPeriod, OverTime, OvrlLat, OvrLong, BeginTime, BeginLat, BeginLong, EndTime, EndLat, EndLong, DeckTime, DeckLat, DeckLong, TrawlSpeed, TrawlDirection, TrawlDuration, Wireout, Equipment, FailReason1stTrawl, FailReason2ndTrawl, FailReason3rdTrawl, FailReason4thTrawl, FailReason5thTrawl, FailReason6thTrawl, SPECIES1, SAMPID1, T1CNT1, T2CNT1, T3CNT1, TOTCNT1, SPECIES2, SAMPID2, T1CNT2, T2CNT2, T3CNT2, TOTCNT2, SPECIES3, SAMPID3, T1CNT3, T2CNT3, T3CNT3, TOTCNT3, SPECIES4, SAMPID4, T1CNT4, T2CNT4, T3CNT4, TOTCNT4, StationAcceptability, StationAcceptComments, StationFail, FailCode, Comments.

Station is a 4-digit code noncontinuous from 009 to 2011.  A total of 140 stations are identified.

LogNumber is a 10-digit code in the following form:

Digit

1	-	P for SCBPP sample

2 to 3	-	Agency collecting the sample (see codes below)

4 to 5 	- 	Type of data (FT = Fish Trawls)

6 to 9	-	SCBPP station number (see Station above)

10	-	Sample replicate number (0 = sample rep 1)

Date (MM-DD-YY)

Depth (meters) is the bottom depth at the time of sampling.  Depths were not corrected for tides.  Sampling depths ranged from 9 to 215 m.

Agency indicates which of 5 groups were responsible for sampling at that station:

HY = City of Los Angeles, Environmental Monitoring Division (Hyperion 	Treatment Plant)

LA = County Sanitation Districts of Los Angeles County

OC = County Sanitation Districts of Orange County

SD = City of San Diego, Metropolitan Wastewater Department

SC = Southern California Coastal Water Research Project

Vessel indicates the name of the boat used.

Weather is a one-digit code with the following format:

0 = Clear (no clouds at any level)

1 = Partly cloudy (scattered or broken)

2 = Continuous layers of clouds

3 = Sand storm, dust storm, or blowing storm

4 = Fog, thick dust or haze

5 = Drizzle

6 = Rain

7 = Snow, or rain and snow mixed

8 = Showers

9 = Thunderstorms

WindSpeed (miles per hour, mph) is the wind speed at the time of sampling.

WindDirection (degrees from magnetic north) is the wind direction at the time of sampling.

SwellHeight (feet) is the swell height at the time of sampling.

SwellPeriod (seconds) is the swell period at the time of sampling.

OverTime, BeginTime, EndTime, and DeckTime (HH:MM, 24 hour clock) are the times the net went into the water, began to trawl, stopped trawling, and came out of the water, respectively.

OvrlLat, OvrLong, BeginLat, BeginLong, EndLat, EndLong, DeckLat, and DeckLong (degrees north and west, respectively) are the recorded latitudes and longitudes at the time the net went into the water, began to trawl, stopped trawling, and came out of the water, respectively.

TrawlSpeed is the speed of the boat during the trawl in knots.

TrawlDirection is a number from 0 to 360 (degrees from magnetic North) representing the tow direction of the trawl.

TrawlDuration (minutes) is the amount of time the net trawled the bottom.

Wireout (meters) is the length of wire paid out during the trawl.

Equipment is a two-digit code representing the type of sampling equipment used.  In this case only one code was listed (26 = 7.6-m headrope otter trawl).

FailReason(1-5)thTrawl was left blank or was the number one indicating that there was a trawl failure.

FailReason6thTrawl is a field used for any unique trawl failure notations.  Maximum number of characters is 80.

SPECIES1, SPECIES2, SPECIES3, and SPECIES4 consists of a two letter code representing fish collected for tissue burden analysis:

LS = longfin sanddab (Citharichthys xanthostigma)

PS = Pacific sanddab (Citharichthys sordidus)

HT = hornyhead turbot (Pleuronichthys verticalis)

CS = California scorpionfish (Scorpaena guttata)

DS = Dover sole (Microstomus pacificus)

SS = speckled sanddab (Citharichthys stigmaeus)

WC = white croaker (Genyonemus lineatus)

ES = English sole (Pleuronectes vetulus)

SAMPID1, SAMPID2, SAMPID3, and SAMPID4 consists of a 10-digit code in the following form:

Digit

1	-	P for SCBPP sample

2 to 3	-	Agency collecting the sample (see codes above)

4 to 5 	- 	Type of fish collected for tissue burden analysis (see codes above)

6 to 9	-	SCBPP station number (see Station above)

10	-	Sample replicate number (0 = sample rep 1)

T1CNT1, T2CNT1, T3CNT1, TOTCNT1, T1CNT2, T2CNT2, T3CNT2, TOTCNT2, T1CNT3, T2CNT3, T3CNT3, TOTCNT3, T1CNT4, T2CNT4, T3CNT4, and TOTCNT4 represent the numbers of tissue burden fish caught for each of three trawls and the total for all three trawls, respectively.

StationAcceptabilility consists of Y/N (Yes/No) representing whether the trawl was acceptable or not.

StationAcceptComments represents any unique notations regarding the acceptability of a station.

StationFail consists of a one letter code, Y or N, indicating an unsuccessful or successful trawl, respectively.

FailCode was left blank or was the number one, indicating a trawl success or failure, respectively.

Comments is a field used for any unique notations.  Maximum number of characters is 80.

FIELD AND LABORATORY TECHNIQUES: A semiballoon otter trawl with a 7.6-m headrope (25 ft), 8.8-m footrope (29 ft), 3.8-cm (1.5 in) body mesh, and a 1.3-cm cod-end mesh (0.5 in) was used to collect samples.  Once the net was brought onboard, fish and invertebrates were identified to species, counted and weighted to the nearest 0.1 kg.  Fish were measured to centimeter size classes (board standard length for bony fish and total length for cartilaginous fishes).  A centimeter size class extends from 0.1 to 1.0 cm with a midpoint of 0.5 cm.  Board (or maximum) standard length extends from the anterior tip of the head (snout or lower jaw) to the base of the tail (end of the hypural plate).  Total length extends from the anterior tip of the head to the tip of the tail.  Qualitative estimates of numbers and weight were made for debris.

QUALITY ASSURANCE AND QUALITY CONTROL EVALUATION:  The SCBPP Quality Assurance Project Plan and SCBPP Field Operations Manual list minimum ongoing QA/QC requirements for trawl sampling.  Each agency was responsible for minimum equipment and training requirements.

DATA USE AND COMPARABILITY:  The data are applicable for locating sampling stations used in the SCBPP and provide information on weather conditions during sampling.  These data are appropriate for use with other data collected during the SCBPP.

See the SCBPP overview file for more information about the sampling design.