ANNUAL REPORT

KAHE GENERATING STATION

NPDES MONITORING PROGRAM

FOR 1996

Prepared for:

Environmental Department

Hawaiian Electric Company
P. O. Box 2750

Honolulu, Hawaii  96840

Prepared by:

AECOS Inc.

970 N. Kalaheo Ave., Suite C300

Kailua, Hawaii  96734

February 1997

EXECUTIVE SUMMARY

The Hawaiian Electric Company Inc. (HECO), under requirements of National Pollutant Discharge Elimination System (NPDES) Permit HI0000019, Section C.5, annually conducts and reports results of a biological monitoring program for waters receiving effluent from the Kahe Generating Station, O'ahu, Hawai'i. This report presents results of monitoring conducted during 1996 for water temperature, sand entrainment, and coral coverage. Specific findings are summarized below.

WATER TEMPERATURE

1.	Kahe discharge temperature elevations above the adjusted intake temperature did not exceed the allowable NPDES daily discharge limit of 8.4 C° (15.0 F°) during 1996.

SAND MOVEMENT

2.	Total sand entrainment by the Kahe circulating cooling water system was estimated to be 2318 m3 (3113 yd3) in 1996, all of which was suspended sand throughput. No sand was pumped from either forebay into the discharge line during 1996. Total sand entrainment was the lowest estimated since sand throughput monitoring began in 1977 and nearly half of the previous lowest estimate in 1995.

REEF CORAL

3.	Replicate coral sampling indicated that significant changes in mean coral coverage occurred only at the station 300 m northeast of the Kahe outfall where significant reductions in the coral coverage have occurred in three of the previous four years since Hurricane Iniki in 1992. However, 1995-96 results indicated significant increases in coverage of the two major component species and in total live coral, the first time this has occurred at this station since Hurricane Iwa in 1982. Over the ten year term from 1986 to 1996, this station registered significant reductions in coverage of the two dominant species and in total coral, despite the significant increases in coverage that occurred in 1995-96.

4.	The significant decreases in coral cover from 1986 to 1996 were direct effects from Hurricane Iniki, which occurred in September 1992. The station where these decreases were greatest was that one most exposed to wave turbulence from the hurricane. No other stations showed significant changes in coral cover in 1995-96 nor significant changes in any major coral species or total coverage between 1986 and 1996.

5.	Results from 1996 monitoring support previous years' conclusions that the Kahe offshore outfall has produced negligible impact on the nearshore coral community and that natural storm disturbances have had greater destructive influence on reef coral coverage throughout the Kahe area.

TABLE OF CONTENTS

PAGE

EXECUTIVE SUMMARY	ii

LIST OF TABLES	v

LIST OF FIGURES	vi

1.  INTRODUCTION	1

1.1  Work Statement	1

1.2  Description of Kahe Generating Station and 	1
		       Circulating Water System

1.3  Historical Overview	4

2.  TEMPERATURE STUDIES	5

2.1  Ambient Temperature Records	5

3.  SEDIMENT THROUGHPUT STUDIES	7

3.1 Introduction	7

3.2 Methods	7

3.2.1 Suspended Sediments	7

3.2.2 Bed Load Sediments	8

3.3 Results and Discussion	8

4.  REEF CORAL  STUDIES	16

4.1 Introduction	16

4.2 Replicate Measurement Methods	16

4.3 Data Analysis Methods	16

4.4 Coral Study Results	19

4.5 Discussion of Results	25

5.  LITERATURE  CITED	30

LIST OF APPENDICES

A.  Marine Monitoring Program, Kahe Generating Station, HECO, August 25, 1986

B.  1996 Daily Circulating Water Discharge Temperatures (°F)

C.  Coral  Coverage at Replicate Quadrats, Stations 2B, 6B, 8A and Control, 1986, 1995 and 1996

LIST OF TABLES

TABLE	TITLE	PAGE

1	Operating characteristics of the Kahe
		Generating Station	2

	2	Kahe Generating Station sand (>110 mm)	9
		throughput estimates	

	3	Estimation of 1996 Kahe Generating Station	11
		sand and silt entrainment based on frequency
		of wave occurrence	

	4	Removal of sand bed load from Kahe cooling	13
		water system (1977-1996)	

	5	Total volumes of sand (m3) entrained by Kahe	14
		Power Station estimated by dredging removal
		and suspended throughput, 1976-1996	

	6	Quadrat photograph record for Kahe coral	18
		monitoring stations	

	7	Percent changes in cover for 1995 vs. 1996	23

	8	Average annual percent changes in cover for 	24
		1986 vs. 1996	

	9	Statistical parameters:  1995 vs. 1996 comparisons	25

10	Statistical parameters:  1986 vs. 1996 comparisons	26

LIST OF FIGURES

FIGURE	TITLE	PAGE

1	Site map of Kahe Generating Station at	3
		Kahe Point on O'ahu

2	Kahe adjusted intake vs. discharge	6
		temperatures - 1996

3	Kahe ambient water temperatures, 1986	6
		through 1996

4	Kahe and Nanakuli areas showing location of	17
		coral community control station

5	Kahe vicinity showing coral reef stations	17

6	Mean total coral coverages at Kahe stations	21
		for 1986, 1995, and 1996

7	Mean coverages of Porites lobata, Pocillopora	22
		meandrina, and combined other coral species
		at Kahe stations, 1986, 1995, and 1996

1  INTRODUCTION

1.1  Work Statement

National Pollutant Discharge Elimination System (NPDES) permit HI0000019, renewed on September 12, 1995 (effective date of October 12, 1995), authorizes Hawaiian Electric Company (HECO) to discharge waste water from the Kahe Generating Station. This NPDES permit contains under Section C.5 (Marine Monitoring Program) the following provision:

"Marine Monitoring Program (see Attachment 1), reported by the Permittee on August 25, 1986 and granted by the Department on September 30, 1986, shall constitute part of the requirements of receiving water monitoring programs."

The above-referenced monitoring program is provided in Appendix A of this report. The monitoring plan describes investigations that HECO conducts to document marine effects of cooling-water discharge from Kahe Generating Station Units 1 through 6. Specific studies covered under this program include monitoring of:

A.  Temperature elevations of thermal effluents,

B.  Quantities of sediment entrainment and throughput, and

C.  Reef coral communities.

An annual report of the data, analysis, and findings of these monitoring studies is submitted to the State Department of Health (DOH) Director and Environmental Protection Agency (EPA) Regional Administrator no later than March 1 of each year.

1.2 	Description of Kahe Generating Station and Circulating Water System

The Kahe Generating Station is located in a valley between Kahe Point and Nanakuli on the Wai`anae coast of the island of O`ahu, approximately 18 miles west of Honolulu (Figure 1). The station currently consists of six oil-fired, steam electric generation units with operational characteristics as shown in Table 1.

Cooling water enters the station by way of an intake basin located at the shoreline. Water then flows by gravity feed to two forebays within the station boundaries, one forebay supplying Units 1 through 4 and the other Units 5 and 6. The forebays and intake basin, which are located inside a weir located at minus two meters from mean lower low water (MLLW), are designed to retain enough cooling water to allow station operation in case of sea level subsidence during a tsunami.

Each unit is supplied condenser cooling water from the forebays by two pumps at the combined flow rates shown in Table 1. A traveling screen is located between the forebay and the circulating water pumps. These traveling screens are vertically mounted, 9.5 mm mesh screens. Debris collected on these screens is washed off using a high-pressure, salt-water spray. At Units 1 through 4, debris is then carried by the wash water through a shallow sluice way to a bar rack sump where larger (greater than 75 mm) material is retained and the smaller material and wash water re-enters the circulating water system on the discharge side of the condensers. All debris from Units 5 and 6 is carried by the wash water into a large, heavy mesh collection basket where it is routinely discarded with other facility trash to the City landfill by maintenance personnel. Common sluice ways are shared by Units 1-2, 3-4 and 5-6.

After passing through the condensers, cooling waters from Units 1 through 4 and Units 5 and 6 combine in the discharge transition basin (Figure 1) and are discharged approximately 250 m (820 ft) offshore. The point of discharge is in a sublittoral sand area at a depth of eight meters (Figure 5). This mode of discharge has been utilized ever since the offshore outfall began operation on December 16, 1976.

Table 1. Operating characteristics of the Kahe Generating Station

Date of	Normal	Circulating	Condenser	Heat
		Commercial	Generating	Water Flow	Temp. Rise	Discharge
	Unit	Operation	Capability	(m3x103/day)	(°C)	(Kg cal x 109/day)

1	Apr. 1963	 88	 398.9	6.1	2.45
	2	Nov. 1964	 88	 389.9	6.1	 2.45
	3	Oct. 1970	 90	 412.2	6.1	 2.51
	4	Aug. 1972	 90	 412.2	6.1	 2.51
	5	Dec. 1974	141	 817.9	5.0	 4.01
	6	Dec. 1980	141	 817.9	5.0	 4.01

Totals		638	3193.5		17.94

The average intake velocity is 105.0 cm/sec (3.4 ft/sec). Average discharge velocity is 134.4 cm/sec (4.4 ft/sec) with all units in operation. Based on dye experiments described in a previous NPDES monitoring report (McCain, 1977), the mean transit time for the passage of cooling water through the station from the shoreline intake bar

Figure 1.  Site map of the Kahe Generating Station at Kahe Point on O'ahu.

screen to the outfall of Units 1 through 4 into the discharge transition basin was 4 min 25 sec; to the effluent ports in the transition basin, 5 min 55 sec. The estimated time of transit through the offshore outfall conduits, using a discharge velocity of 134 m/sec, is 2 min 30 sec. Therefore, the total estimated mean transit time through the station for a unit mass of cooling water is 8 min 25 sec.

1.3 Historical Overview

The Kahe Generating Station commenced operation in 1963. Since then, over 50 reports characterizing the marine environment have been prepared for or by HECO. Early consultant reports by Marine Advisors (1964), B. K. Dynamics (1971), and URS Research Co. (1973) described the general baseline conditions of the Kahe nearshore marine environment. The results of these and other studies were summarized in an Environmental Impact Assessment prepared by Stearns-Roger, Inc. (1973) for the expansion of the Kahe Station to six generating units.

After 1973, HECO's Environmental Department conducted extensive NPDES marine monitoring studies to determine the conditions of the Kahe nearshore environment during the station's expansion to six generating units and the relocation of the Kahe shoreline outfall to its present offshore site (Coles and McCain, 1973; Coles and Fukuda, 1975, 1983, 1984; Environmental Department, 1976; McCain, 1977; Coles, 1979, 1980; Coles et al., 1981, 1982, 1985, 1986; Fukuda and Oda, 1987; Environmental Department, 1988, 1989, 1990, 1991, 1992, 1993 1994 and 1995). Several of these reports also document the impacts of major storms on the Kahe nearshore environment that occurred in 1980, 1982 and 1992, and the subsequent recovery of the storm-damaged areas that are influenced by power station operation. These NPDES monitoring reports were submitted to the DOH upon completion and are herewith incorporated by reference.

The following report presents the results of HECO's 1996 marine monitoring investigations conducted at Kahe. These investigations are continuations of the monitoring programs documented in the above-referenced reports.

2.  TEMPERATURE STUDIES

2.1  Ambient Temperature Records

Since January 1, 1986, HECO has used adjusted intake water temperatures as the reference against which temperature elevations of the thermal effluent are measured. Intake temperatures are adjusted downward by 0.42 C° to determine the reference temperature. Based on over 10 years of temperature records collected at an ambient temperature control station, it was determined that intake temperatures exceeded nearshore ambient bottom temperatures by this amount.

Daily records were kept of maximum, minimum, mean effluent, and mean influent temperatures for 1996. Intake and discharge temperatures were recorded continuously using a Foxboro ERB strip chart recorder and DynathermÒ resistance bulbs. The intake temperature resistance bulb is located in Unit 1-4 forebay and the discharge temperature resistance bulb is located in the transition basin before the discharge pipe (Figure 1).

Discharge and adjusted intake temperature data available from daily operation records are plotted in Figure 2.  Daily discharge and unadjusted intake temperature data logs are provided in Appendix B.  Figure 3 shows trends in ambient temperatures that have occurred since 1986. The maximum adjusted intake (i.e., ambient) temperature for 1996 was 27.9°C, the mean was 25.6°C, and the minimum was 21.8°C.

The present ambient temperature collection method provides a sensitive and readily available range from which temperature elevations above ambient can be compared. During 1996, maximum daily discharge temperature elevations above ambient averaged 5.85°C and the maximum was 7.08°C, never exceeding the permitted limit of 8.4°C above ambient.

Figure 2.  Kahe adjusted intake vs. discharge temperatures - 1996.

Figure 3.  Kahe ambient water temperatures, 1987 through 1996.

3.  SEDIMENT THROUGHPUT STUDIES

3.1  Introduction

The volume of sand and silt entrained and circulated by operation of the Kahe Generating Station is an environmental parameter of considerable geological and biological importance. Wave-suspended sediments are carried in cooling water through the station's condensers and discharged from the station's outfall. Sediment entrainment represents a potential loss of sand from the littoral system which consequently affects the supply of sand available to maintain beach configurations in the area. Biologically, deposition of entrained sand off the outfall and the distribution of silt suspended in the thermal plume represent potential stressors for benthic (bottom-dwelling) organisms, which may have no mechanisms for ridding themselves of sediments, or must expend energy to do so. Burial of hard reef surfaces by sand and silt diminishes the surface area available for recruitment of sessile benthic organisms and eliminates habitat of motile benthos and demersal fishes.

HECO has monitored sediment entrainment and throughput related to the station's circulating cooling water system since 1977. The startup of Kahe Unit 6 resulted in a noticeable increase of annual sediment entrainment rates. However, Hurricane Iwa reduced sand levels along the entire Kahe reef front in late 1982, and sediment entrainment values have been steadily declining since. This is because less sand has been available near the intake to be entrained. The following study presents the eighteenth year of sediment entrainment monitoring with six Kahe units in operation.

3.2  Methods

Two categories of sediment were measured to estimate the total amount of sand presently being entrained by the Kahe Generation Station: 1) suspended and 2) bed load sediments.

3.2.1  Suspended Sediments

Sediment throughput samples were taken at Units 1-4 and 5-6 discharge points within the Kahe outfall transition basin (Figure 1). Samples were collected by placing a 0.25-m diameter plankton net directly in the discharge. The 110 mm mesh plankton net was held approximately 10 m behind the discharge ports for at least three minutes at a depth of 0.5 m.  Flow rates determined with a General Oceanics digital flow meter were then used to calculate the total volumes of water sampled. Samples were collected four times in 1996, including two periods of medium to high surf. Pertinent data relating to weather and sea conditions were recorded.

Measured amounts of suspended sediment were converted to units of ml/m3 by dividing by the total volume of seawater sampled. These were then extrapolated to daily throughput estimates by multiplying by the volume of cooling water circulated under operation of all six generating units (3.257 x 106 m3/day).

Sand throughput calculations were based on wave characteristics determined for a typical year at Kahe by Marine Advisors (1964). Since wave action directly influences the amount of sand and sediment suspended in the water column, percent frequencies of occurrence of summer and winter surf conditions were estimated for wave direction and height. Daily sand throughput estimates obtained during the current sampling period were categorized under similar sea and swell conditions, and correlated to Marine Advisers (1964) seasonal frequency-of-occurrence values for respective wave characteristics to determine a one-year sand throughput estimate. Where sand throughput data under certain wave characteristics were unavailable, average throughput data based on historical information were used in the calculations.

3.2.2  Bed Load Sediments

Much of the sediment entrained by the Kahe Generating Station accumulates as bed load at the shoreline intake basin and within the two intake forebays where water motion is insufficient to keep larger particles suspended. Bed load from the forebays is periodically pumped directly into the circulating water system downstream of the condensers and is carried by effluent to the station's outfall. Generally, HECO operations personnel conduct pumping and record dates, durations and pumping rates to enable calculation of the total volume of sediment pumped from the forebays. However, contractors are hired occasionally to pump or dredge sand from the forebays, intake or discharge basin, during which sand removal volumes are based on contractors' estimates. Whenever sediment was dredged, the volume removed was estimated by the number of truck loads required to transport the sediment to its deposition site.

3.3  Results and Discussion

Suspended sediment samples were collected in February, April, September and December 1996. Estimated daily throughput values with coinciding weather and sea conditions for these measurements are listed in Table 2, along with similar data for all samples collected since 1983. Sediment throughput information and corresponding wave information collected since 1983 (i.e., post-Hurricane Iwa) also are summarized in Table 3. Daily throughput estimates are classified by wave direction (southeast-southwest or northeast-northwest) and wave height class (0-0.9 ft, 1.0-1.9 ft, 2.0-3.9 ft and 4.0 ft and greater).

Table 2. Kahe Generating Station sand (>110 mm) throughput estimates.

Sample	    Daily Throughput	  Wave	Class	  Mean

Physical Conditions	Date	(m3/day)	(yd3/day)	Direction	Ht(ft)	(m3/day)	Condition

Lt-moderate trades	06/14/83	7.6	10.0	E-SW	0-1	7.1	S1
Waves 0-1 ft, South	07/21/83	6.3	 8.3
Swell, Turbidity low	06/18/92	7.3	 9.5	

Moderate tradewinds	08/24/83	12.6	16.5	E-SW	1-2	11.3	S2
Waves 1-3 ft., South	11/04/83	10.0	13.1 
Swell, Turbidity low
to moderate

Light-Moderate SE 	05/16/83	47.5	62.2	S-SW	2-4	19.1	S3
winds, Waves 2-3 ft	11/03/89	 5.3	 6.9
SW Swell,turbidity 	09/26/96	4.6	 6.0
moderate

Light-moderate trades	O9/14/83	 6.3	 8.3	NW-NE	0-1	2.1	S5
Waves 0-1 ft, NW swell	10/17/84	 5.2	 6.8
Turbidity low 	05/22/85	 0.3	 0.4
	08/23/91	 1.6	 2.1
	10/23/92	 0.6	 0.8	
	09/17/93	 9.2	12.1	Note: Sand dredged in 5-6 forebay 
				during sampling
	09/27/93	 0.6	 0.8
	11/30/93	 0.4	 0.5
	12/07/93	 0.3	 0.4
	05/16/94	 0.3	 0.4	Note: Unit 4 down for overhaul
	10/27/94	 0.8	 1.1
	05/05/95	 0.5	 0.7	Note: Unit 5 not running

Light-moderate SW wind	11/28/83	13.8	18.1	NW-NE	1-2 	 4.8	S6
Waves 1-2 ft, NW swell	04/27/84	 9.6	12.6
Water turbidity low to	08/28/85	 1.3	 1.7
moderate	O7/ll/86	 3.2	 4.2
	08/06/87	 2.8	 3.7
Light-Mod. E-NE winds	09/13/90	 1.8	 2.4	NW-NE	1-2
Waves 1-2 ft., NW swell	04/22/96	 1.3	 1.7			
Water turbidity clear

Moderate-heavy SW wind	09/17/87	 8.7	11.4	NW-NE	2-4	 6.1	S7
Waves 2-4 ft,NW Swell	11/27/95	 3.5	  4.8
Turbidity low-moderate

Light-Moderate SE 	12/07/88	14.4	  8.8	S-SW	2-4	10.6	W3
winds, Waves 2-3 ft	01/29/92	11.1	20.0	
SW Swell, turbidity	02/06/96	  6.3	  8.2
moderate

Table 2 (continues).

Sample	    Daily Throughput	Wave 	Class	  Mean
Physical Conditions	Date	(m3/day)	(yd3/day)	Direct.	Ht(ft)	(m3/day)	Condition

Moderate tradewinds	03/02/89	43.9	57.4	S-SW	 4+	17.2	W4
Waves 4-6 ft., S Swell	10/03/89	 4.6	 6.0
Turbidity low to very	12/10/91	 3.0	3.9
high

Light-moderate trades	03/02/91	 2.5	 3.3	NW-NE	0-1	 1.5	W5
Waves 0-1 ft, NE-NW 	03/27/95	 0.4	 0.5	
Turbidity low	
		
Light-moderate trades	03/13/84	10. 5	13.7	NW-NE	1-3	 3.7	W6
Waves 1-3 ft, NE-NW 	12/10/87	 2.0	 2.6
swell, Turbidity low	02/08/91	 7.2	 9.4
	12/03/91	 4.2	 5.5
	12/23/92	 1.2	 1.6
	02/27/94	 0.6	 0.8
	04/13/94	 0.8	 1.1
	12/26/96	 3.3	 4.3

Light-moderate trades	03/23/83	57.0	74.7	NW-NE	2-4	23.7	W7
Waves 2-4 ft, NW swell	03/17/85	10.5	13.7
Turbidity moderate 	12/09/86	19.3	29.2
	02/23/88	23.4	30.6

Moderate WNW winds	12/13/83	25.2	33.1	NW-NE	2-4
Waves 2-4 ft, NW swell	12/14/85	43.8	57.4
Turbidity moderate	12/10/85	31.5	4l.2
to high	12/16/85	22.7	29.7
	12/22/87	14.1	18.4

Moderate-heavy SW wind	09/17/87	 8.7	11.4	NW-NE	2-4
Waves 2-4 ft, NW swell	
Turbidity moderate 	

Moderate-heavy E winds	01/26/89	 3.6	 4.7	NW-NE	2-4
Waves 2-4 ft., NW swell
Turbidity low to moderate

Moderate W winds	02/01/95	9.4	12.3	Nw-NE	4-5	9.4	W8
Waves 4-5 ft, NW swell
Turbidity high

Table 3. Estimation of 1996 Kahe Generating Station sand
and silt entrainment based on frequency of wave occurrence.

(Based on wave data compiled by Marine Advisors (1964) and sand throughput estimates from Table 2)

DAILY	TOTAL THROUGHPUT
	WAVE	FREQUENCY OF	ESTIMATED	THROUGHPUT	FOR EACH WAVE
	HEIGHT	OCCURRENCE	No. DAYS OF	AVERAGE	CLASSIFICATION
	(ft.)	(%)	OCCURRENCE	(m3/day)	(m3)

TYPICAL SUMMER (April - November) WITH SOUTH-SOUTHWEST SWELLS

(Condition S1-S4)

0 - 1	29.6	66.0	7.1		468.6

1 - 2	32.1	72.0	11.3		813.6

2 - 4	37.6	84.0	4.6*		386.4

4 +	0.7	2.0	25.0 †		50.0

Total  =	1718.6

TYPICAL SUMMER (April - November) WITH NORTH-NORTHEAST SWELLS

(Condition S5-S8)

0 - 1	29.6	5.9	2.1		12.4

1 - 2	32.1	6.4	1.3*		8.3

2 - 4	37.6	7.5	6.1		45.7

4 +	0.7	0.2	6.3		3.2

Total  =	69.6

TYPICAL WINTER (December - March) WITH SOUTH-SOUTHEAST SWELLS

(Condition W1-W4)

0 - 1	41.8	4.6	7.1 ‡		32.7

1 - 2	24.0	2.6	11.3 ‡		29.4

2 - 4	27.0	3.0	6.3*		18.9

4 +	7.2	0.8	17.2		13.8

Total   =	113.7

TYPICAL WINTER (December - March) WITH NORTH-NORTHEAST SWELLS

(Condition W5-W8)

0 - 1	41.8	46.0	1.5*		69.0

1 - 2	24.0	26.0 	3.3		85.8

2 - 4	27.0 	29.7	6.3 † 		187.1

4 +	7.2 	7.9	9.4		74.3

Total   =	416.2

TOTAL SAND THROUGHPUT FOR 1996 (m3) =2318

* 1996 value,  † extrapolated value;  ‡ summer value.

Table 3 lists the extrapolated or averaged daily throughput values and calculated total throughput for various conditions using frequencies of occurrence for typical summer and winter surf conditions. The 1996 samples were taken under S3, S6, W3 and W6 throughput conditions and values for these measurements were substituted for the Table 2 means for these conditions for the calculations shown in Table 3. Because no data were available for conditions S4, S8, W1, or W2, estimates for these conditions were extrapolated from values for other wave heights in the same season and wave direction (S4, S8), or using values determined under the same wave conditions in the other season (W1, W2) as indicated in Table 3. The W7 value was extrapolated from the 1996 values for W6, the overall mean value for W5, and the 1995 value for W8.

Annual summer southern swells were estimated to produce 1719 m3 of suspended sediment throughput, while northern swells contributed only 70 m3. During winter, northern swells contributed an estimated 416 m3 of suspended sediment, and southern swells 114 m3. The total suspended sand and silt throughput at the Kahe Generating Station was estimated to be 2318 m3/year (3113 yd3/year) for 1996, the lowest annual value calculated in 18 years of monitoring.

Some of the heavier sediment settles out prior to passage through the power plant. This sediment, which accumulates as bed load in the Kahe Units 1-4 and Units 5-6 forebays (or stilling basins), has been occasionally pumped into the discharge line. This pumped sand may be temporarily retained in the Kahe transition basin but ultimately is discharged by the offshore outfall. Records of sand bed loads removed from the intake forebays since 1977 are summarized in Table 4. No sand was pumped from either forebay during 1996.

The 1981 to 1983 sand entrainment results (Tables 4 and 5) suggest that the increase in sediment entrainment subsequent to 1980 was due to the 33% increase in the circulating water flow rate created by the operation of Unit 6. Since 1984, however, the amount of sand accumulated and removed from Kahe forebays has drastically decreased. Total sediment entrainment has decreased almost continuously since 1986, with the 1996 value of 2318 m3 being the lowest that has been estimated in 18 years of monitoring, nearly half of the 4130 m3 estimated for 1995.

Sand entrainment values have generally decreased steadily since 1982, when waves generated by Hurricane Iwa removed large quantities of sand from the entire reef fronting the Kahe plant, and entrainment has decreased even more following Hurricane Iniki in 1992. These lower entrainment values are attributed to the low availability of sand surrounding the stations intake basin. Diver observations at nearby coral reef monitoring stations before and after Hurricane Iniki indicated that sand levels have remained fairly low and stable.

In order to assess the relative importance of this diversion of sand from normal circulation patterns, the quantities transported offshore by the Kahe outfall can be compared to normal sand movement to offshore waters and to the total reservoir of

Table 4. Removal of sand bed load
from Kahe cooling water system (1977-1996).

Duration			m3	yd3
  Date	Location	(Hr.)	GPM	% Sand	Removed	Removed

1977	1-4	29.0	2,670	4.74	 833	1,090
	5	60.2	2,670	4.74	1,730	2,263
				Total	2,563	3,353

1978	1-4	78.85	2,760	4.74	2,343	3,065	5	77.45	2,760	4.74 	2,359	3,085
				Total	4,702	6,150

1979	1-4	56.7	2,760	4.74	1,683	2,202
	5	27.0	2,760	4.74	 803	1,050
				Total	2,486	3,252

1980	1-4	98.7	2,760	4.74	3,597	4,705
	5	52.3	2,760	4.74	1,414	1,850
				Total	5,011	6,555

1981	1-4	114.0	2,760	4.74	 2,462	 3,220
	5-6	350.0	2,760	4.74	10,493	13,725
				Total	12,955	16,945

1982	1-4	140.91	2,760	4.74	4,187	5,477
	5-6	161.67	2,760	4.74	4,805	6,285
				Total	8,992	11,762

1983	1-41	50.08	2,760	4.74	4,459	 5,832
	5-6	99.00	2,760	4.74	2,942	3,848
				Total	7,401	9,680

1984	1-4	64.80	2,760	4.74	1,940	2,538
	5-6	35.00	2,760	4.74	1,038	1,358
				Total	2,978	3,896

1985	1-4	49.00	2,760	4.74	1,456	1,904

1986	1-4	30.00	2,760	4.74	891	1,166
	1-4		(Sand removed by lift bags)		153	200
	5-6	14.00	2,760	4.74	416	544
				Total	1,460	1,910

1987	1-6	(	No sand removed during 1987)

1988	5-6	11/30-12/02/88		Contracted out	153	200

1989-91	1-6	(	No sand removed 1989 to 1991)

Table 4 (continued).

Duration			m3	yd3
  Date	Location	(Hr.)	GPM	% Sand	Removed	Removed

1992	1-4	7/10-8/14/92		Contracted out	118	154
	5-6	6/15-6/19/92		Contracted out	230	301
				Total	348	455

1993	1-4	8/23-30/93 		(Removed to beach)	505	660
	5-6		(	No sand removed in 1993)

1994	1-4	11/14-11/18/94		(Removed to shoreline 	150	196
	5-6	11/14-11/18/94		adjacent to Intake Basin)	150	196
				Total	300	392
1995	1-4	3/2-3/3/95		Contracted out	10	13

1996	1-6	(	No sand removed during 1996)

Total for 1977 through 1996:	51,320 m3	67,126 yd3
	Annual average	2,851 m3	3,729 yd3

sand available within the littoral system. These parameters were estimated by Marine Advisers (1964) shortly after construction of the first Kahe Station unit. This study determined an average volume of 994,000 m3 (1,300,000 yd3) of sand to be contained in a sublittoral cell between sea level and the 6O-foot offshore contour. Another 53,520 m3 (70,000 yd3) were estimated to make up the beach systems. Although most of the sand recirculates within the system, net movement is to the south with a volume of 1,530 to 2,290 m3 per year escaping to deep water. This estimate of annual natural net loss of sand from the undisturbed Kahe system was later revised upwards to 3,520 m3 (4,600 yd3) (URS Research Corp., 1973; Stearns Roger, Inc., 1973).

The present study indicates that only 2318 m3 of suspended sand and fine sediments were entrained by the Kahe Station and released by the offshore outfall during 1996. This is about 13 to 14% of the sand transport estimated for 1981 and 1982 when sand throughput volumes peaked at 16,073 m3 and 17,800 m3 respectively. The 1996 suspended throughput value is about 70% above the volume estimated for sand transport offshore under natural conditions. In terms of the approximate 1,000,000 m3 of sand estimated to occur within the Kahe littoral cell, the volume of sand entrained and transported by Station cooling water is a minor fraction. Assuming complete loss of this sand to the littoral cell and no production or replenishment within the system, more than 430 years would be required to remove all sand at the 1996 rate of loss. However, sand has been estimated to be produced by benthic organisms within the Kahe littoral cell at rates up to 3,000 m3/year. The portion of sand released by the outfall that can recycle back to the system is unknown. However, these fine-grained fractions may comprise the major portion of total sand released by the outfall.

Table 5.  Total volumes of sand (m3) entrained by Kahe Power Station estimated by dredging removal and suspended throughput, 1976-1996.

Intake	Pump-	Pump-	Suspended	Total Entrainment
	Basin	Dredging	Dredging	Sand	(Suspended
Year	Dredging	Forebay 1-4	Forebay 5-6	Throughput	+ pumped)

1976	2,408	 **	 **	 **	 **
1977	6,079	833	1,730	1,753	 4,316
1978	  0	2,343	2,359*	 **	 **
1979	  0	1,684	803	 **	 **
1980	6,117	3,597	1,414*	3,963	 8,974
1981	1,881	2,462	10,493	3,118	16,073
1982	  0	4,187	4,805	8,808	17,800
1983	  0	4,459	2,942	6,341	13,742
1984	  0	1,940	1,038	7,610	10,988
1985	  0	1,456	0	7,291	 8,747
1986	3,058	1,044	416	8,358	 9,818
1987	  0	  0	  0	8,006	 8,006
1988	  0	  0	153	6,473	6,626
1989	  0	  0	  0	4,914	4,914
1990	  0	  0	  0	5,458	5,458
1991	  0	  0	  0	5,134	5,134
1992	0	118	230	4,999	5,347
1993	1,995 † 	  0	0	5,007	5,007
1994	300 ††	0	0	4,885	4,885
1995		10		4,120	4,130
1996	0	0	0	2,318	2,318

*  Only unit No. 5 in operation;	†  includes 505 m3 dredged from 1-4 forebay
** No data available;	†† Forebay sand transferred adjacent to intake

It is likely that any substantial impact of sand and fine sediment transported by the Kahe outfall on the total littoral sand system would be expressed only over a very long-term period. More significantly the amount of sand entrained and discharged by the Kahe Station has steadily decreased with time since 1982 when sand was massively redistributed by Hurricane Iwa (Table 5). Long term projection of impacts of Kahe Station entrainment of sand on the total littoral sand system is difficult to make with certainty, but it appears that the offshore movement of sand by the station’s discharge has moderated and is now a negligible factor in the dynamics of offshore sand movement.

4.  REEF CORAL STUDIES

4.1  Introduction

The current NPDES permit for the Kahe Generating Station requires annual monitoring of coral coverage for replicate quadrats at four stations (Figures 4 and 5). The control station (CTL; see Figure 4) is located on a reef off Nanakuli Point, 2.1 km (1.3 mi.) northwest of the Kahe offshore outfall. Three stations are in the vicinity of the outfall (Figure 5). Station 6B is the closest, located 60 m (197 ft) southeast of the outfall. Station 8A is 300 m (984 ft) northeast of the outfall and Station 2B is 520 m (1706 ft) south of the outfall. Previous studies have indicated both the direction of movement of the surface thermal plume (Marine Advisors, 1964) and the net transport of the water column (Coles et al., 1982) to be toward the south. Therefore Station 6B is in a location most under the influence of the outfall discharge, followed by Station 2B. This assessment has been verified by long-term measurement of bottom water temperature on the reef (Coles et al., 1982). The control station is well beyond any influence from the Kahe outfall.

4.2  Replicate Measurement Methods

Ten permanent quadrats, each 0.66 m2 in area, were established in 1980 at each of the four stations. Each quadrat was marked with tagged metal stakes driven into the reef and cemented in place with underwater epoxy putty. These stakes have been used as reference points for photographing the bottom with a Nikonos 35mm underwater camera with a 28mm wide-angle lens mounted on a support frame. Photographs have been taken yearly since 1980, and missing stakes have been replaced as needed to maintain the quadrats as close as possible to their original location. In cases where the original quadrat was lost or could not be accurately located, new quadrats have been established to provide each station with a full complement of ten replicate quadrats. The history of quadrat occurrence at the stations is summarized in Table 6. Of the original 40 quadrats established in 1980, 33 were still present in 1996 and used in long-term analysis of changes in benthic coverage on the reef.

4.3  Data Analysis Methods

Photographic transparancies of coral coverage at each quadrat were projected onto a white surface on which a rectangle had been drawn which corresponded to the camera stand's 0.66 m2 quadrat. The projected image of the camera stand quadrat was adjusted in size and position to assure a consistent projected area and view of a station's quadrat area between sampling years. Each photo-quadrat slide represents a permanent record of the condition of the coral reef at the time the photograph was taken.

Figure 4.  Kahe and Nanakuli areas showing location of
coral community control station (CTL).

Figure 5.  Kahe vicinity showing coral reef stations (2B, 6B, and 8A).

Table 6. Quadrat photograph record for Kahe coral monitoring stations.

YEAR

Sta. Quad.	1980	1981	1982	1983	1984	1985	1986	1987	1988	1989	1990	1991	1992	1993	1994	1995	1996

2B	1	X	X	X	X	X	X	NEW	X	X	X	X	X	X	X	X	X	X
2B	2	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
2B	3	X	X	X	X	X	X	MIS	NEW	X	X	X	X	X	X	X	X	X
2B	4	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
2B	5	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
2B	6	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
2B	7	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
2B	8	X	X	X	X	X	X	MIS	X	X	X	X	X	X 	X	X	X	X
2B	9	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
2B	10	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X

6B	1	X	X	X	X	X	X	X	MIS	X	X	X	X	X 	X	X	X	X
6B	2	X	X	X	X	X	X	X	X	MIS	MIS	MIS	NEW	X 	X	X	X	X
6B	3	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
6B	4	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
6B	5	X	X	X	X	X	X	MIS	NEW	X	X	X	NEW	X 	X	X	X	X
6B	6	X	X	X	X	X	X	MIS	X	X	X	X	X	X 	X	X	X	X
6B	7	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
6B	8	X	X	X	X	MIS	MIS	MIS	NEW	X	X	X	X	X 	X	X	X	X
6B	9	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
6B	10	X	X	X	X	X	X	MIS	X	X	X	X	X	X 	X	X	X	X
	
8A	1	X	X	X	X	X	MIS	X	X	X	X	X	X	X	X	X	X	X
8A	2	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
8A	3	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
8A	4	X	MIS	MIS	MIS	MIS	MIS	NEW	X	X	X	X	X	X 	X	X	X	X
8A	5	X	X	X	X	X	X	X	X	X	X	X	X	X 	X	X	X	X
8A	6	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
8A	7	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
8A	8	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
8A	9	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
8A	10	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
	
CTL	1	MIS	X	X	X	POOR	X	X	X	X	X	MIS	X	X	X	X	X	X
CTL	2	X	X	X	MIS	X	X	X	X	X	X	X	X	X	X	X	X	X
CTL	3	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
CTL	4	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
CTL	5	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
CTL	6	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
CTL	7	X	X	MIS	MIS	MIS	MIS	NEW	X	X	X	X	X	X	X	X	X	X
CTL	8	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
CTL	9	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X
CTL	10	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X	X

X = present, MIS = not found, NEW = new quadrat established. POOR = photo not usable.

Analysis of the slide images can be undertaken at various levels of precision which require increasing amounts of time and effort to achieve increasing degrees of resolution of the composition of benthic coverage. Analysis described in the Kahe Monitoring Report on activities for 1991 (Environmental Department, 1992) indicated that use of 500 evenly spaced intercept points within the total photo-quadrat area provided sufficient sampling of the photo-quadrat images to accurately determine percent coverage and encounter the rarer species present. Therefore, this level of analysis was used to determine coral coverage and species composition.

It was determined in 1994 that the original (hand-drawn) point-quadrat screen used in this method since 1991 contained inconsistencies in size and spacing of points. Therefore, a new screen was developed in 1994 using computer-generated points with equal size and spacing (a portion of which is included at normal size in Appendix C). The number of points on this screen that underlay the projected image of each coral species and substratum type in each photo quadrat were tabulated and totaled. Percent coverage for each species and total coral cover were calculated by dividing the numbers of points intercepted for each coral species and substratum type by the total numbers of points occurring in each of the sampling grids.

Changes in coverage between 1995 and 1996 and between 1986 and 1996 were assessed for each species and total coral coverage for all quadrats found in both sampling years. Changes were determined as the net differences in a coral's percent coverage between sampling years. Changes could be positive or negative, depending on whether growth or mortality dominated during the period. The one-year change from 1995 to 1996 was compared to the ten-year change from 1986 to 1996 by dividing results for the latter by 10; therefore, the latter results represent an averaged annual value for the ten-year period.

Sampling in the field at each station consisted of observing 10 0.66-m2 quadrats for a total sampling area of 6.6 m2 at each station when all quadrats could be found. The adequacy of this sampling effort was examined in the 1991 monitoring report (Environmental Department, 1992) by plotting the cumulative numbers of species and cumulative mean total coverages against increasing sampling area for two stations in three sampling years. Based on these results, 10 quadrats were considered adequate for sampling the coral community at each station, and the 500-point method was used to analyze changes in benthic coverage within each quadrat.

4.4  Coral Study Results

Coverages of hermatypic corals, a zoanthid, and sand on the quadrats are given in the tables in Appendix C.  Coral cover was highest and most diverse at Station 2B, where quadrat cover in 1996 ranged up to 73% and seven species occurred.  Lowest coral abundance occurred at Station 8A, where quadrat cover ranged down to 3.1% in 1996.  The coral community was dominated at all stations by massive or encrusting Porites lobata, followed in order of abundance by Pocillopora meandrina, Montipora patula, Montipora verrucosa, Leptastrea purpurea, Porites compressa, and the zoanthid Palythoa tuberculosa.  Sand was intermittently and infrequently found on the quadrats, with a maximum cover value of 1.9% at any quadrat at any time.

Figure 6.  Mean total coral coverages at Kahe stations for 1986, 1995, and 1996.

Mean coverage for each station in the three sampling years is shown in Figure 6 for total coral coverage and in Figure 7 for Porites lobata, Pocillopora meandrina, and all remaining species combined.  Figure 6 suggests that mean total coral cover increased slightly at Station 2B from 1986 to 1995 and conrtinued to increase (from 25% to 28%) between 1995 and 1996.  Mean total coverage at Stations 6B increased from 12.5% to 16.4% from 1986 to 1995 and again increased to 18.0% in 1996. In contrast to these two stations where coverage showed a continuous increase in the 10-year period, total coverage at the control station, increased only slightly between 1986 and 1995, then decreased slightly (from 8.7% to 7.4%) between 1995 and 1996.  Mean total coral cover at Station 8A decreased substantially from about 15% to 4.6% in the nine years between 1986 and 1995 and then increased slightly to 6.9% between 1995 to 1996.

Patterns of mean coverage shown by the two dominant species and the combined remaining species (Figure 7) demonstrates that most of the coverage and patterns of change with time shown by the total coral community were accounted for by Porites lobata. Mean cover of P. lobata ranged up to 18% in 1995 and to 19% in 1996, accounting for up to 90% of mean total coverage. The pattern of change with time shown by P. lobata was in all respects similar to that of mean total coverage, indicating that this species was the major component of the change in mean total cover.

Pocillopora meandrina, the second most abundant species, showed a maximum cover of only about 8%, and its patterns of change were not similar to P. lobata or total cover. Pocillopora meandrina mean coverage was quite stable at three of the stations from 1986 to 1996 showing coverages of around 5% at Stations 2B and 6B and about 1% at CTL. At Station 8A, P. meandrina coverage decreased substantially from 7.7% in 1986 to less than as 1% by 1995, then increased to 2.1% by 1996.

The remaining five combined species were negligible in their effect on total coral coverage, having a mean combined cover of less than 4% at any station during any year.  Coverage by these combined species declined from 1986 to 1996 at 8A and CTL and increased at 2B and 6B.

Net changes in coral coverage by species that occurred within each quadrat from 1995 to 1996 are shown in Table 7, while mean annual changes for the decade from 1986 to 1996 are given in Table 8. Net changes in total cover within a given quadrat between 1995 and 1996 ranged from an increase of 13.0% at Station 2B to a decrease of 9.7% at the control station (Table 7). Average annual changes for each species between 1986 and 1996 ranged from an increase of 5.3% to a decrease of 2.6%, both at Station 2B (Table 8).

Statistical analyses were made to determine if the changes suggested in Figures 6 and 7 and Tables 7 and 8 were significant. The pairwise differences for each species and for

Figure 7.  Mean coverages of Porites lobata, Pocillopora meandrina and combined other coral species at Kahe stations for 1986, 1995, and 1996.

Table 7.  Percent changes in cover for 1995 vs. 1996.

| Station | Quadrat | Porites lobata | Porites compressa | Pocillopora meandrina | Leptastrea purpurea | Montipora verrucosa | Montipora patula | Palythoa tuberculosa | Sand | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2B | 1 | 3.9 | 0.0 | -0.4 | 1.9 | 0.2 | 2.1 | 0.4 | 0.0 | 8.0 |
| 2B | 2 | -4.1 | 0.0 | 2.7 | 0.0 | 0.0 | 1.0 | 0.0 | 0.0 | -0.4 |
| 2B | 3 | 0.6 | 0.0 | 1.4 | 0.0 | -0.8 | 2.1 | 0.0 | 0.0 | 3.3 |
| 2B | 4 | 1.0 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.6 |
| 2B | 5 | -4.1 | 0.0 | -1.2 | 0.0 | -0.2 | -2.7 | 0.0 | 1.4 | -6.8 |
| 2B | 6 | 1.6 | 3.7 | 2.5 | 0.0 | 0.0 | 0.4 | 0.0 | 0.0 | 8.2 |
| 2B | 7 | 1.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.8 |
| 2B | 8 | 1.6 | 0.0 | -0.6 | 0.0 | 0.0 | 0.8 | 0.0 | 0.0 | 1.9 |
| 2B | 9 | 10.5 | 0.0 | 1.0 | 0.0 | 1.4 | 0.0 | 0.0 | 0.0 | 13.0 |
| 2B | 10 | 0.4 | 0.0 | 1.9 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 | 2.1 |
| 6B | 1 | -2.1 | 0.0 | 0.4 | -0.2 | 0.6 | 0.8 | 0.0 | 0.0 | -0.4 |
| 6B | 2 | 2.9 | 0.0 | -2.9 | 0.0 | 0.4 | -0.4 | 0.0 | -0.6 | -0.6 |
| 6B | 3 | 1.2 | 0.0 | 2.1 | 0.0 | 1.9 | 0.0 | 0.0 | 0.0 | 5.2 |
| 6B | 4 | 0.4 | 0.0 | 0.6 | 0.0 | 1.2 | 0.0 | -0.6 | 0.4 | 2.1 |
| 6B | 5 | 1.0 | 0.0 | 0.8 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 1.6 |
| 6B | 6 | 1.4 | 0.0 | 3.9 | -0.2 | 0.0 | -0.2 | 0.0 | 0.0 | 4.9 |
| 6B | 7 | 1.4 | 0.0 | -4.3 | 0.0 | 0.0 | 0.6 | 0.0 | 0.0 | -2.3 |
| 6B | 8 | 1.0 | 0.0 | 1.4 | -0.6 | 0.0 | 3.7 | 0.0 | -1.0 | 4.5 |
| 6B | 9 | 2.3 | 0.0 | 1.2 | 0.0 | 0.0 | 0.0 | 0.0 | -0.6 | 2.9 |
| 6B | 10 | -2.7 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -2.1 |
| 8A | 1 | 1.4 | 0.0 | 0.2 | 0.0 | 0.2 | 0.0 | 0.0 | -2.3 | -0.4 |
| 8A | 2 | 0.6 | 0.0 | 0.4 | 0.0 | 0.0 | 0.2 | 0.0 | 0.8 | 2.0 |
| 8A | 3 | 2.0 | 0.0 | 1.0 | 0.0 | 0.0 | 0.2 | 0.0 | 0.4 | 3.6 |
| 8A | 4 | 1.4 | 0.0 | 0.2 | -0.2 | 0.0 | 0.0 | 0.0 | -1.9 | -0.4 |
| 8A | 5 | -1.7 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -1.1 |
| 8A | 6 | 1.6 | 0.0 | 0.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 3.0 |
| 8A | 7 | 2.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 2.4 |
| 8A | 8 | 3.5 | 0.0 | 1.8 | 0.0 | 0.0 | 0.0 | 0.0 | -0.4 | 4.9 |
| 8A | 9 | -0.4 | 0.0 | 0.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.4 |
| 8A | 10 | 0.6 | 0.0 | 8.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 8.4 |
| CTL | 1 | -4.6 | 0.0 | -5.8 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | -9.7 |
| CTL | 2 | -0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.4 |
| CTL | 3 | 1.6 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 2.2 |
| CTL | 4 | 1.0 | 0.0 | -5.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -4.0 |
| CTL | 5 | 0.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.7 |
| CTL | 6 | -1.7 | 0.0 | -0.6 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | -2.5 |
| CTL | 7 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 |
| CTL | 8 | -0.2 | 0.0 | -3.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -3.3 |
| CTL | 9 | 0.1 | 0.0 | 0.6 | 0.0 | 0.6 | 0.8 | 0.0 | 0.0 | 2.2 |
| CTL | 10 | 2.6 | 0.0 | -0.2 | 0.0 | -1.0 | 0.0 | 0.0 | 0.0 | 1.4 |

Table 8. Average annual percent changes in cover for 1986 vs 1996.

| Station | Quadrat | Porites lobata |  | Porites compressa |  | Pocillopora meandrina |  | Leptastrea purpurea |  | Montipora verrucosa |  | Montipora patula |  | Palythoa tuberculosa |  | Sand |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2B | 1 | -0.2 | 0.0 |  | 0.2 |  | 1.0 |  | 0.0 |  | 0.3 |  | 0.0 |  | 0.0 |  | 1.3 |  |  |  |
| 2B | 2 | -0.8 |  | 0.0 |  | -0.2 |  | 0.0 |  | 0.0 |  | 0.2 |  | -0.1 |  | 0.0 |  | -0.9 |  |  |
| 2B | 3 | Replaced in 1987 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2B | 4 | -0.3 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.4 |  |  |
| 2B | 5 | -0.2 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.1 |  | -0.2 |  |  |
| 2B | 6 | 3.7 |  | 1.1 |  | 0.5 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 5.3 |  |  |
| 2B | 7 | 0.2 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.1 |  |  |
| 2B | 8 | 0.6 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.3 |  | 0.0 |  | 0.0 |  | 0.8 |  |  |
| 2B | 9 | 1.1 |  | 0.0 |  | -0.5 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.7 |  |  |
| 2B | 10 | -1.9 |  | 0.0 |  | -0.5 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.1 |  | -0.1 |  | -2.6 |  |  |
| 6B | 1 | 0.1 |  | 0.0 |  | -0.2 |  | 0.0 |  | 0.0 |  | 0.2 |  | -0.1 |  | 0.0 |  | 0.0 |  |  |
| 6B | 2 | Replaced in 1991 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 3 | 0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.2 |  | 0.2 |  | -0.3 |  | 0.0 |  | 0.2 |  |  |
| 6B | 4 | 0.4 |  | 0.0 |  | -0.5 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  |  |
| 6B | 5 | Replaced in 1991 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 6 | Missing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 7 | 0.8 |  | 0.0 |  | -0.4 |  | 0.0 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | 0.6 |  |  |
| 6B | 8 | Replaced in 1987 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 9 | 0.2 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.4 |  |  |
| 6B | 10 | 0.6 |  | 0.0 |  | 1.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 1.6 |  |  |
| 8A | 1 | -0.5 |  | 0.0 |  | -0.6 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.1 |  | -1.2 |  |  |
| 8A | 2 | -0.3 |  | 0.0 |  | -0.7 |  | 0.0 |  | 0.0 |  | -0.0 |  | 0.0 |  | 0.0 |  | -1.0 |  |  |
| 8A | 3 | -0.5 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.6 |  |  |
| 8A | 4 | Replaced in 1986 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A | 5 | -0.3 |  | 0.0 |  | -0.8 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.0 |  | 0.0 |  | -1.2 |  |  |
| 8A | 6 | -0.5 |  | 0.0 |  | -0.4 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.4 |  | -1.3 |  |  |
| 8A | 7 | -0.9 |  | 0.0 |  | -1.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.1 |  | -2.0 |  |  |
| 8A | 8 | -0.9 |  | 0.0 |  | -0.8 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -1.7 |  |  |
| 8A | 9 | -0.5 |  | 0.0 |  | -0.4 |  | 0.0 |  | -0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.8 |  |  |
| 8A | 10 | -0.6 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.1 |  | -0.8 |  |  |
| CTL | 1 | 0.0 |  | 0.0 |  | -0.0 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.1 |  |  |
| CTL | 2 | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  |  |
| CTL | 3 | -0.1 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.0 |  |  |
| CTL | 4 | -0.2 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.3 |  |  |
| CTL | 5 | 0.2 |  | 0.0 |  | -0.1 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  |  |
| CTL | 6 | -0.2 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.0 |  | -0.2 |  |  |
| CTL | 7 | Replaced in 1987 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CTL | 8 | -0.1 |  | 0.0 |  | -0.4 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.5 |  |  |
| CTL | 9 | 0.8 |  | 0.0 |  | 0.1 |  | 0.0 |  | 0.1 |  | 0.1 |  | 0.0 |  | -0.3 |  | 0.8 |  |  |
| CTL | 10 | -0.2 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | 0.0 |  | -0.1 |  | -0.3 |  |  |

Table 9.  Statistical parameters: 1995 vs. 1996 comparisons.

| Station | Species | N | Mean | SE Mean | t | p>t |
| --- | --- | --- | --- | --- | --- | --- |
| 2B | P. lobata | 10 | 1.26 | 1.30 | 0.97 | 0.36 |
|  | P. compressa | 1 | - | - | - | - |
|  | P. meandrina | 10 | 0.78 | 0.42 | 1.85 | 0.10 |
|  | L. purpurea | 1 | - | - | - | - |
|  | M. verrucosa | 4 | 0.15 | 0.44 | 0.35 | 0.73 |
|  | M. patula | 6 | 0.62 | 0.71 | 0.87 | 0.40 |
|  | P. tuberculosa | 2 | 0.10 | 0.24 | 0.43 | 0.68 |
|  | Total coral | 10 | 3.18 | 1.73 | 1.84 | 0.10 |
|  | Sand | 2 | 0.62 | 0.74 | 0.84 | 0.42 |
| 6B | P. lobata | 10 | 0.70 | 0.56 | 1.26 | 0.12 |
|  | P. compressa | 0 | - | - | - | - |
|  | P. meandrina | 10 | 0.39 | 0.75 | 0.52 | 0.61 |
|  | L. purpurea | 3 | -0.38 | 0.21 | -1.63 | 0.14 |
|  | M. verrucosa | 4 | 1.03 | 0.52 | 2.00 | 0.08 |
|  | M. patula | 7 | 0.62 | 0.55 | 1.13 | 0.29 |
|  | P. tuberculosa | 1 | - | - | - | - |
|  | Total coral | 10 | 1.59 | 0.89 | 1.78 | 0.11 |
|  | Sand | 4 | -0.46 | 0.34 | -1.37 | 0.20 |
| 8A | P. lobata | 10 | 1.11 | 0.45 | 2.45 | 0.04 |
|  | P. compressa | 0 | - | - | - | - |
|  | P. meandrina | 5 | 1.41 | 0.75 | 1.88 | 0.09 |
|  | L. purpurea | 1 | - | - | - | - |
|  | M. verrucosa | 1 | - | - | - | - |
|  | M. patula | 1 | - | - | - | - |
|  | P. tuberculosa | 0 | - | - | - | - |
|  | Total coral | 10 | 2.39 | 0.83 | 2.88 | 0.02 |
|  | Sand | 8 | 0.34 | 0.42 | -0.79 | 0.45 |
| CTL | P. lobata | 10 | -0.02 | 0.63 | -0.03 | 0.98 |
|  | P. compressa | 0 | - | - | - | - |
|  | P. meandrina | 7 | -1.92 | 1.07 | -1.79 | 0.11 |
|  | L. purpurea | 0 | - | - | - | - |
|  | M. verrucosa | 3 | 0.06 | 0.47 | 0.14 | 0.89 |
|  | M. patula | 2 | 0.30 | 0.43 | 0.70 | 0.50 |
|  | P. tuberculosa | 0 | - | - | - | - |
|  | Total coral | 10 | -1.29 | 1.17 | -1.10 | 0.30 |
|  | Sand | 0 | - | - | - | - |

Table 10.  Statistical parameters: 1986 vs. 1996 comparisons.

| Station | Species | N | Mean | SE Mean | t | p>t |
| --- | --- | --- | --- | --- | --- | --- |
| 2B | P. lobata | 9 | 5.05 | 4.97 | 1.02 | 0.34 |
|  | P. compressa | 1 | - | - | - | - |
|  | P. meandrina | 9 | -1.08 | 1.09 | -0.99 | 0.35 |
|  | L. purpurea | 1 | - | - | - | - |
|  | M. verrucosa | 2 | 0.62 | 0.41 | 1.50 | 0.37 |
|  | M. patula | 4 | 1.91 | 0.53 | 3.57 | 0.04 |
|  | P. tuberculosa | 3 | -0.54 | 0.08 | -7.04 | 0.02 |
|  | Total coral | 9 | 4.58 | 7.10 | 0.65 | 0.54 |
|  | Sand | 3 | 0.07 | 0.68 | 0.11 | 0.92 |
| 6B | P. lobata | 5 | 3.34 | 1.36 | 2.46 | 0.07 |
|  | P. compressa | 0 | - | - | - | - |
|  | P. meandrina | 5 | -1.94 | 1.2 | -1.62 | 0.18 |
|  | L. purpurea | 1 | - | - | - | - |
|  | M. verrucosa | 4 | 1.03 | 0.37 | 2.81 | 0.07 |
|  | M. patula | 4 | 1.80 | 0.47 | 3.83 | 0.03 |
|  | P. tuberculosa | 2 | -1.96 | 1.34 | -1.46 | 0.40 |
|  | Total coral | 5 | 2.40 | 1.02 | 2.36 | 0.08 |
|  | Sand | 2 | 1.13 | 0.72 | 1.57 | 0.36 |
| 8A | P. lobata | 9 | -5.57 | 0.74 | -7.49 | <0.001 |
|  | P. compressa | 0 |  |  |  |  |
|  | P. meandrina | 9 | -5.40 | 1.03 | -5.23 | <0.001 |
|  | L. purpurea | 0 |  |  |  |  |
|  | M. verrucosa | 1 | - | - | - | - |
|  | M. patula | 2 | 0.20 | 0.21 | 0.99 | 0.50 |
|  | P. tuberculosa | 1 | - | - | - | - |
|  | Total coral | 9 | -11.7 | 1.53 | -7.65 | <0.001 |
|  | Sand | 6 | -1.07 | 0.62 | -1.72 | 0.15 |
| CTL | P. lobata | 10 | -0.03 | 1.00 | -0.03 | 0.98 |
|  | P. compressa | 0 | - | - | - | - |
|  | P. meandrina | 7 | -0.57 | 0.59 | -0.98 | 0.37 |
|  | L. purpurea | 0 | - | - | - | - |
|  | M. verrucosa | 2 | 1.02 | 0.41 | 2.50 | 0.24 |
|  | M. patula | 4 | 0.72 | 0.10 | 7.00 | 0.09 |
|  | P. tuberculosa | 0 | - | - | - | - |
|  | Total coral | 10 | -0.38 | 1.24 | -0.31 | 0.77 |
|  | Sand | 2 | -2.35 | 0.95 | -2.47 | 0.24 |

total coverage in each quadrat were used in Students paired t-tests. Results are shown in Table 9 for the 1995-96 comparisons and in Table 10 for the 1986-96 comparisons. Although most species showed increases from 1995 to 1996, only two significant increases in coverage of individual species occurred: both Porites lobata (p<0.04) and total coral (p<0.02) increased significantly at Station 8A.

Similar analyses for 10-year changes in coral cover between 1986 and 1996 (Table 10) showed significant decreases at Station 8A for Porites lobata (p<0.001), Pocillopora meandrina. (p<0.001) and total coral (p<0.001), and slight but significant increases in Montipora patula at Stations 2B and 6B.  The soft coral Palythoa tuberculosa decreased significantly at Station 2B but was a trivial component of the total benthic coverage at any time.

4.5  Discussion of Results

Under the HECO Kahe Monitoring Program measurements of coral coverage in the Kahe area have been undertaken annually since 1980 and the data now comprises one of the longest continuous records of coral growth and mortality available worldwide. The most recent effort measured conditions four years following Hurricane Iniki in September 1992 and nearly 14 years after Hurricane Iwa in November 1982. Both hurricanes passed to the west of O`ahu, and maximum impact from storm waves of these hurricanes on O`ahu occurred on the Wai`anae coast, which includes the Kahe area. Generated wave heights were estimated at 5 to 7 m in 1982 (Coles and Fukuda, 1984) and 7 to 8 m in 1992 (Brock, 1992).

Short-term, qualitative estimates of hurricane damage to the Kahe reef and corals were made by Brock (1992), who compared conditions in November 1992 with a similar survey conducted in 1982 by the HECO Environmental Department (Coles and Fukuda, 1983). Despite the potential for coral destruction that could have resulted from Hurricane Iniki, Brock (1992) found surprisingly little apparent damage to corals or reef structure. Coral damage was limited to breakage of the branching coral Pocillopora meandrina, primarily in the area of Station 8A (Figure 5) and to the south and seaward. Qualitative observations made in August 1993 concurred with Brock's (1992) initial assessment of minimal damage following Hurricane Iniki; little or no indication of coral breakage or damage was apparent at any station (Environmental Department 1994). Even in the offshore area south of Kahe Point, where damage from Hurricane Iniki was concentrated (Brock, 1992), it was difficult to discern differences in 1993 from previous years (post-Hurricane Iwa) within the perspective of qualitative observations.

However, quantitative photo-quadrat measurements of changes in coral coverage from 1986 to 1996 confirm findings from previous year's monitoring, that significant reductions in total coral coverage have occurred since Hurricane Iniki in 1992.

Significant decreases in mean coverage of Porites lobata and of total coral occurred from 1992 to 1993 (Environmental Department, 1994) and again from 1993 to 1994 on the 10 quadrats at Station 2B. Significant decreases occurred in mean coverage of Pocillopora meandrina and total coral in 1992-93 at Station 6B (Environmental Department, 1994) and of Montipora patula in 1993-94 at Station 6B.

The present one-year changes (1995 to 1996) suggest that these trends of decreasing coverage at Stations 2B and 6B have moderated. Although total coral and all but one species showed negative changes in 1994 at Station 2B, none of these decreases were significant. The current 1995-96 monitoring results showed all species and total coral at 2B to have had mean net increases in cover, although these mean increases were not significant. At Station 6B, 60 m southeast of the outfall, two years of significant declines in coral coverage from 1992 to 1994 were followed by significant increases in Porites lobata and Montipora patula in 1994-95.

Station 8A is in the area that was most impacted by Hurricane Iniki (Brock, 1992) and coral coverage at this station has shown substantial decline since 1992. Pocillopora meandrina and total coral showed significant decreases in 1992-93 (Environmental Department, 1994), and Porites lobata,  Pocillopora meandrina and total coral coverage all showed significant decreases again in 1994-95 (Environmental Department, 1996). Changes in the intervening 1993-94 period were also negative, although not significant (Environmental Department, 1995). The present results are the first since Hurricane Iniki that have indicated a coral coverage increase at Station 8A in a sampling year. Total coral and Porites lobata increased significantly in 1995-96, and mean coverage of Pocillopora meandrina increased substantially although not significantly (Table 9).

Over the long term from 1986 to 1996, except for significant small increases in Montipora patula at Stations 2B and 6B and a significant small decrease in Palythoa tuberculosa at Station 2B, the only substantial changes were at Station 8A.  Here, even with the significant increases that occurred in 1995-96, the ten year decreases in Porites lobata, Pocillopora meandrina, and total coral were highly significant (p<0.001).  These negative changes reflect the continuous decline that had occurred at this station between 1992 to 1995, and the present results suggest the first regrowth of coral that has occurred since Hurricane Iniki struck the Hawaiian Islands in 1992.

These results support conclusions of previous monitoring reports, which have indicated no detrimental effects of the Kahe offshore outfall on the reef coral community in the area. At Station 6B, which is the closest station to the outfall in the direction of the surface thermal plume, 1995-96 changes in coverage of total coral and of all species except Leptastrea purpurea were positive. This location near the Kahe offshore outfall continues to support one of the most flourishing coral communities in the Kahe area and on the island of O`ahu, and is a favorite area of recreational SCUBA divers. The other station in the direction of the outfall plume, Station 2B, showed no decreases in total coral or any component species. Coverage remained substantially higher at Station 2B than at any other station. Cessation of the long term decline in coral coverage at Station 8A supports previous conclusions (Environmental Department, 1994, 1995, 1996) that this decline was due to hurricane impacts rather than effects from the Kahe Station discharge. Continued monitoring will determine if this recovery trend in coral coverage continues.

5. LITERATURE CITED

B-K Dynamics, Inc. 1971. Marine environmental impact analysis, Kahe Power Plant. Report to Hawaiian Electric co., Inc. 350 p.

Brock, R. E. 1992. Status of coral communities fronting the Kahe Generating Facility, Kahe Point, Oahu following Hurricane Iniki. Report to the Environmental Department, Hawaiian, Electric Co. Inc. 12 p.

Coles, S. L. 1979. Annual report. Kahe Generating Station. NPDES monitoring program, vol. 1, text. Honolulu. 263 p.

______ 1980. Annual report. Kahe Generating Station. NPDES monitoring program, vol. 1, text. Honolulu. 277 p.

______ and D. T. Fukuda. 1975. Reef coral and algal communities of the Kahe nearshore region of Oahu, Hawaii. Environmental Department, Hawaiian Electric Co., Inc., 89 p.

______ D. T. Fukuda, and C. R. Lewis. 1981. Annual report. Kahe Generating Station. NPDES monitoring program, vol. 1, text. Honolulu, 237 p.

______ D. T. Fukuda, and C. R. Lewis. 1982. Annual report. Kahe Generating Station. NPDES monitoring program, vol. 1, text, 221 p.

______ and D. T. Fukuda. 1983. Annual report. Kahe Generating Station. NPDES monitoring program, vol. 1, Text, 173 p.

______ and D. T. Fukuda. 1984. Annual report. Kahe Generating Station, NPDES monitoring program, vol. 1, text, 225 p.

______ D. T. Fukuda, and K. S. Oda. 1985 Annual report. Kahe Generating Station, NPDES monitoring program, vol. 1, text, 172 p.

______ K. S. Oda, and J. M. Peck. 1986 Annual report. Kahe Generating Station, NPDES monitoring program, 109 p.

______ and J. C. McCain. 1973. Effects of the Kahe Generating Station on the nearshore environment. A report of the 1973 monitoring program. Hawaiian Electric Co., Inc., Honolulu, 199 p.

Environmental Dept. 1976 Interim report, Kahe Generating Station monitoring Program. Hawaiian Electric Co., Inc., Honolulu, 311 p.

______ 1988. Annual report. Kahe Generating Station, NPDES monitoring program. 66 p.

______ 1989. Annual report. Kahe Generating Station, NPDES monitoring program. 69 p.

______ 1990. Annual report. Kahe Generating Station, NPDES monitoring program. 62 p.

______ 1991. Annual report. Kahe Generating Station, NPDES monitoring program. 60 p.

______ 1992. Annual report. Kahe Generating Station, NPDES monitoring program. 57 p.

______ 1993. Annual report. Kahe Generating Station, NPDES monitoring program for 1992, 33 p.

______ 1994. Annual report. Kahe Generating Station, NPDES monitoring program for 1993, 32 p.

______ 1995. Annual report. Kahe Generating Station, NPDES monitoring program for 1994, 32 p.

______ 1996. Annual report. Kahe Generating Station, NPDES monitoring program for 1995, 32 p.

Fukuda, D. T. and K. S. Oda. 1987. Annual report. Kahe Generating Station, NPDES monitoring program, 69 p.

Marine Advisers, Inc. 1964. Analysis of littoral processes, Kahe, Oahu. Report to the Hawaiian Electric Co., 62 p.

McCain, J. C. 1977. Final report. Kahe generating station, NPDES monitoring program, vol. 1, text. Hawaiian Electric Company, Inc., Honolulu, 427 p.

Stearns Roger Inc. 1973. Environmental assessment, Kahe Generating Station Units Five and Six. Report to the Hawaiian Electric Co., Inc., 350 p.

URS Research Co. 1973. Marine environment impact assessment report for Hawaiian Electric Co., Inc., Kahe Point Facility, Oahu, Hawaii. URS no. 7220-3, 280 p.

APPENDIX A

MARINE MONITORlNG PROGRAM

KAHE GENERATING STATION, HECO

AUGUST 25, 1986

I. INTRODUCTION

This monitoring study describes the investigations which Hawaiian Electric Company Inc. will conduct in order to continue the documentation of the marine effects of the cooling water discharge of Units 1-6 at the Kahe Generating Station. Hawaiian Electric Company proposes to conduct this program during the term of the National Pollutant Discharge Elimination System (NPDES) permit for Kahe 1-6 which was reissued on March 23, 1984, and which expires March 31, 1988.

II.	 DESCRIPTION OF MONITORING STUDIES

A. Temperature Elevations of Thermal Effluent

The current NPDES permit states that "In no instance shall the temperature of the effluent exceed 15 degrees F (8.4 degrees C) above the natural temperature of the receiving waters." Historically, the natural temperature of the receiving waters has been taken as the ambient benthic temperature of a control station well removed from the influence of the thermal discharge. In lieu of the "natural temperature", HECO will use an adjusted intake water temperature as the baseline from which temperature elevations of the thermal discharge will be measured. Based on three years of data consisting of 128 observations between July 1982 and June 1985, the intake temperature exceeds the ambient benthic temperature by a mean value of 0.42 degrees C. Consequently, the new "natural temperature" (i.e., the adjusted intake temperature) will be the measured intake water temperature minus 0.42 degrees C.

The temperature elevations of the thermal discharge (i.e. the thermal effluent temperature minus the adjusted intake temperature) will be monitored for various power plant loads and tidal/current conditions and recorded on a weekly basis.

B. Sediment Throughput Study

Wave-suspended sediments are carried in cooling water through the station's condensers and discharged from the station's outfall into the receiving water.

Suspended sediment samples will be taken at the points of discharge of the thermal effluents from Units 1-4 and 5-6 into the Kahe outfall transition basin (Figure 1). Samples will be taken with a 0.25 meter diameter plankton net placed directly in the discharge. Flow rates will be determined with a General Oceanics digital flow meter which is used to calculate the total volume of water sampled. Quantities of sediment sampled will be converted to milliliters of suspended sediment per cubic meter by dividing the total volume of seawater sampled. Values of milliliters of sediment per cubic meter will be extrapolated to daily throughput estimates by multiplying by the volume of cooling water circulated under operation cubic meters per day).

Sampling will take place quarterly and also during periods of increased wave action.

C. Reef Coral Communities

Ten replicate quadrats were established in 1980 at Stations 2B, 6B, 8A and a control station off Piliokahe Beach Park (Nanakuli Point), approximately 1.8 km north of the outfall (See Figure 2). These replicate quadrats will be monitored for coral coverage and the data analyzed to determine whether statistically significant changes in coral coverage have occurred.

D. Annual Report on Monitoring Studies

An annual report of the data, analysis and findings will be submitted to the DoH Director and EPA Regional Administrator no later than March 1 of each year.

APPENDIX B

1996 DAILY CIRCULATING WATER DISCHARGE TEMPERATURES (°F)

APPENDIX C

CORAL COVERAGE AT REPLICATE QUADRATS

STATIONS 2B, 6B, 8A AND CONTROL

1986, 1995 AND 1996

1986 Kahe Coral Percent Coverage

| Station | Quad. | Porites | Porites | Pocill. | Lept. | Mont. |  | Mont. |  |  | Paly. |  | Sand | Total |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | lobata | comp. | mean. | purp. | verruc. |  | patula |  |  | tuber. |  |  | Coral |  |
| 2B | 1 | 12.0 | 0.0 | 1.9 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 13.8 |  |
| 2B | 2 | 13.0 | 0.0 | 10.3 | 0.0 | 0.0 |  | 0.0 |  |  | 0.6 |  | 0.0 | 23.9 |  |
| 2B | 3 | Replaced | in 1987 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2B | 4 | 12.6 | 0.0 | 8.7 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 21.2 |  |
| 2B | 5 | 23.7 | 0.0 | 3.3 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 27.0 |  |
| 2B | 6 | 16.7 | 2.1 | 1.4 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 20.2 |  |
| 2B | 7 | 15.3 | 0.0 | 7.6 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 22.9 |  |
| 2B | 8 | 14.4 | 0.0 | 3.3 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 17.7 |  |
| 2B | 9 | 23.9 | 0.0 | 6.4 | 0.0 | 0.4 |  | 0.0 |  |  | 0.0 |  | 0.0 | 30.7 |  |
| 2B | 10 | 20.8 | 0.0 | 7.4 | 0.0 | 0.0 |  | 0.0 |  |  | 0.6 |  | 0.6 | 29.5 |  |
| 6B | 1 | 10.7 | 0.0 | 2.7 | 0.0 | 0.6 |  | 0.0 |  |  | 0.6 |  | 0.0 | 14.6 |  |
| 6B | 2 | Replaced | in 1991 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 3 | 4.3 | 0.0 | 3.1 | 0.0 | 0.0 |  | 1.9 |  |  | 3.3 |  | 0.0 | 12.6 |  |
| 6B | 4 | 3.7 | 0.0 | 12.0 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 15.7 |  |
| 6B | 5 | Replaced | in 1991 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 6 | Missing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 7 | 7.0 | 0.0 | 3.9 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 10.9 |  |
| 6B | 8 | Replaced | in 1987 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6B | 9 | 4.9 | 0.0 | 3.9 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 8.9 |  |
| 6B | 10 | Missing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A | 1 | 7.8 | 0.0 | 6.0 | 0.0 |  | 0.0 |  | 0.0 | 0.0 |  | 1.2 |  |  | 15.1 |
| 8A | 2 | 5.2 | 0.0 | 7.8 | 0.0 | 0.0 |  | 0.2 |  |  | 0.0 |  | 0.4 | 13.6 |  |
| 8A | 3 | 8.0 | 0.0 | 7.6 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 15.7 |  |
| 8A | 4 | Replaced | in 1987 |  |  |  |  |  |  |  |  |  |  |  |  |
| 8A | 5 | 7.0 | 0.0 | 8.7 | 0.0 | 0.0 |  | 0.0 |  |  | 0.4 |  | 0.0 | 16.1 |  |
| 8A | 6 | 14.0 | 0.0 | 5.4 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 4.3 | 23.7 |  |
| 8A | 7 | 16.5 | 0.0 | 9.9 | 0.6 | 0.0 |  | 0.0 |  |  | 0.0 |  | 1.0 | 28.0 |  |
| 8A | 8 | 16.1 | 0.0 | 10.3 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 26.4 |  |
| 8A | 9 | 9.1 | 0.0 | 4.3 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 13.4 |  |
| 8A | 10 | 9.1 | 0.0 | 9.1 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 1.4 | 19.6 |  |
| CTL | 1 | 2.8 | 0.0 | 0.9 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 3.6 |  |
| CTL | 2 | 4.7 | 0.0 | 0.0 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 4.7 |  |
| CTL | 3 | 5.8 | 0.0 | 0.0 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 5.8 |  |
| CTL | 4 | 9.7 | 0.0 | 1.9 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 11.5 |  |
| CTL | 5 | 12.0 | 0.0 | 1.4 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 13.4 |  |
| CTL | 6 | 6.4 | 0.0 | 0.0 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 6.4 |  |
| CTL | 7 | Replaced | in 1987 |  |  |  |  |  |  |  |  |  |  |  |  |
| CTL | 8 | 3.9 | 0.0 | 3.7 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 0.0 | 7.6 |  |
| CTL | 9 | 3.1 | 0.0 | 0.0 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 3.3 | 6.4 |  |
| CTL | 10 | 9.5 | 0.0 | 2.3 | 0.0 | 0.0 |  | 0.0 |  |  | 0.0 |  | 1.4 | 13.2 |  |

1995 Kahe Coral Percent Coverage

| Station | Quad. | Porites | Porites | Pocill. | Lept. | Mont. | Mont. | Paly. | Sand | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | lobata | comp. | mean. | purp. | verruc. | patula | tuber. |  | Coral |
| 2B | 1 | 5.6 | 0.0 | 4.7 | 8.2 | 0.0 | 0.6 | 0.0 | 0.0 | 19.2 |
| 2B | 2 | 9.3 | 0.0 | 5.6 | 0.0 | 0.0 | 0.8 | 0.0 | 0.0 | 15.7 |
| 2B | 3 | 16.1 | 0.0 | 10.5 | 0.0 | 1.0 | 0.0 | 0.0 | 0.0 | 27.6 |
| 2B | 4 | 8.9 | 0.0 | 6.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 15.7 |
| 2B | 5 | 25.6 | 0.0 | 3.1 | 0.0 | 0.2 | 2.7 | 0.0 | 0.0 | 31.5 |
| 2B | 6 | 51.8 | 8.9 | 4.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 64.7 |
| 2B | 7 | 16.1 | 0.0 | 6.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 23.1 |
| 2B | 8 | 19.2 | 0.0 | 2.5 | 0.0 | 0.0 | 1.9 | 0.0 | 0.0 | 23.5 |
| 2B | 9 | 24.1 | 0.0 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 24.5 |
| 2B | 10 | 1.4 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.2 | 0.0 | 1.9 |
| 6B | 1 | 13.6 | 0.0 | 0.0 | 0.2 | 0.4 | 0.8 | 0.0 | 0.0 | 15.1 |
| 6B | 2 | 15.3 | 0.0 | 2.9 | 0.0 | 0.0 | 3.3 | 0.0 | 2.5 | 23.9 |
| 6B | 3 | 4.5 | 0.0 | 1.4 | 0.0 | 0.0 | 3.9 | 0.0 | 0.0 | 9.9 |
| 6B | 4 | 6.8 | 0.0 | 6.0 | 0.0 | 0.2 | 0.0 | 0.6 | 0.0 | 13.6 |
| 6B | 5 | 9.5 | 0.0 | 3.7 | 0.0 | 0.0 | 0.4 | 0.0 | 0.0 | 13.6 |
| 6B | 6 | 10.9 | 0.0 | 7.0 | 0.2 | 0.0 | 1.0 | 0.0 | 0.0 | 19.2 |
| 6B | 7 | 14.0 | 0.0 | 4.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 18.8 |
| 6B | 8 | 13.0 | 0.0 | 6.6 | 0.6 | 0.0 | 0.0 | 0.0 | 2.1 | 22.3 |
| 6B | 9 | 5.2 | 0.0 | 3.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 9.5 |
| 6B | 10 | 8.9 | 0.0 | 9.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 17.9 |
| 8A | 1 | 1.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 2.3 | 3.5 |
| 8A | 2 | 1.6 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.9 |
| 8A | 3 | 0.8 | 0.0 | 5.4 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 6.4 |
| 8A | 4 | 1.0 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 2.5 | 3.7 |
| 8A | 5 | 5.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 5.6 |
| 8A | 6 | 7.6 | 0.0 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 8.0 |
| 8A | 7 | 5.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 5.2 |
| 8A | 8 | 3.7 | 0.0 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.4 | 4.5 |
| 8A | 9 | 4.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 4.7 |
| 8A | 10 | 2.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 2.9 |
| CTL | 1 | 7.8 | 0.0 | 6.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 14.2 |
| CTL | 2 | 5.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 5.4 |
| CTL | 3 | 3.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 3.3 |
| CTL | 4 | 7.0 | 0.0 | 6.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 13.0 |
| CTL | 5 | 12.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 12.8 |
| CTL | 6 | 5.6 | 0.0 | 0.6 | 0.0 | 0.0 | 0.8 | 0.0 | 0.0 | 7.0 |
| CTL | 7 | 4.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 4.3 |
| CTL | 8 | 2.9 | 0.0 | 3.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 6.2 |
| CTL | 9 | 11.1 | 0.0 | 0.4 | 0.0 | 0.8 | 0.0 | 0.0 | 0.0 | 12.4 |
| CTL | 10 | 4.5 | 0.0 | 2.9 | 0.0 | 1.0 | 0.0 | 0.0 | 0.0 | 8.5 |

1996 Kahe Coral Percent Coverage

| Station | Quad. | Porites | Porites | Pocill. | Lept. | Mont. | Mont. | Paly. | Sand | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | lobata | comp. | mean. | purp. | verruc. | patula | tuber. |  | Coral |
| 2B | 1 | 9.5 | 0.0 | 4.3 | 10.1 | 0.2 | 2.7 | 0.4 | 0.0 | 27.2 |
| 2B | 2 | 5.2 | 0.0 | 8.2 | 0.0 | 0.0 | 1.9 | 0.0 | 0.0 | 15.3 |
| 2B | 3 | 16.7 | 0.0 | 12.0 | 0.0 | 0.2 | 2.1 | 0.0 | 0.0 | 30.9 |
| 2B | 4 | 9.9 | 0.0 | 7.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 17.3 |
| 2B | 5 | 21.4 | 0.0 | 1.9 | 0.0 | 0.0 | 0.0 | 0.0 | 1.4 | 24.7 |
| 2B | 6 | 53.4 | 12.6 | 6.6 | 0.0 | 0.0 | 0.4 | 0.0 | 0.0 | 73.0 |
| 2B | 7 | 17.1 | 0.0 | 6.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 23.9 |
| 2B | 8 | 20.8 | 0.0 | 1.9 | 0.0 | 0.0 | 2.7 | 0.0 | 0.0 | 25.4 |
| 2B | 9 | 34.6 | 0.0 | 1.4 | 0.0 | 1.4 | 0.0 | 0.0 | 0.0 | 37.5 |
| 2B | 10 | 1.9 | 0.0 | 2.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 3.9 |
| 6B | 1 | 11.5 | 0.0 | 0.4 | 0.0 | 1.0 | 1.6 | 0.0 | 0.0 | 14.6 |
| 6B | 2 | 18.1 | 0.0 | 0.0 | 0.0 | 0.4 | 2.9 | 0.0 | 1.9 | 23.3 |
| 6B | 3 | 5.8 | 0.0 | 3.5 | 0.0 | 1.9 | 3.9 | 0.0 | 0.0 | 15.1 |
| 6B | 4 | 7.2 | 0.0 | 6.6 | 0.0 | 1.4 | 0.0 | 0.0 | 0.4 | 15.7 |
| 6B | 5 | 10.5 | 0.0 | 4.5 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 15.3 |
| 6B | 6 | 12.4 | 0.0 | 10.9 | 0.0 | 0.0 | 0.8 | 0.0 | 0.0 | 24.1 |
| 6B | 7 | 15.5 | 0.0 | 0.4 | 0.0 | 0.0 | 0.6 | 0.0 | 0.0 | 16.5 |
| 6B | 8 | 14.0 | 0.0 | 8.0 | 0.0 | 0.0 | 3.7 | 0.0 | 1.0 | 26.8 |
| 6B | 9 | 7.4 | 0.0 | 4.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 12.4 |
| 6B | 10 | 6.2 | 0.0 | 9.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 15.9 |
| 8A | 1 | 2.5 | 0.0 | 0.4 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 3.1 |
| 8A | 2 | 2.3 | 0.0 | 0.6 | 0.0 | 0.0 | 0.2 | 0.0 | 0.8 | 3.9 |
| 8A | 3 | 2.9 | 0.0 | 6.4 | 0.0 | 0.0 | 0.4 | 0.0 | 0.4 | 10.0 |
| 8A | 4 | 2.5 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 3.3 |
| 8A | 5 | 3.9 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 4.5 |
| 8A | 6 | 9.2 | 0.0 | 1.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 11.1 |
| 8A | 7 | 7.2 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 7.6 |
| 8A | 8 | 7.2 | 0.0 | 2.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 9.4 |
| 8A | 9 | 4.3 | 0.0 | 0.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 5.1 |
| 8A | 10 | 3.3 | 0.0 | 8.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 11.3 |
| CTL | 1 | 3.3 | 0.0 | 0.6 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 4.5 |
| CTL | 2 | 4.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 4.9 |
| CTL | 3 | 4.9 | 0.0 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 5.5 |
| CTL | 4 | 8.0 | 0.0 | 1.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 9.0 |
| CTL | 5 | 13.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 13.5 |
| CTL | 6 | 3.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 0.0 | 0.0 | 4.5 |
| CTL | 7 | 4.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 4.9 |
| CTL | 8 | 2.7 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 2.9 |
| CTL | 9 | 11.3 | 0.0 | 1.0 | 0.0 | 1.4 | 0.8 | 0.0 | 0.0 | 14.5 |
| CTL | 10 | 7.2 | 0.0 | 2.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 9.8 |

1994-95 POINT QUADRAT SAMPLING GRID

The page following represents one half of the point-quadrat grid used to quantify coral and substratum type from projected images of the bottom previously photographed onto color transparency medium (35 mm slides). Quantification involved counting the species and substratum types under each of the points on the full sheet after size adjustment of the projected image to match the field quadrat frame to the grid sheet boundaries. Grid lines were used only to maintain orientation.