Hydrography Survey Report: Western Gulf of Maine

Regional Marine Research Program

R/V Gulf Challenger Cruises

16 March 1994/ 21 March 1994

23 May 1994

3 August 1994

31 October 1994

16 March 1995/ 20 March 1995

H. Feng, K. Garrison, F.L. Bub, and W. Brown

Ocean Process Analysis Laboratory

Institute for the Study of Earth, Ocean and Space

Dept. of Earth Sciences

University of New Hampshire

Durham, NH 03824

( 25 August 1995 )

Contents

0. Abstract

1. Introduction

2. Data Acquisition

3. Data Processing

4. Data Corrections

5. Data Presentations

6. Acknowledgements

7. References

Abstract

This report describes the hydrographic measurements obtained in the western

Gulf of Maine between March 1994 and March 1995 as part of one of the

University of New Hampshire's components in the Regional Marine Research

Program (RMRP). This report and these data can be accessed through the RMRP

Research Environmental Data and Information Management System (REDIMS) via

the WWW address: http://opal-www.unh.edu/redims.html.

1. Introduction

This data report presents the University of New Hampshire (UNH)

hydrographic survey measurements, obtained as part of the Regional Marine

Research Program (RMRP)-supported observational/modeling study of the

western Gulf of Maine (GOM) coastal current. This joint UNH/Dartmouth

research investigates the structure and temperature variability of the

coastal current in the western GOM, the role of the coastal current in the

general alongcoast transport and the linkage between the coastal current

and circulation in the interior of the Gulf. This region is characterized

by stratification, which is somewhat stronger in the western GOM than

elsewhere in the coastal gulf because the tidal currents are relatively

less energetic there. It appears that the combination of the stronger

stratification and the river-discharge enhanced coastal current are

important in the distribution and transport of toxic dinoflagellates (i.e.

red tide) during the spring and early summer (Franks and Anderson, 1992).

These UNH measurements complement those of other RMRP researchers

addressing the latter issue.

The UNH field observation program consisted of the deployment of moored

conductivity/temperature/bottom pressure instrumentation and a series of

seasonal hydrographic cruises. The moored arrays were deployed at

representative sites: in the coastal current and interior Gulf, Cape

Porpoise (CP) and Wilkinson Basin (WB) respectively (Figure 1.1). The

moored pressure array will provide across-shelf pressure difference

fluctuations which are proportional to along-shelf geostrophic currents

(Brown et al., 1985). The temperature/conductivity measurements will

resolve the hourly changes in the physical properties of the water at

several depths at the two moorings. Hydrographic profile measurements from

a series of cruises will provide relatively high (spatial) resolution water

property information along the across-shelf transect of the moored

measurements. The combination of the time series from the moorings and the

spatial series from hydrographic cruises helps us to understand the

characteristics of the along-coast currents in the western Gulf. This

report describes the hydrographic data from the R/V Gulf Challenger cruises

of March 16, March 21, May 23, August 3, and October 31, 1994 and March 16

and March 20 1995. A total of 97 CTD cast stations were occupied at the

locations listed in Table 1. The CTD station arrangement was designed to

obtain the across-shelf hydrographic information along the transect from

nearshore (25m) to the deepest water in Wilkinson Basin (269m).

2. Data Acquisition

For the March 16 and 21 1994 cruises, a Seacat Profiler (manufactured by

the Sea-Bird Electronic, Inc.) was used to measure electrical conductivity

and temperature versus pressure (CTD) profiles. The sampling rate of this

CTD system was set at 2 scans per second. Data acquisition, display and

storage of the CTD were managed by an on-board desktop computer with the

Sea-Bird SEASOFT CTD software.

For all the other cruises, an internally recording CTD profiling system

with Sea Bird sensors (Martini, 1990) was used. The typical sampling rate

for this system is 32 scans per second. The data acquisition and display

were also controlled by a desktop computer. The raw data were stored in

computer memory and later stored on floppy disk.

At each of the 97 stations during the four cruises, the CTD was lowered at

a rate of about 20 meters per minute to a depth within 5 meters of the

bottom. After each cast, the temperature and salinity profiles were

displayed on the laptop computer's screen to check if the data acquisition

procedure was functioning normally.

A bottom water sample was collected with a 5-liter Niskin bottle at every

CTD station. The conductivity of each water sample was measured using a

Guildline 8400A Autosal. The corresponding salinity was used to correct the

salinity values derived from the raw CTD measurements (see below).

3. Data Processing

The steps in processing the CTD data consisted of:

* Normalization of raw CTD measurements:

o Raw ASCII hexidecimal CTD output was converted into scientific

and engineering units.

* Editing Normalized CTD Data:

o Noise contamination due particularly to ship roll was removed.

For instance, on the CTD downcast, data associated with

decreasing pressure were discarded; on the upcast, data

associated with decreasing pressure were discarded.

* Filtering and Interpolation of Edited Data:

o A triangular filter was used to produce the 1decibar (dbar)

averaged temperature and salinity versus pressure data files from

the unequally-spaced cast data from each station. The profile

data were stored on floppy disks for use in post-processing and

plotting.

4. Data Corrections & Estimated Accuracy/Precision

Pressure: At each of the stations, the pressure was recorded when the CTD

instrument was on the ship's deck just before the start of the cast. The

average deck pressure was removed from the profile pressure data. A

pre-cruise calibration of the pressure sensor was used when the raw CTD

data were normalized. The estimated uncertainty in the pressure

measurements is plus or minus 0.2 dbar (Garrison and Brown, 1989).

Temperature: The pre-cruise calibration for the CTD temperature sensors

resulted in a residual variance (Tobs.-Tcal.) of less than 0.001oC in the

range from 2oC to 18oC. Since these inaccuracies were within the

manufacturer's specified accuracy of temperature sensors (plus or minus

0.005oC), the CTD temperatures were not corrected further.

Salinity: CTD salinity was computed from the measured conductivity and

temperature using the practical salinity scale of 1980 (Fofonoff and

Millard, 1983). The computed salinities were corrected by comparing

CTD-derived salinities with near-bottom bottle sample salinities collected

on every cast according to the procedure described by Garrison and Brown

(1989).

For the March 16 and 21 1994 surveys, bottom salinity ranged from about

31.7 - 33.7 psu. Bottom bottle salinity was then plotted versus CTD-derived

salinity. The CTD-derived minus bottle salinity difference was also plotted

versus depth (Figure 1.2). Such plots provide us with the basis to reject

spurious data probably related to leaking sample bottles, strong salinity

gradient, incorrect sampling, etc. The bottle data from station 1 to

station 4 were eliminated because of an apparent (unknown) systematic

problem. A visual comparison between bottom bottle salinity and CTD

salinity showed that CTD salinity was systematically about 0.4 psu less

than bottom bottle salinity. A linear regression of bottle salinity on CTD

salinity yielded the salinity correction equation:

Sc = -0.545 + 1.0036*S,

where S is CTD salinity with a standard deviation of discrepancies of

0.0105 psu. This equation was used to correct all of the March 1994 SeaCat

CTD salinities.

For the May 23 1994 survey, a different CTD measurement system was used,

but the salinity correction method was similar to that for the March cruise

data. We rejected two obviously bad data points from station 2 and station

24 (Figure 2.2). The linear salinity correction equation, based on the

CTD/bottle salinity intercomparison in Figure 2.2 was

Sc = 0.0073 + 1.0021*S,

with a standard deviation of discrepancies of 0.0181 psu.

For August 3, 1994 survey correction, we rejected data points from stations

1 and 15 (Figure 3.2). The correction equation, based on the CTD/Bottle

salinity intercomparison, was

Sc = -0.2456 + 1.0089*S,

with a standard deviation of discrepancies of 0.0155 psu.

For October 31, 1994 survey, the salinity correction equation, based on the

CTD/bottle salinity intercomparison (Figure 4.2) was

Sc = -0.4016 + 1.0135*S,

with a standard deviation of discrepancies of 0.0109 psu.

For the March 16 and 20, 1995 surveys, the salinity correction equation

based on the CTD/bottle salinity intercomparison (Figure 5.2) was

Sc = -0.2525 +1.0063*S,

with a standard deviation of discrepancies of 0.0302 psu.

5. Data Presentations

The corrected hydrographic data are presented in several standard

presentations including: 1) station profile plots, station [Image]-S

diagrams; 2) vertical section contour plots. All the derivatives from the

basic CTD data and plots were computed and generated using MATLAB.

Profiles and [Image]-S Diagrams: A set of profile plots (potential

temperature, salinity, sigma-theta) and [Image] -S diagram for each of the

97 CTD stations are presented in Figures A1-A97. Two profile plots are

given in each figure. The surface to 25m deep profile plotpresents details

of water property structure in the main thermocline/halocline/pycnocline

zone. The other plot presents the water property structure for the entire

water column.

Vertical Section: Potential temperature, salinity, and sigma-theta sections

for several sets of stations from each cruise are presented in Figures 1.3

- 1.5, Figures 2.3 - 2.6, Figures 3.3 - 3.5, Figures 4.3 - 4.6, and Figures

5.3-5.4 respectively. Contour intervals are 0.5oC for temperature, 0.2 psu

for salinity and 0.2 kg m-3 for sigma-theta for the March and October; and

1.0oC for temperature, 0.5 psu for salinity and 0.5 kg m-3 for sigma-theta

for the May and August. The CTD station numbers are indicated along the top

horizontal axis.

6. Acknowledgements

We appreciate the efforts of the captain and crew of UNH's R/V Gulf

Challenger in helping us conduct both this and the other components of the

field program. We are grateful for the help provided by T. Loder and C.

Coniaris in processing the bottle salinities. H. Feng, K. Garrison and W.

Brown are supported by RMRP UM-S228 and Sea Grant R/CE-108, UM-S221 #2;

Frank Bub is supported by NOAA GLOBEC NA36GP0298.

7. References

Brown, W. S., N. R. Pettigrew, and J.D. Irish, 1985. The Nantucket Shoals

Flux Experiment (NSFE79) Part II: The structure and variability of

across-shelf pressure gradients, J. Phys. Oceanogr., 15, 749-774.

Fofonoff, N. P. and R. C. Millard Jr., 1983. Algorithms for compilation of

fundamental properties of seawater, UNESCO Technical Papers in Marine

Science, no. 44. UNESCO, Paris, France, 53 pages.

Franks, J.S., and D.M. Anderson, 1992. Alongshore transport of a toxic

phytoplankton bloom in a buoyancy current: Alexandrium tramarense in the

Gulf of Maine, Marine Biology, 112, 153-164.

Garrison, K. M. and W. S. Brown, 1989. Hydrographic survey in the Gulf of

Maine July-August 1987, UNH Tech. Rpt. No. UNHMP-T/DR-SG-89-5, Univ. of NH,

Durham, NH.

Martini, M. , 1990. Profile system design, construction and testing, M.S.

thesis, University of New Hampshire, Durham, NH.

Table 1

CTD station information for R/V Gulf Challenger March/May/August/October

1994 and March 1995 Cruises. Nearby moorings are indicated in parentheses

where appropriate. Position, depth, date, and time are for start of cast.

The stations at the Cape Porpoise (CP) and the Wilkinson Basin (WB) mooring

sites are indicated.

CTD station    Latitude   Longitude   Water Depth  Time     Date

number     (deg min N) (deg min W)   (meters)     (Z)

________________________________________________________________________

001         43 16.71    70 12.01       107      14:40  16 MAR 94

002         43 21.04    70 21.81        51      17:54  16 MAR 94

003         43 19.08    70 17.71        67      18:21  16 MAR 94

004 (CP)    43 16.63    70 11.99       109      18:45  16 MAR 94

005 (WB)    42 46.56    69 44.88       269      13:58  21 MAR 94

006         42 52.14    69 49.92       257      15:53  21 MAR 94

007         42 55.77    69 54.83       237      16:42  21 MAR 94

008         43 03.65    69 59.81       155      17:27  21 MAR 94

009         43 07.26    70 03.25        51      17:56  21 MAR 94

011         43 12.03    70 07.76       143      18:55  21 MAR 94

012         43 14.35    70 09.96       132      19:20  21 MAR 94

013         43 16.45    70 11.80       111      19:44  21 MAR 94

014         43 17.99    70 14.96        82      20:25  21 MAR 94

015         43 18.98    70 17.35        61      20:42  21 MAR 94

016         43 19.96    70 19.54        50      20:56  21 MAR 94

017         43 21.03    70 21.78        46      21:10  21 MAR 94

018         43 21.82    70 23.85        29      21:23  21 MAR 94

001         43 21.92    70 23.47        23      10:15  23 MAY 94

002         43 21.29    70 22.16        53      10:36  23 MAY 94

003         43 19.98    70 19.77        53      11:13  23 MAY 94

004         43 19.07    70 17.57        60      11:32  23 MAY 94

005         43 17.83    70 15.66        84      12:19  23 MAY 94

006 (CP)    43 16.60    70 12.30       110      13:36  23 MAY 94

007         42 14.39    70 09.07       139      14:02  23 MAY 94

008         43 12.10    70 07.81       152      14:28  23 MAY 94

009         43 09.63    70 05.79       152      14:56  23 MAY 94

010         43 07.30    70 03.66        52      15:17  23 MAY 94

011         43 03.73    70 00.09       158      15:48  23 MAY 94

012         42 56.04    69 55.18       235      16:47  23 MAY 94

013         42 52.31    69 50.23       256      17:39  23 MAY 94

014 (WB)    42 46.92    69 45.33       269      19:52  23 MAY 94

015         42 50.21    69 55.88       213      20:42  23 MAY 94

016         42 52.93    70 05.80        94      21:23  23 MAY 94

017         42 54.02    70 08.12        63      21:40  23 MAY 94

018         42 54.58    70 12.34       140      22:03  23 MAY 94

019         42 56.83    70 18.94       127      22:37  23 MAY 94

020         42 59.10    70 27.34        98      23:15  23 MAY 94

021         42 59.62    70 29.15        87      23:29  23 MAY 94

022         43 01.73    70 31.67        75      23:48  23 MAY 94

023         43 02.09    70 34.41        57      24:04  23 MAY 94

024         43 02.49    70 37.58        27      00:21  23 MAY 94

025         43 03.12    70 39.12        24      00:34  23 MAY 94

001         43 21.74    70 24.30        24      11:53  03 AUG 94

002         43 21.07    70 22.05        54      12:23  03 AUG 94

003         43 20.08    70 19.87        48      13:09  03 AUG 94

004 (CP)    43 19.19    70 17.52        55      13:31  03 AUG 94

005         43 28.28    70 15.23        87      13:51  03 AUG 94

006 (CP)    43 16.77    70 12.06       103      14:09  03 AUG 94

007         43 14.54    70 10.10       133      15:30  03 AUG 94

008         43 12.33    70 07.70       143      15:54  03 AUG 94

009         43 09.90    70 05.66       148      16:40  03 AUG 94

010         43 07.34    70 03.64        51      17:14  03 AUG 94

011         43 03.79    70 00.12       158      17:48  03 AUG 94

012         42 56.12    69 55.06       232      18:39  03 AUG 94

013         42 52.53    69 49.90       254      19:19  03 AUG 94

014 (WB)    42 47.04    69 44.80       268      20:11  03 AUG 94

015         42 50.22    69 55.72       215      21:35  03 AUG 94

001 (CP)    43 16.71    70 12.30       108      18:14  31 OCT 94

002         43 18.27    70 15.50        84      18:52  31 OCT 94

003         43 19.28    70 17.76        53      19:05  31 OCT 94

004         43 19.98    70 20.03        60      19:29  31 OCT 94

005         43 21.14    70 21.21        52      19:41  31 OCT 94

006         43 21.97    70 24.15        26      19:58  31 OCT 94

007         43 14.51    70 10.08       134      21:06  31 OCT 94

008         43 12.14    70 07.93       140      21:33  31 OCT 94

009         43 09.98    70 07.50       149      21:58  31 OCT 94

010         43 07.40    70 03.45        51      22:24  31 OCT 94

011         43 03.71    70 00.19       154      22:50  31 OCT 94

012         42 55.96    69 54.97       224      23:36  31 OCT 94

013         42 52.37    69 49.83       254      00.12  01 NOV 94

014 (WB)    42 46.99    69 44.50       266      01.03  01 NOV 94

015         42 50.11    69 55.57       214      02:05  01 NOV 94

016         42 51.84    70 08.32        99      02:53  01 NOV 94

017         42 53.84    70 08.84        62      03.17  01 NOV 94

018         42 54.65    70 12.27       128      03:36  01 NOV 94

019         42 56.86    70 19.09       126      04:08  01 NOV 94

020         42 59.12    70 27.51        96      04:44  01 NOV 94

021         42 59.61    70 29.92        90      05:01  01 NOV 94

022         43 01.74    70 31.50        64      05:23  01 NOV 94

023         43 02.13    70 34.54        52      05:39  01 NOV 94

024         43 02.41    70 37.77        32      05:56  01 NOV 94

025         43 03.06    70 39.15        26      06:09  01 NOV 94

001 (CP)    43 16.74    70 12.07       110      15:54  16 MAR 95

002         43 18.20    70 15.40        85      16:30  16 MAR 95

003         43 19.03    70 17.67        64      16:50  16 MAR 95

004         43 20.10    70 19.92        47      17:07  16 MAR 95

005         43 21.06    70 22.21        53      17:25  16 MAR 95

006         43 21.96    70 24.13        25      17:41  16 MAR 95

007         43 22.02    70 24.15        25      17:49  16 MAR 95

008 (WB)    42 47.06    69 44.94       269      14:41  20 MAR 95

009         42 50.22    69 55.81       214      15:44  20 MAR 95

010         42 52.97    70 05.87        93      16:28  20 MAR 95

011         42 53.96    70 06:31        64      16:48  20 MAR 95

012         42 54.65    70 12.41       149      17:14  20 MAR 95

013         42 56.18    70 19.00       125      17:48  20 MAR 95

014         42 59.27    70 27.56        98      18:26  20 MAR 95

015         42 01.73    70 33.71        58      18:57  20 MAR 95

CP:  Cape Porpoise mooring site

WB:  Wilkinson Basin mooring site