Edward Peltzer

Total Organic Carbon Methodology

Peltzer, Edward T. (1993).  Shipboard determination of total organic carbon

by a high temperature combustion/direct injection technique.  U.S. Joint

Global Ocean Flux Study - Equatorial Pacific Protocols, 1993, section 21A.

Alternatively, see:  Chapter 16 of the JGOFS Data Protocols.  Determination

of Dissolved Organic Carbon by a High Temperature Combustion/Direct Injection

Technique.  JGOFS Report Nr. 19

The EqPac methods were followed explicity on the Ross Sea Process cruise 3

with three exceptions:

1.  Low carbon water (LCW) prepared onboard ship using a commercial NanoPure

(R) UV/TOC system was used to measure the instrument blank as opposed to

the carbon free distilled water (CFDW) that was used in EqPac.

2.  The LCW prepared onboard was checked against LCW prepared at WHOI using a

Milli-Q UV/TOC (R) system and shipped to New Zealand in glass bottles.

This water was assigned a background TOC concentration of zero.

3.  Arabian sea deep water was used to prepare the daily calibration

standards.  This water has a background TOC concentration of 42.3 uMC.

PLEASE NOTE:

1.  These samples were NOT filtered; ergo they are TOTAL ORGANIC CARBON

(TOC) analyses,  not dissolved organic carbon analyses (DOC)

2.  During this cruise some very low level contamination was encountered.

This contamination was introduced during the sampling process.  It could

be minimized by drawing the TOC samples immediately after the oxygen

samples.

3.  In the data table, obviously "bad samples" have been removed.  However,

given the low level contamination we were experiencing, it is possible

that some contaminated samples are still present.  Our best guess at a

clean data set has been reported here.