CSC Technical Report CSC/1-99/001

NOAA Cruise APR98SAB:

South Atlantic Bight Cruise

Ruth W. Kiambo

SC Seagrant

NOAA Coastal Services Center

2234 Hobson Avenue

Charleston, SC 29405

Mary E. Culver

TPMC/ NOAA Coastal Services Center

2234 Hobson Avenue

Charleston, SC 29405

Patricia A. Tester

Elin Haugen

NOAA Center for Coastal Fisheries

Habitat Research

101 Pivers Island Road

Beaufort, NC 28516

Richard P. Stumpf

NOAA National Center for Coastal Ocean Science

Center for Coastal Monitoring and Assessment

Silver Spring, MD 20910

Ajit Subramaniam

REMSA, Inc.

NOAA Science Center Room 711b

4700 Silver Hill Road, Stop 9910

Washington DC 20233-9910

John C. Brock

NOAA Coastal Services Center

Now at: U.S. Geologic Survey

600 4th St.

St. Petersburg, FL  33701

David Eslinger

NOAA Coastal Services Center

2234 Hobson Avenue

Charleston, SC 29405

January 1999

UNITED STATES DEPARTMENT OF COMMERCE

National Oceanic and Atmospheric Administration

COASTAL SERVICES CENTER

2234 S. Hobson Avenue, Charleston, SC 29405-2413

Abstract

The algorithms for the calculation of chlorophyll a concentrations in the coastal waters of the U.S. need to be verified for the Sea-viewing Wide Field-of-view Sensor (SeaWiFS) on board the SeaStar spacecraft.  This requires precise optical measurements below the sea surface in coastal waters from which remote sensing reflectance, downwelling irradiance and upwelling radiance can be calculated.

A total of 30 stations in the South-Atlantic Bight between Beaufort, North Carolina and Jacksonville, Florida were occupied between April 3, 1998 and April 27, 1998.  In-situ measurements of temperature, spectral downwelling irradiance, and spectral upwelling radiance were made along with above-surface spectral downwelling irradiance.  Samples were collected for measurement of  surface chlorophyll a concentration, phytoplankton pigments, and total suspended sediments.  In-situ depth, temperature and salinity were also measured.

Chlorophyll a and TSS concentrations in waters of the South Atlantic Bight during this cruise in April were highly variable.  Chlorophyll a concentrations tended to be higher near shore, and TSS showed no discernible geographic pattern, with high and low concentrations being measured at both nearshore and offshore stations.

The OC2 version 2 algorithm over-estimated the chlorophyll a concentration by a factor of 4 and as much as a factor of 50.  CZCS pigment concentrations are estimated more successfully with ratios of measured to estimated pigment ranging from 1 to 5.  The overestimates may be due, in large part, to high concentrations of colored dissolved organic matter (cdom).  High diffuse attenuation coefficients at 380 and 412 nm were measured; these wavelengths are affected by cdom absorption.  The OC2 algorithm needs refinement in order to estimate chlorophyll a in coastal waters of the South Atlantic Bight.

Table of Contents

I.	Introduction	1

II.	Objectives	1

III.	Methods	1

A.	Sampling Location	1

B.	Sampling Platform	1

C.	Sample Collection Methods Summary	1

D.	Sampling Gear	3

E.	Bottle Samples	4

F.	Optical Data Processing	4

IV.	Results	5

A.	Bottle Samples	5

B.  Optical data	6

C.  Algorithm Evaluation	7

V.  Summary	10

VI.  References	10

VII.	Metadata	10

VIII.  Appendix A - Water Column Profile Data Figures	18

IX.  Appendix B - Example Profile Header information	78

X.	Appendix C - Calibration Certificates	79

List of Figures

Figure 1.  Location of stations	2

Figure 2.  Deployment of the PRR600s	2

List of Tables

Table 1.  Station Locations	3

Table 2.  Center Wavelengths for the PRR System	4

Table 3.  Pigment Analyses	5

Table 4.  Summary of Optical Profiles	6

Table 5  Algorithm Evaluation	8

Acknowledgments

We thank the Captain and the crew of the R/V Cape Hatteras for assistance provided.  A NOAA Coastal Ocean Program grant to Dr. Patricia Tester and Dr. Richard Stumpf made this cruise possible.

# I.	Introduction

Monitoring the health of U.S. coastal waters is an important goal of the National Oceanic and Atmospheric Administration (NOAA).  Satellite ocean color sensors are capable of providing regular synoptic water quality data for the U.S. coast.  Algorithms are used to derive products such as chlorophyll biomass from satellite data to study short and long term changes in water quality; however, these algorithms need to be evaluated and validated.  Towards this purpose, scientists from the National Ocean Service (NOS) undertook a twenty-five day cruise in the South Atlantic Bight (Beaufort, NC to Jacksonville, FL).

# II.	Objectives

The objectives of this cruise were to obtain sub-surface downwelling irradiance, upwelling radiance, chlorophyll pigment concentration and total suspended sediment concentration in coastal waters.  The remote sensing reflectance measurements calculated from these samples were used to evaluate and validate the OC2 version 2 (O’Reilly et al. 1998) algorithm for the NASA/OrbImage Sea-viewing Wide Field-of-view Sensor (SeaWiFS).

# III.	Methods

A description of the sample collection methods and instruments is detailed in the following sections.

## A.	Sampling Location

Optical profile measurements were made at a total of 30 stations, from April 3, 1998 thru April 27, 1998 (Figure 1).  Surface samples were acquired at these locations for determination of total suspended solids (TSS) concentration and for chlorophyll analysis by fluorometric and High-Pressure Liquid Chromatography (HPLC) techniques.

## B.	Sampling Platform

The R/V Cape Hatteras, belonging to the Duke/University of North Carolina Oceanographic Consortium, was used on this cruise.  The R/V Cape Hatteras is a 41-meter (m), steel hull, twin diesel engine oceanographic research vessel.  It operates both on the continental shelf and in the deep sea in the western North Atlantic, concentrating in the region between Nova Scotia and the Caribbean.

## C.	Sample Collection Methods Summary

A Biospherical Instruments, Inc. Profiling Reflectance Radiometer (PRR600s) was deployed off the starboard side of the vessel using a davit (Figure 2). The PRR600s measured in-situ spectral downwelling irradiance, spectral upwelling radiance, and temperature.  Surface bucket samples were obtained for pigment analysis.  In-situ temperature, salinity, and density were also measured at some stations with a SeaBird Conductivity-Temperature-Depth (CTD) sensor.

Figure 1.  Location of stations

Figure 2.  Deployment of the PRR600s

Table 1.  Station Locations

| Date | Station | Latitude | Longitude | Time | ChlF | TSS |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (GMT) | VolFilt (L) | VolFilt (L) |
| 4/3/98 | 6 | 34.6522 | -76.6583 | - | 0.1 | 1.0 |
| 4/4/98 | 25 | 34.6522 | -76.6583 | - | 0.1 | 1.0 |
| 4/5/98 | 35 | 34.5403 | -77.2544 | 14:09 | 0.1 | - |
| 4/5/98 | 38 | 34.5403 | -77.2544 | 16:11 | 0.1 | 1.0 |
| 4/6/98 | 54 | 33.8478 | -77.4336 | 14:24 | 0.5 | 1.5 |
| 4/6/98 | 55 | 33.8478 | -77.4336 | - | 0.5 | 2.0 |
| 4/6/98 | 57 | 34.2256 | -77.6444 | 18:16 | 0.5 | 2.0 |
| 4/6/98 | 60 | 34.2961 | -77.6872 | 19:24 | 0.2 | 1.0 |
| 4/7/98 | 82 | 34.2853 | -77.6803 | 17:17 | 0.3 | 2.0 |
| 4/8/98 | 96 | 33.6233 | -76.3044 | - | 1.0 | 2.0 |
| 4/8/98 | 97 | 33.5639 | -76.4914 | 16:08 | 1.0 | 2.0 |
| 4/10/98 | 104a | 34.4903 | -77.2233 | - | 0.2 | 1.0 |
| 4/11/98 | 104b | 34.2933 | -77.6825 | 16:15 | 0.2 | 1.0 |
| 4/12/98 | 104c | 33.7100 | -77.3875 | 17:11 | 0.5 | 2.0 |
| 4/13/98 | 104d | 33.2306 | -77.7178 | 17:06 | 0.5 | 2.0 |
| 4/14/98 | 117 | 30.8361 | -80.7631 | 16:06 | 0.5 | 1.5 |
| 4/15/98 | 144a | 30.9069 | -81.3039 | 18:09 | 0.2 | 1.0 |
| 4/18/98 | 144b | 30.2267 | -81.2900 | 16:54 | 0.1 | 1.5 |
| 4/18/98 | 144c | 30.0764 | -81.2467 | 18:19 | 0.1 | 1.5 |
| 4/19/98 | 200 | 29.9303 | -81.2628 | - | 0.1 | 1.5 |
| 4/20/98 | 221 | 30.4142 | -80.0019 | - | 1.0 | 2.0 |
| 4/21/98 | 234 | 30.4181 | -80.9633 | 14:42 | 0.5 | 2.0 |
| 4/21/98 | 235 | 30.4183 | -81.1564 | 15:56 | 0.5 | 2.0 |
| 4/22/98 | 243 | 32.2206 | -80.1325 | 15:45 | 0.25 | 1.5 |
| 4/23/98 | 269 | 32.3858 | -80.2889 | 16:13 | 0.1 | 1.0 |
| 4/24/98 | 293 | 32.6947 | -79.7297 | 14:58 | 0.1 | 1.0 |
| 4/25/98 | 314 | 31.8750 | -79.0367 | 15:38 | 1.0 | 2.0 |
| 4/26/98 | 333 | 32.9297 | -79.2861 | 16:36 | 0.25 | 1.0 |
| 4/27/98 | 357 | 32.9143 | -79.2748 | 16:59 | 0.25 | 1.0 |
| 4/27/98 | 359 | 32.8640 | -79.4731 | 18:49 | 0.25 | 1.0 |

## D.	Sampling Gear

The PRR600s (Serial No. 9643) is a spectroradiometer manufactured by Biospherical Instruments, Inc., which measures seven channels of downwelling irradiance, seven channels of upwelling radiance (Table 2), depth, tilt, roll, and temperature.  A surface unit (PRR610 - Serial No. 9644) is used to measure seven matched channels of surface downwelling irradiance on deck.  Channels 1 to 6 on all sensors and channel 7 on the radiance sensor are narrow band (10 nanometer [nm] full width at half-maximum) centered at the indicated wavelengths, and channel 7 on the irradiance sensor is a broadband detector that measures Photosynthetically Available Radiation (PAR) between 400 and 700 nm (Table 2).

The irradiance and radiance sensors of the PRR600s are separate units, mounted such that the collectors are on the same horizontal plane.  The instrument mount was attached to a tension release on a kevlar reinforced electrical cable.  The PRR610 surface unit was strapped onto a radio antenna on the starboard side of the vessel, close to the davit used to lower the PRR600s (Figure 2).

Table 2.  Center Wavelengths for the PRR System

| Channel No. | PRR600s Downwelling Light Sensor | PRR600s Upwelling Light Sensor | PRR610 |
| --- | --- | --- | --- |
| 1 | 380 nm | 380 nm | 380 nm |
| 2 | 412 nm | 412 nm | 412 nm |
| 3 | 443 nm | 443 nm | 443 nm |
| 4 | 490 nm | 490 nm | 490 nm |
| 5 | 510 nm | 510 nm | 510 nm |
| 6 | 555 nm | 555 nm | 555 nm |
| 7 | PAR | 683 nm | PAR |

## E.	Bottle Samples

The chlorophyll biomass was determined using a Turner Designs fluorometer (Parsons et al. 1984).  The TSS concentration was measured as described by Parsons et al. (1984).  Phytoplankton pigment concentrations were determined by HPLC as described in Tester et al. (1995).  Discrete surface water samples were obtained for these analyses using a bucket, at the same time as the PRR cast.

## F.	Optical Data Processing

The PRR data was processed using the Bermuda Bio-Optics Project (BBOP) processing software (Siegel et al. 1995).  A least common denominator (LCD) file was created from the binary data files, the cast card files, the calibration files, and cruise notes.  The LCD file header contains the metadata for the cast and includes information on the parameters sampled, parameters derived, filters used, and the statistical results of the regression used to extrapolate to the sub-surface.  An example header is presented in Appendix B.  The pressure channel data was recalculated using an offset to adjust for the distance of the pressure sensor from the cosine collector.  The tops and bottoms of the individual profiles were marked using an interactive Matlab script and the corresponding record numbers were inserted into the LCD header section.  Data less than the dark threshold was replaced by -9.9x1035.  The data was then quality controlled using flags for data with tilt and roll angles greater than 10, and records in which the surface incident irradiance was not uniform.  The temperature channel was despiked, in two passes with a difference threshold.  A moving average was calculated for the temperature channel.  The data were separated into upcast and downcast profiles and then binned to 0.5-m bins.  Subsurface downwelling irradiance and upwelling radiance were extrapolated to just below the surface, and spectral attenuation coefficients were calculated for the optical channels over a five-point moving window.

# Results

Results of the study are detailed below.

## Bottle Samples

TSS concentrations ranged from 3.495 to 22.01 mg/l(Table 3).  Chlorophyll a concentrations as measured using HPLC  ranged widely from 0.066 to 3.001 g/l.  Chlorophyll a concentrations tended to be higher in the nearshore areas, and TSS concentrations showed no consistent pattern with distance from shore.

Table 3.  Pigment Analyses

| Station | Latitude | Longitude | TSS mg/L | Total CHL g/l | 19'-But g/l | Fuco g/l | 19'-Hex g/l | Diadin g/l | Zea g/l | Chl a g/l |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 34.6522 | -76.6583 | 7.83 | 0.078 | 0 | 0.025 | 0.004 | 0.005 | 0.003 | 0.042 |
| 25 | 34.6522 | -76.6583 | 12.25 | 2.314 | 0 | 1.044 | 0.033 | 0.180 | 0.040 | 0.523 |
| 35 | 34.5403 | -77.2544 |  | 1.002 | 0 | 0.467 | 0 | 0.080 | 0 | 0.420 |
| 38 | 34.5403 | -77.2544 | 14.11 | 1.035 | 0 | 0.439 | 0 | 0.094 | 0.225 | 0.298 |
| 54 | 33.8478 | -77.4336 | 20.733 | 0.171 | 0.024 | 0.064 | 0.077 | 0.048 | 0 | 0.053 |
| 55 | 33.9970 | -77.5198 | 5.115 | 0.218 | 0.015 | 0.052 | 0.033 | 0.031 | 0.022 | 0.140 |
| 57 | 34.2256 | -77.6444 | 4.575 | 0.161 | 0 | 0.042 | 0.008 | 0.021 | 0 | 0.066 |
| 60 | 34.2961 | -77.6872 | 22.01 | 0.518 | 0 | 0.245 | 0 | 0.067 | 0.017 | 0.236 |
| 82 | 34.2853 | -77.6803 | 11.245 | 0.295 | 0 | 0.120 | 0 | 0.054 | 0 | 0.149 |
| 96 | 33.6233 | -76.3044 | 9.44 | 0.375 | 0.025 | 0.068 | 0.065 | 0.041 | 0 | 0.208 |
| 97 | 33.5639 | -76.4914 | 3.495 | 0.416 | 0.021 | 0.078 | 0.076 | 0.034 | 0.052 | 0.200 |
| 104a | 34.4903 | -77.2233 | 13.32 | 1.617 | 0 | 0.448 | 0 | 0.070 | 0 | 0.161 |
| 104b | 34.2933 | -77.6825 | 7.17 | 1.488 | 0 | 0.727 | 0 | 0.129 | 0 | 0.714 |
| 104c | 33.7100 | -77.3875 | 3.975 | 0.289 | 0.009 | 0.058 | 0.030 | 0.025 | 0 | 0.152 |
| 104d | 33.2306 | -77.7178 | 8.275 | 0.111 | 0 | 0.052 | 0.022 | 0.014 | 0 | 0.049 |
| 117 | 30.8361 | -80.7631 | 15.927 | 0.810 | 0 | 0.289 | 0.039 | 0.057 | 0.036 | 0.190 |
| 144a | 30.9069 | -81.3039 | 10.85 | 1.278 | 0 | 0.696 | 0 | 0.155 | 0.031 | 0.169 |
| 144b | 30.2267 | -81.2900 | 17.047 | 1.898 | 0 | 0.736 | 0.030 | 0.166 | 0.023 | 0.192 |
| 144c | 30.0764 | -81.2467 | 5.78 | 0.926 | 0 | 0.400 | 0.021 | 0.094 | 0.035 | 0.319 |
| 200 | 29.9303 | -81.2628 | 7.513 | 3.001 | 0 | 1.461 | 0.029 | 0.236 | 0.025 | 0.770 |
| 221 | 30.4142 | -80.0019 | 8.18 | 0.088 | 0.006 | 0.042 | 0.010 | 0.010 | 0.038 | 0.054 |
| 234 | 30.4181 | -80.9633 | 8.98 | 0.538 | 0 | 0.242 | 0.012 | 0.061 | 0.022 | 0.195 |
| 235 | 30.4183 | -81.1564 | 12.01 | 0.742 | 0 | 0.305 | 0.019 | 0.085 | 0 | 0.135 |
| 243 | 32.2206 | -80.1325 | 17.787 | 0.487 | 0 | 0.221 | 0 | 0.054 | 0.041 | 0.204 |
| 269 | 32.3858 | -80.2889 | 12.4 | 1.471 | 0.018 | 0.520 | 0.036 | 0.139 | 0.062 | 0.390 |
| 293 | 32.6947 | -79.7297 | 7.08 | 0.938 | 0 | 0.288 | 0 | 0.068 | 0.023 | 0.117 |
| 314 | 31.8750 | -79.0367 | 4.995 | 0.066 | 0.003 | 0.015 | 0.010 | 0.007 | 0 | 0.032 |
| 333 | 32.9297 | -79.2861 | 10.66 | 1.905 | 0 | 0.675 | 0.024 | 0.094 | 0 | 0.252 |
| 357 | 32.9143 | -79.2748 | 15.26 |  |  |  |  |  |  |  |
| 359 | 32.864 | -79.4731 | 6.45 |  |  |  |  |  |  |  |

## B.  Optical data

The optical data were collected in at various locations of the South Atlantic Bight.  The profiles of downwelling irradiance, upwelling radiance, and PAR are shown in Appendix A (Figures A.1.a – A.29.a).  Figures A.1.b – A.29.b show the tilt and roll of the PRR600 and when calculable, the spectral diffuse attenuation.  Comments on the quality of the profile and the depth of measurement nearest to the surface are shown in Table 4.  The above surface downwelling irradiance (Es) should be constant during a profile if there was no change in the light field due to passing clouds etc.  This was tested by calculating the coefficient of variation (standard deviation/mean) for Es. Tilt or roll of greater than 10° do not allow for the robust measurement of downwelling irradiance or upwelling radiance.

Many of the stations off the coasts of South Carolina, Georgia, and Florida exhibited very high attenuation coefficients (exceeding 1 m-1) for the blue wavelengths, particularly 380 and 412 nm.  These wavelengths are affected by the absorption of colored dissolved organic matter (cdom) as well as chlorophyll a and would, if present, affect the pigment estimated from irradiance and radiance measurements.  The stations off the coast of North Carolina did not exhibit these high attenuation coefficients in the blue wavelengths.

Table 4.  Summary of Optical Profiles

| Cast # | Depth of First | Max. Coefficient of | Comments |
| --- | --- | --- | --- |
|  | Measurement (m) | Variation Of Es |  |
| P980405B1 |  |  | Incomplete coverage at surface |
| P980405B2 | 1.38 | 1.8 | Cast ok; increased tilt at bottom |
| P980405C1 | 3.23 | 0.6 | Incomplete coverage at surface |
| P980405C2 | 0.68 | 0.4 | Uniform, good cast |
| P980406A1 | 2.18 | 1.2 | Shadow and cloud effects throughout |
| P980406A2 | 0.75 | 1.1 | Shadow and cloud effects in Ed |
| P980406B1 | 1.78 | 3.2 | Shadow over surface sensor, shadow effects |
| P980406B2 | 1.73 | 53.8 | Shadow over surface sensor; ship shadow effects |
| P980406C1 | 0.79 | 1.1 | Cast ok; some shadow effects; tilt 12 - 21 |
| P980406C2 | 0.55 | 1.0 | Cast ok; some shadow effects |
| P980406F1 | 1.37 | 8.6 | Uniform, good cast |
| P980406F2 | 0.21 | 5.4 | Surface extrapolation may be low |
| P980407A1 | 0.96 | 0.2 | Ship shadow in top 2 m. |
| P980407A2 | 0.38 | 0.2 | Ship shadow in top 5 m |
| P980411A1 | 1.23 | 0.4 | Shadow over surface sensor |
| P980411A2 | 1.81 | 0.5 | Shadow over surface sensor |
| P980412A1 | 2.07 | 0.9 | Shadow effects with noisy cast |
| P980412A2 | 0.60 | 0.5 | Noisy cast with some high angles |
| P980413A1 | 1.67 | 1.4 | Noisy cast with unreliable surface extrapolation. |
| P980413A2 | 0.97 | 1.1 | Shadow over surface sensor |
| P980414A1 | 2.67 | 1.0 | Incomplete coverage near surface; tilt 16-33 |
| P980414A2 | 0.93 | 0.8 | Cast ok |
| P980415A1 | 0.80 | 15.6 | High variability in Eslow sun angle |
| P980415A2 | 0.53 | 8.9 | Slight ship shadow effects near surface |
| P980418A1 | 1.25 | 0.5 | Noisy cast |
| P980418A2 | 1.10 | 0.6 | Invalid Ed(412) near surface |
| P980418B1 | 1.89 | 0.9 | Cast ok |
| P980418B2 | 0.44 | 0.5 | Cast ok |
| P980421A1 | 1.70 | 2.1 | Cast ok; high Ed(380) near surface |
| P980421A2 | 0.52 | 2.7 | Uniform, good cast |
| P980421B1 | 1.65 | 1.1 | Ship shadow effects near surface |
| P980421B2 | 0.30 | 1.2 | Noisy Lu near surface |
| P980422A1 | 1.89 | 1.0 | Cast ok; slight shipshadow effects |
| P980422A2 | 0.96 | 1.0 | Cast ok |
| P980422B1 | 0.62 | 0.9 | Uniform, good cast |
| P980422B2 | 1.20 | 1.5 | Cast ok; some high angles |
| P980423A1 | 1.41 | 31.8 | Shadows some high angless |
| P980423A2 | 0.37 | 0.5 | High angles |
| P980424A1 | 0.83 | 0.8 | Cast ok |
| P980424A2 | 0.30 | 0.4 | Cast ok; low Lu(380) |
| P980426B1 | 1.33 | 0.9 | Uniform, good cast |
| P980426B2 | 0.43 | 0.5 | Cast ok; high tilt |
| P980427A1 | 0.67 | 0.3 | Uniform, good cast |
| P980427A2 | 0.76 | 0.4 | Uniform, good cast |
| P980427B1 | 1.29 | 0.4 | Cast ok; tilt increasing at bottom |
| P980427B2 | 0.36 | 0.3 | Uniform, good cast; tilt increasing at bottom |

## C.  Algorithm Evaluation

The downwelling irradiance and upwelling radiance were extrapolated to the null depth just below the surface (E0-) by the BBOP software.  The downwelling irradiance was propagated through the water-air interface using a transmission loss of 4% (SeaBAM Tech memo).  The upwelling radiance was propagated through water-air interface using a factor of 0.544 (SeaBAM Tech memo).  The above water (E0+) downwelling irradiance and upwelling radiance are shown in Table 5 (Ed+() and Lu+() respectively).  The above water downwelling irradiance measured by the reference sensor mounted on the ship are also shown (Es(). The SeaWiFS OC2 algorithm (O’Reilly et al. 1998) was used to calculate the satellite estimates of chlorophyll a and Coastal Zone Color Scanner (CZCS) pigment concentration.  Values for Es were used in the algorithm unless the coefficient of variation for Es exceeded 10%, then Ed+ was used.  The ratios of satellite-derived to measured quantities for chlorophyll a and CZCS pigment are shown in Table 5.

Table 5  Algorithm Evaluation

| Station | 35 | 38 | 54 | 57 | 60 | 97 | 104 | 124 | 144 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | 4/5/98 | 4/5/98 | 4/6/98 | 4/6/98 | 4/6/98 | 4/11/98 | 4/13/98 | 4/15/98 | 4/18/98 |  |
| Time GMT | 14:09 | 16:11 | 14:41 | 18:16 | 19:24 | 16:08 | 17:06 | 18:09 | 16:54 |  |
| TSS mg/L | -999.99 | 14.11 | 20.73 | 4.575 | 22.01 | 7.17 | 8.26 | 10.85 | 17.05 |  |
| ChlF ug/l | 2.44 | -999.99 | 0.52 | 0.38 | 1.22 | 0.12 | 0.48 | 2.91 | 2.76 |  |
| Chla HPLC |  | 0.36 | 0.05 | 0.066 | 0.24 | 0.71 | 0.05 | 0.17 | 0.19 |  |
| PRR File | p980405b2 | p980405c2 | p980406a2 | p980406c1 | p980406f1 | p980411a1 | p980413a2 | p980415a1 | p980418a1 |  |
| Ed0-380 | 35.326 | 50.581 | 48.764 | 83.637 | 57.429 | 74.780 | 36.727 | 112.760 | 109.350 |  |
| Ed0-412 | 49.859 | 87.433 | 72.040 | 96.999 | 89.301 | 103.040 | 89.209 | 98.940 | 127.649 |  |
| Ed0-443 | 62.286 | 112.206 | 78.447 | 139.205 | 101.649 | 126.660 | 114.926 | 69.033 | 116.538 |  |
| Ed0-490 | 71.968 | 132.956 | 88.371 | 144.311 | 116.804 | 163.507 | 152.161 | 73.535 | 97.908 |  |
| Ed0-510 | 74.983 | 130.606 | 93.233 | 126.072 | 124.721 | 199.574 | 198.387 | 76.665 | 82.262 |  |
| Ed0-555 | 72.357 | 130.786 | 99.110 | 122.998 | 119.780 | 199.007 | 245.976 | 82.979 | 68.821 |  |
| Lu0-380 | 0.151 | 0.379 | 0.135 | 0.183 | 0.308 | 0.341 | 0.113 | 0.022 | 0.129 |  |
| Lu0-412 | 0.430 | 0.906 | 0.247 | 0.361 | 0.782 | 0.788 | 0.220 | 0.379 | 0.342 |  |
| Lu0-443 | 0.695 | 1.496 | 0.345 | 0.596 | 1.312 | 1.279 | 0.389 | 0.422 | 0.475 |  |
| Lu0-490 | 1.204 | 2.626 | 0.494 | 0.967 | 2.126 | 2.163 | 0.567 | 0.916 | 0.716 |  |
| Lu0-510 | 1.335 | 2.888 | 0.479 | 0.971 | 2.177 | 2.364 | 0.657 | 1.102 | 0.813 |  |
| Lu0-555 | 1.419 | 3.144 | 0.344 | 0.814 | 2.141 | 2.561 | 0.541 | 1.498 | 0.955 |  |
| Lu0-683 | 0.292 | 0.657 | 0.040 | 0.088 | 0.346 | 0.526 | 0.088 | 0.692 | 0.308 |  |
| Es0+380 | 38.141 | 74.550 | 55.902 | 70.950 | 58.578 | 72.590 | 78.527 | 68.451 | 77.611 |  |
| Es0+412 | 61.161 | 117.623 | 89.770 | 113.086 | 92.936 | 115.735 | 124.191 | 108.749 | 123.671 |  |
| Es0+443 | 72.344 | 140.506 | 108.187 | 134.093 | 111.446 | 136.694 | 146.684 | 129.304 | 147.084 |  |
| Es0+490 | 78.452 | 153.040 | 117.565 | 143.972 | 120.401 | 147.766 | 157.521 | 140.596 | 159.940 |  |
| Es0+510 | 78.397 | 152.001 | 116.404 | 143.930 | 118.557 | 148.884 | 158.164 | 142.611 | 162.442 |  |
| Es0+555 | 76.616 | 148.510 | 113.359 | 141.309 | 113.915 | 146.756 | 155.134 | 141.178 | 160.688 |  |
| Lwn380 | 0.202 | 0.260 | 0.123 | 0.131 | 0.268 | 0.240 | 0.073 | 0.009 | 0.085 |  |
| Lwn412 | 0.649 | 0.710 | 0.254 | 0.294 | 0.776 | 0.628 | 0.163 | 0.339 | 0.255 |  |
| Lwn443 | 0.982 | 1.089 | 0.326 | 0.454 | 1.204 | 0.957 | 0.272 | 0.601 | 0.330 |  |
| Lwn490 | 1.605 | 1.794 | 0.439 | 0.702 | 1.847 | 1.531 | 0.377 | 1.250 | 0.468 |  |
| Lwn510 | 1.733 | 1.933 | 0.418 | 0.686 | 1.868 | 1.615 | 0.423 | 1.403 | 0.509 |  |
| Lwn555 | 1.855 | 2.119 | 0.304 | 0.577 | 1.881 | 1.747 | 0.349 | 1.735 | 0.595 |  |
| ChlaOC2v2 | 2.9725 | 3.1357 | 0.9018 | 1.327 | 2.191 | 2.884 | 1.751 | 4.702 | 3.764 |  |
| CZCSPigOC2 | 5.2141 | 5.6257 | 1.1939 | 1.843 | 3.439 | 4.998 | 2.580 | 10.313 | 7.344 |  |
| MeasChla | -999.000 | 0.360 | 0.053 | 0.066 | 0.236 | 0.714 | 0.049 | 0.170 | 0.192 |  |
| sat/meas | -999.000 | 8.7104 | 17.0147 | 20.103 | 9.284 | 4.040 | 35.744 | 27.742 | 19.604 |  |
| MeasPig | 2.435 | -999.000 | 0.515 | 0.382 | 1.218 | 2.729 | 0.482 | 2.910 | 2.760 |  |
| sat/meas | 1.2207 | -999.000 | 1.7510 | 3.473 | 1.799 | 1.057 | 3.634 | 1.616 | 2.661 |  |

| Station | 144 |  |  | 234 |  |  | 235 |  |  | 247 |  |  | 293 |  |  | 333 |  |  | 357 |  |  | 359 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | 4/18/98 |  |  | 4/21/98 |  |  | 4/21/98 |  |  | 4/22/98 |  |  | 4/24/98 |  |  | 4/26/98 |  |  | 4/27/98 |  |  | 4/27/98 |  |  |  |  |  |
| Time GMT | 18:19 |  |  | 14:42 |  |  | 15:56 |  |  | 17:29 |  |  | 14:58 |  |  | 16:36 |  |  | 16:59 |  |  | 18:49 |  |  |  |  |  |
| TSS mg/L | 5.78 |  |  | 8.98 |  |  | 12.01 |  |  | 9.092 |  |  | 7.08 |  |  | 10.66 |  |  | 15.26 |  |  | 6.450 |  |  |  |  |  |
| ChlF ug/l | 1.64 |  |  | 1.03 |  |  | 1.27 |  |  | 1.867 |  |  | 1.33 |  |  | 2.91 |  |  | 1.96 |  |  | 1.515 |  |  |  |  |  |
| Chla HPLC | 0.32 |  |  | 0.20 |  |  | 0.14 |  |  | 0.361 |  |  | 0.12 |  |  | 0.25 |  |  |  |  |  | -999 |  |  |  |  |  |
| PRR File | p980418b1 |  |  | p980421a1 |  |  | p980421b1 |  |  | p980422b1 |  |  | p980424a2 |  |  | p980426b1 |  |  | p980427a1 |  |  | p980427b1 |  |  |  |  |  |
| Ed0-380 |  | 51.019 |  |  | 181.752 |  |  |  | 87.078 |  |  | 61.636 |  |  | 23.564 |  | 60.497 |  |  | 103.693 |  |  | 62.495 |  |  |  |  |
| Ed0-412 |  | 94.363 |  |  | 125.432 |  |  |  | 77.677 |  |  | 96.541 |  |  | 57.894 |  | 125.654 |  |  | 117.195 |  |  | 102.600 |  |  |  |  |
| Ed0-443 |  | 141.398 |  |  | 121.198 |  |  |  | 77.890 |  |  | 113.716 |  |  | 70.322 |  | 141.351 |  |  | 121.045 |  |  | 120.903 |  |  |  |  |
| Ed0-490 |  | 176.193 |  |  | 126.968 |  |  |  | 80.020 |  |  | 131.127 |  |  | 102.247 |  | 150.101 |  |  | 126.214 |  |  | 129.635 |  |  |  |  |
| Ed0-510 |  | 201.182 |  |  | 129.232 |  |  |  | 87.976 |  |  | 146.817 |  |  | 102.764 |  | 146.041 |  |  | 136.177 |  |  | 127.303 |  |  |  |  |
| Ed0-555 |  | 197.786 |  |  | 130.643 |  |  |  | 89.214 |  |  | 144.101 |  |  | 99.958 |  | 144.605 |  |  | 129.853 |  |  | 125.432 |  |  |  |  |
| Lu0-380 |  | 0.039 |  |  | 0.026 |  |  |  | 0.029 |  |  | 0.056 |  |  | 0.003 |  | 0.051 |  |  | 0.148 |  |  | 0.051 |  |  |  |  |
| Lu0-412 |  | 0.240 |  |  | 0.143 |  |  |  | 0.185 |  |  | 0.409 |  |  | 0.114 |  | 0.185 |  |  | 0.350 |  |  | 0.280 |  |  |  |  |
| Lu0-443 |  | 0.411 |  |  | 0.184 |  |  |  | 0.259 |  |  | 0.713 |  |  | 0.307 |  | 0.352 |  |  | 0.554 |  |  | 0.473 |  |  |  |  |
| Lu0-490 |  | 0.768 |  |  | 0.250 |  |  |  | 0.357 |  |  | 1.419 |  |  | 0.891 |  | 0.688 |  |  | 0.986 |  |  | 0.817 |  |  |  |  |
| Lu0-510 |  | 0.913 |  |  | 0.273 |  |  |  | 0.405 |  |  | 1.729 |  |  | 1.067 |  | 0.825 |  |  | 1.127 |  |  | 0.933 |  |  |  |  |
| Lu0-555 |  | 1.031 |  |  | 0.284 |  |  |  | 0.467 |  |  | 2.256 |  |  | 1.364 |  | 1.022 |  |  | 1.299 |  |  | 1.079 |  |  |  |  |
| Lu0-683 |  | 0.200 |  |  | 0.087 |  |  |  | 0.134 |  |  | 0.748 |  |  | 0.343 |  | 0.264 |  |  | 0.351 |  |  | 0.264 |  |  |  |  |
| Es0+380 |  | 69.999 |  |  | 57.788 |  |  |  | 72.531 |  |  | 79.201 |  |  | 61.026 |  | 75.966 |  |  | 75.407 |  |  | 69.403 |  |  |  |  |
| Es0+412 |  | 110.477 |  |  | 95.123 |  |  |  | 116.407 |  |  | 125.238 |  |  | 97.650 |  | 121.183 |  |  | 120.243 |  |  | 112.057 |  |  |  |  |
| Es0+443 |  | 131.472 |  |  | 113.424 |  |  |  | 137.858 |  |  | 147.975 |  |  | 117.114 |  | 143.796 |  |  | 142.860 |  |  | 133.741 |  |  |  |  |
| Es0+490 |  | 141.218 |  |  | 123.235 |  |  |  | 148.513 |  |  | 158.768 |  |  | 126.692 |  | 155.418 |  |  | 154.503 |  |  | 145.797 |  |  |  |  |
| Es0+510 |  | 141.425 |  |  | 125.207 |  |  |  | 150.019 |  |  | 159.550 |  |  | 125.904 |  | 157.215 |  |  | 156.178 |  |  | 148.338 |  |  |  |  |
| Es0+555 |  | 138.296 |  |  | 124.331 |  |  |  | 148.202 |  |  | 156.650 |  |  | 122.860 |  | 155.062 |  |  | 154.285 |  |  | 147.139 |  |  |  |  |
| Lwn380 |  | 0.029 |  |  | 0.023 |  |  |  | 0.020 |  |  | 0.036 |  |  | 0.003 |  | 0.034 |  |  | 0.100 |  |  | 0.037 |  |  |  |  |
| Lwn412 |  | 0.201 |  |  | 0.138 |  |  |  | 0.147 |  |  | 0.301 |  |  | 0.108 |  | 0.141 |  |  | 0.268 |  |  | 0.230 |  |  |  |  |
| Lwn443 |  | 0.320 |  |  | 0.166 |  |  |  | 0.193 |  |  | 0.493 |  |  | 0.268 |  | 0.250 |  |  | 0.397 |  |  | 0.362 |  |  |  |  |
| Lwn490 |  | 0.569 |  |  | 0.212 |  |  |  | 0.251 |  |  | 0.935 |  |  | 0.736 |  | 0.463 |  |  | 0.667 |  |  | 0.586 |  |  |  |  |
| Lwn510 |  | 0.657 |  |  | 0.222 |  |  |  | 0.275 |  |  | 1.102 |  |  | 0.862 |  | 0.534 |  |  | 0.734 |  |  | 0.640 |  |  |  |  |
| Lwn555 |  | 0.746 |  |  | 0.228 |  |  |  | 0.316 |  |  | 1.442 |  |  | 1.111 |  | 0.660 |  |  | 0.843 |  |  | 0.734 |  |  |  |  |
| ChlaOC2v2 |  |  | 6.420 |  |  | 2.496 |  | 3.657 |  |  | 6.190 |  |  | 5.858 |  |  |  | 5.041 |  |  | 3.705 |  |  | 3.624 |  |  |  |
| CZCSPigOC2 |  |  | 4.080 |  |  | 4.093 |  | 7.036 |  |  | 16.049 |  |  | 14.659 |  |  |  | 11.505 |  |  | 7.173 |  |  | 6.942 |  |  |  |
| MeasChla |  |  | 0.319 |  |  | 0.195 |  | 0.135 |  |  | 0.361 |  |  | 0.117 |  |  |  | 0.252 |  |  | -999.000 |  |  | -999.000 |  |  |  |
| sat/meas |  |  | 12.790 |  |  | 12.798 |  | 27.089 |  |  | 17.145 |  |  | 50.070 |  |  |  | 20.002 |  |  | -999.000 |  |  | -999.000 |  |  |  |
| MeasPig |  |  | 1.643 |  |  | 1.029 |  | 1.267 |  |  | 1.867 |  |  | 1.328 |  |  |  | 2.910 |  |  | 1.958 |  |  | 1.515 |  |  |  |
| sat/meas |  |  | 5.045 |  |  | 3.977 |  | 2.886 |  |  | 3.315 |  |  | 4.411 |  |  |  | 1.732 |  |  | 1.892 |  |  | 2.392 |  |  |  |

# V.  Summary

Chlorophyll a and TSS concentrations in waters of the South Atlantic Bight during this cruise in April were highly variable.  Chlorophyll a concentrations tended to be higher near shore, and TSS showed no discernible geographic pattern, with high and low concentrations being measured at both nearshore and offshore stations.

The OC2 version 2 algorithm over-estimated the chlorophyll a concentration by a factor of 4 and as much as a factor of 50.  CZCS pigment concentration is estimated more successfully with ratios of measured to estimated pigment ranging from 1 to 5. The overestimates may be due, in large part, to high concentrations of colored dissolved organic matter (cdom).  High diffuse attenuation coefficients at 380 and 412 nm were measured; these wavelengths are affected by cdom absorption.  The OC2 algorithm needs refinement in order to estimate chlorophyll a in coastal waters of the South Atlantic Bight.

# VI.  References

O'Reilly, J. E., S. Maritorena, B. G. Mitchell, D. A. Siegel, K. L. Carder, S. A. Garver, M. Kahru, and C. McClain. (1998).  Ocean color chlorophyll algorithms for SeaWiFS. J. Geophys. Res. 103(C11): 24937-24953.

Parsons, T. R., Y. Maita and C. M. Lalli (1984). A Manual For Chemical And Biological Methods For Seawater Analysis, Pergamon Press.

Siegel, D. A., M. C. O'Brien, J. C. Sorensen, D. A. Konnoff and E. Fields (1995). BBOP Data Processing and Sampling Procedures. Vol: 19, Institute for Computational Earth System Science, UC Santa Barbara, Santa Barbara, CA,  23 pp.

Smith, R. C., K. S. Baker and P. Dustan (1981). Fluorometric Techniques for the Measurement of Oceanic Chlorophyll in the Support of Remote Sensing. SIO Ref. 81-17, Visibility Laboratory, Scripps Institution of Oceanography, La Jolla, CA 92093,  14 pp.

Tester, P.A., M.E. Geesey, C. Guo. H.W. Paerl and D.F. Millie (1995) Evaluating phytoplankton dynamics in the Newport River estuary (North Carolina, USA) by HPLC-derived pigment profiles. Mar. Ecol. Prog. Ser. 124:237-245.

# VII.	Metadata

Identification Information:

Citation:

Citation Information:

Originator:  Ajit Subramaniam, REMSA, NOAA Coastal Services Center

Originator:  David Eslinger, NOAA Coastal Services Center

Originator:  Patricia A. Tester, NOAA Center for Coastal Fisheries Habitat Research

Originator:  Elin Haugen, NOAA Center for Coastal Fisheries Habitat Research

Originator:  Richard P. Stumpf, NOAA National Center for Coastal Ocean Science

Originator:  John C. Brock, USGS Center for Coastal Geology

Originator:  Mary E. Culver, TPMC, NOAA Coastal Services Center

Originator:  Ruth W. Kiambo, SeaGrant, NOAA Coastal Services Center

Publication Date: January 1999

Title: NOAA CSC/CRS Cruise APR98SAB: South Atlantic Bight Cruise

Geospatial Data Presentation Form:  profile

Series Information:

Series Name:  CSC Technical Report

Issue Identification: CSC/1-99/001

Publication Information:

Publication Place:  Charleston, South Carolina

Publisher:  NOAA Coastal Services Center

Online Linkage:  http://www.csc.noaa.gov/crs/cruises/apr98sab/index.html

Description:

Abstract: See page iii.

Purpose:  See Objective on page 1.

Time Period of Content:

Time Period Information:

Range of Dates/Times:

Beginning Date:  19980403

Ending Date:  19980427

Currentness Reference: Publication Date

Status:

Progress:  Complete

Maintenance and Update Frequency:  Unknown

Spatial Domain:

Bounding Coordinates:

West Bounding Coordinate:  -81.3039

East Bounding Coordinate:  -76.3044

North Bounding Coordinate:  34.6522

South Bounding Coordinate:  29.9303

Keywords:

Theme:

Theme Keyword Thesaurus:  None

Theme Keyword:  oceanography

Theme Keyword:  bio-optical

Theme Keyword:  turbidity

Theme Keyword:  water clarity

Theme Keyword:  algal blooms

Theme Keyword:  coastal water optics

Theme Keyword:  case II algorithms

Theme Keyword:  light attenuation

Theme Keyword:  in-situ optical profiling

Theme Keyword:  ocean color satellites

Theme Keyword:  coastal ocean algorithm development

Theme Keyword:  downwelling irradiance

Theme Keyword:  upwelling radiance

Theme Keyword:  temperature

Theme Keyword:  chlorophyll

Theme Keyword:  spectral attenuation

Place:

Place Keyword Thesaurus:  None

Place Keyword:  North Carolina

Place Keyword:  South Carolina

Place Keyword:  Georgia

Place Keyword:  Florida

Place Keyword:  South Atlantic Bight

Place Keyword:  United States

Temporal:

Temporal Keyword Thesaurus: None

Temporal Keyword:  Spring

Temporal Keyword:  April, 1998

Access Constraints:  None

Use Constraints: This data was acquired for scientific research and is applicable for algorithm validation purposes.  Knowledge of in-water optics is expected of users for interpretation of the data.  Users of this data are required to provide appropriate attribution in the form of co-authorship for any publications that use this data, unless formal permission to do otherwise is granted by NOAA/CSC.

Point of Contact:

Contact Information:

Contact Organization Primary:

Contact Organization: NOAA Coastal Services Center

Contact Address:

Address Type:  mailing and physical address

Address:  2234 S. Hobson Avenue

City:  Charleston

State or Province:  South Carolina

Postal Code:  29405-2413

Country:  USA

Contact Voice Telephone:  (843) 740-1200

Contact Facsimile Telephone:  (843) 740-1224

Contact Electronic Mail Address:  csc@csc.noaa.gov

Hours of Service:  8AM-5PM, M-F

Data Set Credit:  Master and crew of R/V Cape Hatteras.  Sample processing was conducted by Elin Haugen at the NOAA Center for Coastal Fisheries Habitat Research in Beaufort, NC.  A NOAA Coastal Ocean Program grant to Dr. Patricia Tester and Dr. Richard Stumpf made this cruise possible.

Data Quality Information:

Attribute Accuracy:

Attribute Accuracy Report:  Refer to the Process Step section for specific calibration information.  The primary instrumentation on the cruise are sent to the respective manufacturers for calibration at least once per year. Calibration certificates for the relevant instrumentation are available in the full written report.  Secondary instrumentation was calibrated only upon purchase.  Laboratory calibrations of the Turner Designs fluorometer and the HPLC are conducted as needed using known concentrations of purified photosynthetic pigment extracts (measured using a spectophotometer) purchased commercially or isolated from algal cultures.

Logical Consistency Report:  The PRR data was processed using the Bermuda Bio-Optics Project (BBOP) processing software.  A least common denominator (LCD) file was created from the binary data files, the cast card files, the calibration files, and cruise notes.  The LCD file header contains the metadata for the cast and includes information on the parameters sampled, parameters derived, filters used, and the statistical results of the regression used to extrapolate light to the sub-surface.  An example header is presented in Appendix C.  The pressure channel data was recalculated using an offset to adjust for the distance of the pressure sensor from the cosine collector.  The tops and bottoms of the individual profiles were marked using an interactive Matlab script and the corresponding record numbers were inserted into the LCD header section.  Data less than the dark threshold was replaced by -9.9x1035.  Then the data was quality controlled using flags for data with tilt and roll angles greater than 10 (flag value greater than 0 in the “aq-1Tilt-1Roll” field), and records where the surface incident irradiance was not uniform (flag value greater than 0 in the “kq-1ed412” field).  The temperature, transmissometer, and fluorometer data were despiked, in two passes, with a difference threshold.  A moving average was calculated for these channels.  The data were separated into upcast and downcast profiles and then binned to 0.5-m bins.  Spectral attenuation coefficients were calculated for the optical channels over a five point moving window.  Subsurface downwelling irradiance and upwelling radiance were extrapolated to just below the surface using data from the top 3 meters.  The statistics for calculation of subsurface irradiance and radiance are shown in Appendix B.

Completeness Report: Refer to the separate sections of Logical Consistency, Methodology, and Process Steps for descriptions of  completeness of the data.

Lineage:

Methodology:

Methodology Type:  Shipboard Deployments and data collection

Methodology Identifier:

Methodology Keyword Thesaurus:  None

Methodology Keyword:  bio-optical data

Methodology Keyword:  depth profiles

Methodology Keyword:  spectral downwelling irradiance measurement

Methodology Keyword:  spectral upwelling radiance

Methodology Keyword:  temperature measurement

Methodology Keyword:  bottle sampling

Methodology Keyword:  CTD profiles

Methodology Keyword:  water sampling

Methodology Keyword:  Niskin bottles

Methodology Description: The Profiling Reflectance Radiometer (PRR) cage was deployed off the stern of the boat, to measure in-situ spectral downwelling irradiance, spectral upwelling radiance, temperature, chlorophyll fluorescence, light scattering, quantum scalar irradiance, and beam attenuation.  Following the PRR cast in-situ temperature, salinity, and density were measured with a Conductivity-Temperature-Depth (CTD) instrument.  Water samples for chlorophyll biomass, HPLC-determined photosynthetic pigments, and total suspended solids were obtained from a separate cast.

Methodology Type:  Lab calibration of fluorometer and analysis of chlorophyll extracts.

Methodology Identifier:

Methodology Keyword Thesaurus:  None

Methodology Keyword:  chlorophyll

Methodology Keyword:  fluorescence

Methodology Keyword:  fluorometer

Methodology Keyword:  extraction

Methodology Description:  The concentration of purified chlorophyll a, dissolved in 90% acetone (10% water), was measured using a spectrophotometer and used to calibrate the Turner Designs Model 10-AU fluorometer.  Aboard ship, a measured volume of seawater was filtered onto a Whatman GF/F filter and stored in liquid nitrogen until lab analysis.  In the lab the filter was ground and extracted in 10 ml of 90% acetone and left in a freezer (-20 C) overnight.  After centrifugation, the chlorophyll concentration in the supernatant was measured using the fluorometer.

Methodology Citation:

Citation Information:

Originator: T.R. Parsons

Originator:  Y. Maita

Originator:  C.M. Lalli

Publication Date:  1984

Title:  A manual for Chemical and Biological Methods for Seawater Analysis

Publication Information:

Publication Place:  New York, New York, USA

Publisher:  Pergamon Press

Methodology Type:  HPLC calibration and analysis of phytoplankton pigments.

Methodology Identifier:

Methodology Keyword Thesaurus:  None

Methodology Keyword:  photosynthetic pigments

Methodology Keyword:  high performance liquid chromatography

Methodology Keyword:  chlorophylls

Methodology Keyword:  carotenoids

Methodology Keyword:  chemotaxonomy

Methodology Description:  The concentration of purified photosynthetic pigments were measured using a spectrophotometer.  In a series of runs, known amounts (generally 1 to 100 ng) were injected into the HPLC to establish the retention time and define a calibration curve for each pigment.  Aboard ship, a measured volume of seawater was filtered onto a Whatman GF/F filter that was stored in liquid nitrogen.  In the lab the filter was ground and extracted with 1.0 ml of 90% acetone, placed in the freezer overnight to extract.  After centrifugation, 0.2 ml was injected into the HPLC to quantify the pigments present.

Methodology Citation:

Citation Information:

Originator:  S.W. Wright

Originator:  S.W. Jeffrey

Originator:  R.F.C. Mantoura

Originator:  C.A. Llewellyn

Originator:  T. Bjornland

Originator:  D. Repeta

Originator:  N. Welschmeyer

Publication Date:  1991

Title:  Improved HPLC method for the analysis of chlorophylls and carotenoids from marine phytoplankton.

Series Information:

Series Name:  Marine Ecology Progress Series

Issue Identification:  Volume 77: 183-196.

Methodology Type: Measurement of total suspended solids

Methodology Identifier:

Methodology Keyword Thesaurus:  None

Methodology Keyword:  total suspended solids

Methodology Keyword:  suspended particulate matter

Methodology Description:  A measured volume of seawater was filtered through a dried and pre-weighed Whatman GF/F filter.  The filter was subsequently dried at 60C and re-weighed.

Methodology Citation:

Citation Information:

Originator: T.R. Parsons

Originator:  Y. Maita

Originator:  C.M. Lalli

Publication Date:  1984

Title:  A manual for Chemical and Biological Methods for Seawater Analysis

Publication Information:

Publication Place:  New York, New York, USA

Publisher:  Pergamon Press

Process Step:

Process Description:  Calibration of the Biospherical PRV-600s PRR Spectroradiometer.

Process Date:  19980108

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  Biospherical Instruments, Inc.

Contact Address:

Address Type:  mailing and physical address

Address:  5340 Riley Street

City:  San Diego

State or Province:  California

Postal Code:  92110-2621

Country:  USA

Contact Voice Telephone:  (619) 686-1888

Process Step:

Process Description:  Calibration of the Biospherical PRV610 PRR Spectroradiometer.

Process Date:  19980108

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  Biospherical Instruments, Inc.

Contact Address:

Address Type:  mailing and physical address

Address:  5340 Riley Street

City:  San Diego

State or Province:  California

Postal Code:  92110-2621

Country:  USA

Contact Voice Telephone:  (619) 686-1888

Process Step:

Process Description:  Calibration of Satlantic SAS II MVD 7-channel Radiance Sensor system.

Process Date: 19980122

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  Satlantic, Inc.

Contact Address:

Address Type:  mailing and physical address

Address:  3295 Barrington Street

City:  Halifax

State or Province:  Nova Scotia

Postal Code:  B3K 5X8

Country:  Canada

Contact Voice Telephone:  (902) 492-4780

Spatial Data Organization Information:

Indirect Spatial Reference:  South Atlantic Bight, North Carolina, South Carolina, Georgia, Florida, USA.

Distribution Information:

Distributor:

Contact Information:

Contact Organization Primary:

Contact Organization: NOAA Coastal Services Center

Contact Address:

Address Type:  mailing and physical address

Address:  2234 S. Hobson Avenue

City:  Charleston

State or Province:  South Carolina

Postal Code:  29405-2413

Country:  USA

Contact Voice Telephone:  (843) 740-1200

Contact Facsimile Telephone:  (843) 740-1224

Contact Electronic Mail Address:  csc@csc.noaa.gov

Hours of Service:  8AM-5PM, M-F

Resource Description:  APR98SAB Cruise Report

Distribution Liability:  None

Custom Order Process:  The data can be accessed on-line at http://www.csc.noaa.gov/crs/cruises/.

Metadata Reference Information:

Metadata Date:  19981026

Metadata Review Date:  19981026

Metadata Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  NOAA, Coastal Services Center

Contact Position:  Metadata Specialist

Contact Address:

Address Type:  mailing and physical address

Address:  2234 Hobson Avenue

City:  Charleston

State or Province:  South Carolina

Postal Code:  29405-2413

Country:  USA

Contact Voice Telephone:  (843) 740-1200

Contact Facsimile Telephone:  (843) 740-1224

Contact Electronic Mail Address:  csc@csc.noaa.gov

Hours of Service:  8AM-5PM, M-F

Metadata Standard Name:  Content Standard for National Biological Information Infrastructure Metadata.

Metadata Standard Version:  December 1995

# VIII.  Appendix A - Water Column Profile Data Figures

# IX.  Appendix B - Example Profile Header information

The following information is found as a header on all BBOP processed files.

<cruise_info>

filename p980405b

date 04-05-1998

day_of_year 95

file_created 09:09:03

cruise

position 77 15.262 34 32.387

longitude 77 15.262

latitude 34 32.387

sky_state clouds

operator_name hw

sun_position 2

cruise_id  ch16-97

session_started 09:09:09

session_stopped 09:10:17

depth_offset 0.32

most_recent_dark_file

deck_comparison_file

cal_date_uw9643 010798

cal_date_sfc9644 010798

affiliations NOAA_Coastal_Services_Center

investigators John_Brock, Pat_Tester

contact csc@csc.noaa.gov

calibration_files unit0_010798.cfl/unit1_010798.cfl/unit2_010798.cfl

downcast_ended 09:09:20.121 16

upcast_ended 09:10:05.890 89

collection_software_version prrprof_002086c

number_units 1

collection_cal_file 96439644.cfl;prr-600 #9643/9644 calibration file 2/10/97 cac

lcd_calib_file 0 /csc/nac1/crs/cruisedata/calib_prr/unit0_010798.cfl

1 /csc/nac1/crs/cruisedata/calib_prr/unit1_010798.cfl

2 /csc/nac1/crs/cruisedata/calib_prr/unit2_010798.cfl

lcdfile_created Jul 24 1998 11:11:41

<sampled_parameters>

# X.	Appendix C - Calibration Certificates

The following pages contain the calibration history of the PRR600 instrument.