CSC Technical Report: NOAA CSC 99043-PUB

NOAA Cruise JUL98NAN:

Nantucket Shoals Cruise

Ajit Subramaniam1

Mary Culver2

David Phinney3

Douglas Phinney3

Jeffery Brown3

Brian Schieber4

John Brock5

David Eslinger2

Christopher Brown6

1REMSA, Inc.

E/RA3, WWBG, Room 711b

5200 Auth Road

Camp Springs, MD  20746-4304

2NOAA Coastal Services Center

2234 South Hobson Avenue

Charleston, SC 29405

3Bigelow Laboratory for Ocean Sciences

180 McKown Point Road

West Boothbay Harbor, ME 04575

4SAIC General Sciences Corp.

SeaWiFS Project

NASA Goddard Space Flight Center

Code 970.2

Greenbelt, MD 20771

5Center for Coastal Geology

U.S. Geological Survey

600 4th St.

St. Petersburg, FL  33701

6NOAA NESDIS

E/RA3, WWBG, Room 105

5200 Auth Road

Camp Springs, MD  20746-4304

May 1999

UNITED STATES DEPARTMENT OF COMMERCE

National Oceanic and Atmospheric Administration

COASTAL SERVICES CENTER

#### 2234 South Hobson Avenue, Charleston, SC 29405-2413

Abstract

The July 1998 Nantucket Shoals cruise (JUL98NAN) was a cooperative effort among the National Oceanic and Atmospheric Administration's Coastal Service Center's Coastal Remote Sensing Program (NOAA/CSC/CRS), the Bigelow Laboratory for Ocean Sciences and the Sea-viewing Wide Field-of-view Sensor (SeaWiFS) Project Office at National Aeronautics and Space Administration (NASA) Goddard Space Flight Center.  Seventeen stations were occupied over five days in Massachusetts Bay, Buzzard’s Bay, Nantucket Sound, and Nantucket Shoals.

The standard NASA chlorophyll algorithm consistently overestimates the chlorophyll concentration in Massachusetts Bay by at least a factor of three, while it appears to be reasonably accurate in the Nantucket Shoals region.  A regional algorithm needs to be developed for the Massachusetts Bay in order to make SeaWiFS data useful to the coastal managers administering a water body with high anthropogenic impact.  There is excellent radiometric agreement at station D2 for the 490, 510, and 555 nm bands, indicating that it is the chlorophyll algorithm and not the atmospheric correction algorithm that is affecting the accuracy of the chlorophyll estimates.

Table of Contents

I.  Introduction	1

II.  Objectives	1

III.  Methods	1

A.  Sampling Location	1

B.  Sampling Platform	2

C.  Sample Collection Methods Summary	3

D.  Sampling Gear	7

E.  Bottle Samples	7

F.  Optical Data Processing	7

IV.  Results	8

A.  Bottle Samples	8

B.  Optical Data	11

C.  Algorithm Evaluation	11

D.  Satellite Data	15

V.  Summary	17

VI.  References	17

VII.  Metadata	18

VIII.  Appendix A - Water Column Profile Data Figures	25

IX.  Appendix B - Example Header Information	37

X.  Appendix C - Calibration Certificates	40

List of Figures

Figure 1.  Location of Stations.	2

Figure 2.  The R/V Gulf Challenger.	3

Figure 3.  Deployment of the SMSR.	4

Figure 4.  Optical Cage with the SPMR.	4

Figure 5.  Remote Sensing Reflectance Spectra.	11

Figure 6  SeaWiFS Chlorophyll Data for July 9, 1998 with Stations D1-D4 Marked.	15

Figure 7.  Chlorophyll Product Determined Three Different Ways.	16

Figure 8.  Satellite Derived and In-situ Measured Normalized Water Leaving Radiance for July 9, 1998.	16

List of Tables

Table 1.  Station Locations.	5

Table 2.  Center Wavelengths for the SPMR System.	7

Table 3.  Fluorometric Pigment Analyses.	8

Table 4.  HPLC Pigment Abbreviations.	10

Table 5.  HPLC Pigment Analysis Results.	10

Table 6.  Algorithm Evaluation.	12

Acknowledgments

We thank the Captain Paul Pelletier and the crew of the R/V Gulf Challenger for assistance provided.  A NASA SIMBIOS (Sensor Intercomparison and Merger for Biological and Interdisciplinary Oceanic Studies) contract to Drs. Christopher Brown and John Brock as well as a NOAA Coastal Ocean Program grant to Dr. Charles Yentsch and David Phinney contributed to this cruise.

# I.  Introduction

Monitoring the health of U.S. coastal waters is an important goal of the National Oceanic and Atmospheric Administration (NOAA).  Satellite ocean color sensors are capable of providing regular synoptic water quality data for the U.S. coast.  Algorithms are used to derive products such as chlorophyll biomass from satellite data in order to study short and long term changes in water quality; however, these algorithms need to be evaluated and validated.  Satellite ocean color imagery from the Ocean Color Temperature Sensor (OCTS) and the Sea-viewing Wide Field-of-view Sensor (SeaWiFS) show extremely high chlorophyll concentrations within Massachusetts Bay, and relatively high chlorophyll concentrations over Nantucket Shoals.  It is uncertain how much of this chlorophyll signal detected by the satellite sensors was actually due to phytoplankton biomass and how much was due to other substances in the water such as colored dissolved organic matter and suspended sediments.  Knowledge of the validity of the satellite data in these regions is imperative to better understand the true productivity of the Nantucket Shoals region and the impact that the highly urbanized coast around Boston has on the water quality in Massachusetts Bay.

Towards this purpose, scientists from the NOAA Coastal Services Center (CSC) Coastal Remote Sensing (CRS) program, the Bigelow Laboratory for Ocean Sciences and the SeaWiFS Project Office at the National Aeronautics and Space Administration (NASA) Goddard Space Flight Center undertook a five-day cruise in Massachusetts Bay, Nantucket Sound, and Nantucket Shoals.  This program is funded in part through a NASA Sensor Intercomparison and Merger for Biological and Interdisciplinary Oceanic Studies (SIMBIOS) contract awarded to CRS and the Ocean Color Program at the NOAA National Environmental Satellite, Data and Information Service (NESDIS) Office of Research and Applications.

# II.  Objectives

The objectives of this cruise were to obtain sub-surface downwelling irradiance, upwelling radiance, chlorophyll pigment concentration and total suspended sediment concentration in Massachusetts Bay and Nantucket Shoals.  The remote sensing reflectance measurements calculated from these samples were used to evaluate and validate the OC2 version 2 (O’Reilly et al. 1998) algorithm for the NASA/OrbImage ocean color sensor, SeaWiFS.

# III.  Methods

A description of the sample collection methods and instruments is detailed in the following sections.

## A.  Sampling Location

Optical profile measurements were made from July 6, 1998 through July 10, 1998, at a total of 17 stations (Figure 1.).  Surface water samples were acquired at these locations for determination of particulate absorption, total suspended solids (TSS) concentration and for chlorophyll analysis by fluorometric and High-Performance Liquid Chromatography (HPLC) techniques.

Figure 1.  Location of Stations.

## B.  Sampling Platform

The University of New Hampshire’s R/V Gulf Challenger, a 50-foot aluminum hull, twin engine ship, was used on this cruise (Figure 2).  This boat is capable of travelling at up to 20 knots and thus can be used to sample multiple stations for a single SeaWiFS pass.  The SeaWiFS Project office criteria specify that stations should be more than 10 km apart and sampling should be completed within two hours of overpass.  This vessel has a draft of 5 feet, and thus was ideal for working in the shallower waters of the Nantucket Shoals.  The vessel has a stern “A-frame” which was used to deploy the optical cage.  The A-frame put the cage at least 1 m away from the stern of the boat, minimizing ship shadow effects on the SPMR (SeaWiFS Profiling Multichannel Radiometer).

Figure 2.  The R/V Gulf Challenger.

## C.  Sample Collection Methods Summary

A Satlantic, Inc. SeaWiFS SPMR and a SeaWiFS multi-channel surface radiometer (SMSR) were used on this cruise.  The SMSR is a surface tethered buoy that measures above-water spectral downwelling irradiance and in-situ spectral upwelling radiance 75 cm below the surface (Figure 3.).  It was floated away from the side of the vessel to avoid ship shadow artifacts in the calculation of water leaving radiance, and data was collected for 5 to 10 minutes.  The SPMR is normally used as a free-falling device that measures downwelling irradiance and upwelling radiance, temperature, and salinity.  On this cruise, it was fixed inside the optics cage because the water depth may have been inadequate for a free falling instrument. The optics cage contained a Wetlabs AC-9 to measure spectral absorption at nine wavelengths, a Hobilabs Hydroscat to measure spectral backscatter at 6 wavelengths, and a SeaBird CTD to measure in-situ temperature, salinity, and density (Figure 4).  The AC-9 was rigged with a valve and a 0.2 m filter cartridge such that it measured total absorption on the downcast and dissolved absorption on the upcast.  Niskin bottles were used to obtain water samples from three to five depths at each station for particulate absorption, total suspended solids concentration, and pigment analyses.

Figure 3.  Deployment of the SMSR.

Figure 4.  Optical Cage with the SPMR.

Table 1.  Station Locations.

| Station | Date | Time On | Time Off | Time GMT | Latitude | Longitude | Total Depth (m) | Secchi Depth | Sky State | Sea State | Wind speed | Measurements | Bottle Depths(m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 06-Jul-98 | 12:20 | 13:03 | 16:30 | 42.895 | -70.373 | 102.0 | 7.0 | 90% Haze, cirrus clouds | flat calm | 0-3 knts | 1,2,3,5,6,7,8 | 0,10,20,30,50 |
| A2 | 06-Jul-98 | 14:20 | 15:08 | 18:30 | 42.551 | -70.448 | 106.0 | 8.0 | high cirrus, with some clear patches | flat calm | 0-3 knts | 1,2,3,5,6,7,8,10,11 | 0,10,20,30,50 |
| A3 | 06-Jul-98 | 16:20 | 16:50 | 20:30 | 42.366 | -70.779 | 26.4 | 7.5 | 80% Haze, cirrus clouds | flat calm | 0-3 knts | 1,2,3,5,6,7,8 | 0,10,20 |
| B1 | 07-Jul-98 | 9:25 | 10:22 | 13:35 | 42.170 | -70.575 | 33.4 | 11.0 | clear overhead, 40 %cirrus and thick clouds in horizon | flat calm | 5 knts | 1,2,3,4,5,6,7,8,10,11 | 0,10,20,30 |
| B2 | 07-Jul-98 | 11:00 | 11:54 | 15:05 | 42.116 | -70.380 | 63.7 | 10.0 | clear overhead, cirrus clouds around | 6 in waves | 5 knts | 1,2,3,4,5,6,7,8,10,11 | 0,10,20,30,50 |
| B3 | 07-Jul-98 | 12:55 | 13:50 | 17:00 | 41.908 | -70.228 | 32.0 | 11.0 | high cirrus, 70% cloud cover | glassy calm | 0 knts | 1,2,3,4,5,6,7,8,10,11 | 0,10,20,30 |
| B4 | 07-Jul-98 | 14:30 | 15:00 | 18:35 | 41.835 | -70.454 | 23.7 | 7.0 | 95% overcast, overcast overhead | 6 in waves | 5 knts | 1,2,3,4,5,6,7,8 | 0,10,20 |
| C1 | 08-Jul-98 | 11:00 | 12:10 | 15:05 | 41.250 | -70.417 | 28.7 | 8.0 | 100% overcast | 6 in waves | 0-3 knts | 1,2,3,4,5,7,8,9 | 0,10,25 |
| C2 | 08-Jul-98 | 13:00 | 13:54 | 17:05 | 41.217 | -70.166 | 13.0 | 6.0 | 100% overcast, fog bank approaching | 6 in waves | 0-3 knts | 1,2,3,4,5,6,7,8,9 | 0,10 |
| C3 | 08-Jul-98 | 15:00 | 16:05 | 19:05 | 41.082 | -69.867 | 20.5 | 6.5 | 100% overcast, light drizzle | 1 foot waves | 6 knts | 1,2,3,4,5,6,7,8,9 | 0,10,20 |
| C4 | 08-Jul-98 | 16:45 | 17:55 | 20:50 | 40.999 | -69.683 | 39.0 | 8.0 | 100% overcast, light drizzle | flat calm | 0-3 knts | 1,2,3,4,5,6,7,8,9 | 0,10,20,30 |
| D1 | 09-Jul-98 | 9:40 | 10:25 | 13:45 | 41.250 | -69.923 | 32.0 | 5.0 | clear overhead, 10% clouds in horizon | 1 foot waves | 6 knts | 1,2,3,4,5,6,7,8,9,10,11 | 0,10,20,30 |
| D2 | 09-Jul-98 | 11:15 | 12:06 | 15:20 | 41.149 | -69.748 | 24.0 | 7.0 | clear overhead, 10% clouds in horizon | glassy calm | 0-3 knts | 1,2,3,4,5,6,7,8,9,10,11 | 0,10,20 |

| Station | Date | Time On | Time Off | Time GMT |  | Latitude |  | Longitude | Total Depth(m) | Secchi Depth | Sky State | Sea State | Wind speed | Measurements | BottleDepths(m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2U | 09-Jul-98 | 12:33 | 12:40 | 16:37 |  | 41.053 |  | -69.843 | 13.0 |  | clear overhead, 10% clouds in horizon | 6 inch waves | 6 knts | 1,2,3,10,11 | 0 |  |
| D3 | 09-Jul-98 | 12:50 | 13:47 | 17:35 |  | 41.015 |  | -69.884 | 35.8 | 7.0 | clear overhead, haze in horizon | 1 foot waves | 5-8 knts | 1,2,3,4,5,6,7,8,9,10,11 | 0,10,20,30 |  |
| D3U | 09-Jul-98 | 14:05 | 14:12 | 18:08 |  | 41.042 |  | -69.958 | 28.5 |  | clear overhead, haze in horizon | 1 foot waves | 5-8 knts | 1,2,3,10,11 | 0 |  |
| D4 | 09-Jul-98 | 14:40 | 15:00 | 18:45 |  | 41.117 |  | -70.052 | 20.7 | 6.5 | clear overhead | 1 foot waves | 6 knts | 1,2,3,4,5,6,7,8,9,10,11 | 0,10,20 |  |
| E1 | 10-Jul-98 | 11:30 | 12:07 | 15:35 |  | 42.121 |  | -70.484 | 51.0 | 11.0 | clear overhead | 1 foot waves | 5-8 knts | 1,2,3,4,5,6,7,8,9,10,11 | 0,10,20 |  |
| E2 | 10-Jul-98 | 12:37 | 12:48 | 16:38 |  | 42.207 |  | -70.486 | 64.0 | 10.0 | clear overhead, 30% overcast | 1 foot waves | 5-8 knts | 1,2,3,4,6,7,8,9,11 | 0 |  |
| 1-ChlF (Volume filtered=100 mL) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2-Ap (Volume filtered = 1000 mL) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3-TSS (Volume filtered = 1000 mL) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4-HPLC pigments (Volume filtered = 1000 mL) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5-AC9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6-CTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7-Hydroscat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8-SMSR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9-SPMR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10-SIMBAD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11-Sunphotometer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## D.  Sampling Gear

The SPMR (Serial No. 024) and the SMSR (Serial No. 024) are multi-spectral radiometers manufactured by Satlantic, Inc.  The SPMR is a free-falling instrument that measures 13 wavelengths of downwelling irradiance, 12 wavelengths of upwelling radiance (Table 2.), depth, temperature, conductivity, salinity, tilt, and roll.  The SMSR is a surface tethered buoy that measures 13 wavelengths of above-water downwelling irradiance and 13 wavelengths of upwelling radiance at 75 cm below the surface.  Both the instruments are designed to be used away from the ship to avoid perturbations of the in-situ light field by ship shadow.

Table 2.  Center Wavelengths for the SPMR System.

| Channel No. | #### SPMR Downwelling Irradiance | SPMR Upwelling Radiance | #### SMSR Downwelling Irradiance | SMSR Upwelling Radiance |
| --- | --- | --- | --- | --- |
| 1 | 339.3 nm | -- | 339.2 nm | 339.9 nm |
| 2 | 379.8 nm | 379.8 nm | 380.1 nm | 380.1 nm |
| 3 | 412.4 nm | 411.2 nm | 411.2 nm | 412.4 nm |
| 4 | 443.2 nm | 443.3 nm | 442.0 nm | 442.8 nm |
| 5 | 489.6 nm | 490.3 nm | 490.4 nm | 489.9 nm |
| 6 | 509.4 nm | 510.4 nm | 510.5 nm | 509.7 nm |
| 7 | 520.8 nm | 519.2 nm | 519.3 nm | 519.9 nm |
| 8 | 554.9 nm | 554.3 nm | 554.9 nm | 554.4 nm |
| 9 | 565.5 nm | 565.1 nm | 564.9 nm | 565.3 nm |
| 10 | 619.1 nm | 619.1 nm | 619.2 nm | 619.3 nm |
| 11 | 665.3 nm | 665.6 nm | 665.4 nm | 664.4 nm |
| 12 | 669.7 nm | 670.0 nm | 670.1 nm | 669.8 nm |
| 13 | 683.9 nm | 683.8 nm | 682.2 nm | 682.7 nm |

## E.  Bottle Samples

The chlorophyll biomass was determined using a Turner Designs fluorometer (Parsons et al. 1984).  The Total Suspended Solids (TSS) concentration was measured as described by Parsons et al. (1984).  Phytoplankton pigment concentrations were determined by HPLC as described in Tester et al. (1995).  Discrete water samples were obtained for these analyses using Niskin bottles deployed just before the optics cage cast.

## F.  Optical Data Processing

Data for stations A1 to B4, where only the SMSR was used, were processed using the Satlantic Proview software.  The data were binned and averaged.  The water leaving radiance was calculated from the measured above surface downwelling irradiance and the radiance at 0.75 m below the surface.  The below surface radiance was propagated to the surface using the attenuation coefficient (k) calculated as per Morel (1988).  Briefly, the approximate chlorophyll concentration at that location was calculated using the 443/555-band ratio. The k at 490 nm for that chlorophyll concentration was calculated and then transferred to the other wavelengths using the relationships detailed in Morel (1988).

The normalized water leaving radiance data for stations C1 to E2 were calculated from the SMSR data as detailed above except that the k for each station was calculated from the SPMR measurements.  The SPMR data was binned to 0.5 m bins and k was calculated for the top 10 m.  This k was then applied to the SMSR radiance measurement 0.75 m below the surface to propagate it to the surface.  In both above cases, the radiance was propagated through the air-water interface as detailed in O’Reilly et al. (1998).

# IV.  Results

Results of the study are detailed below.

## A.  Bottle Samples

The surface chlorophyll concentrations (Chl a) varied from about 0.4 g/l to 2.0 g/l (Table 3).  Inside Massachusetts Bay, the maximum chlorophyll concentrations were at about 10 m, while on the Nantucket shoals the water column was well mixed with uniform chlorophyll concentrations.  The bottle samples showed that the chlorophyll concentrations were higher in the Shoals than inside Massachusetts Bay, even close to the shore.  Chlorophyll a concentrations at the surface as measured using the HPLC technique varied from 0.1 in Massachusetts Bay to 1.5 g/l on the Shoals (Tables 4 and 5).

Table 3.  Fluorometric Pigment Analyses.

All pigment values in g/l.

| Station | Depth (m) | Avg Chl a | Avg Pheo | Avg Chl a+Pheo |
| --- | --- | --- | --- | --- |
| A1 | S | 0.657 | 0.125 | 0.782 |
| A1 | 10 | 1.175 | 0.436 | 1.610 |
| A1 | 20 | 0.759 | 0.434 | 1.193 |
| A1 | 30 | 0.148 | 0.611 | 0.760 |
| A1 | 50 | 0.081 | 0.390 | 0.471 |
| A2 | S | 0.651 | 0.131 | 0.782 |
| A2 | 10 | 1.211 | 0.320 | 1.531 |
| A2 | 20 | 0.542 | 0.286 | 0.828 |
| A2 | 30 | 0.286 | 0.307 | 0.592 |
| A2 | 50 | 0.145 | 0.217 | 0.362 |
| A3 | S | 0.771 | 0.137 | 0.908 |
| A3 | 10 | 2.115 | 0.530 | 2.644 |
| A3 | 20 | 0.349 | 0.539 | 0.888 |
| B1 | S | 0.482 | 0.121 | 0.603 |
| B1 | 10 | 1.229 | 0.362 | 1.591 |
| B1 | 20 | 1.030 | 0.183 | 1.213 |
| B1 | 30 | 0.249 | 0.367 | 0.616 |
| B2 | S | 0.566 | 0.090 | 0.656 |
| B2 | 10 | 1.301 | 0.210 | 1.511 |
| B2 | 20 | 2.802 | 0.598 | 3.400 |
| B2 | 30 | 2.042 | 0.622 | 2.664 |
| B2 | 50 | 0.217 | 0.320 | 0.537 |
| Station | Depth (m) | Avg Chl a | Avg Pheo | Avg Chl a+Pheo |
| B3 | S | 0.476 | 0.087 | 0.563 |
| B3 | 10 | 0.813 | 0.161 | 0.974 |
| B3 | 20 | 0.759 | 0.288 | 1.047 |
| B3 | 30 | 0.518 | 0.237 | 0.756 |
| B4 | S | 1.139 | 0.233 | 1.372 |
| B4 | 10 | 1.410 | 0.459 | 1.869 |
| B4 | 20 | 0.633 | 0.401 | 1.034 |
| C1 | S | 0.777 | 0.270 | 1.047 |
| C1 | 10 | 2.603 | 0.618 | 3.221 |
| C1 | 25 | 4.013 | 0.998 | 5.010 |
| C2 | S | 1.229 | 0.421 | 1.650 |
| C2 | 10 | 1.699 | 0.408 | 2.108 |
| C3 | S | 1.482 | 0.745 | 2.227 |
| C3 | 10 | 1.500 | 0.786 | 2.286 |
| C3 | 20 | 1.482 | 0.804 | 2.286 |
| C4 | S | 1.518 | 0.868 | 2.386 |
| C4 | 10 | 1.247 | 0.920 | 2.167 |
| C4 | 20 | 1.392 | 0.795 | 2.187 |
| C4 | 30 | 1.374 | 0.793 | 2.167 |
| D1 | S | 1.988 | 0.457 | 2.446 |
| D1 | 10 | 1.808 | 0.698 | 2.505 |
| D1 | 20 | 1.898 | 0.568 | 2.465 |
| D1 | 30 | 1.898 | 0.746 | 2.644 |
| D2 | S | 1.609 | 0.578 | 2.187 |
| D2 | 10 | 1.952 | 0.672 | 2.624 |
| D2 | 20 | 1.916 | 0.709 | 2.624 |
| UD2 | S | 1.645 | 0.363 | 2.008 |
| D3 | S | 1.374 | 0.197 | 1.571 |
| D3 | 10 | 1.464 | 0.524 | 1.988 |
| D3 | 20 | 1.862 | 0.445 | 2.306 |
| D3 | 30 | 2.097 | 0.528 | 2.624 |
| UD3 | S | 1.374 | 0.296 | 1.670 |
| D4 | S | 1.392 | 0.298 | 1.690 |
| D4 | 10 | 1.934 | 0.452 | 2.386 |
| D4 | 20 | 2.097 | 0.607 | 2.704 |
| E1 | S | 0.398 | 0.076 | 0.473 |
| E1 | 10 | 2.368 | 0.396 | 2.764 |
| E1 | 20 | 0.759 | 0.235 | 0.994 |
| E2 | S | 0.434 | 0.131 | 0.565 |

Table 4.  HPLC Pigment Abbreviations.

| Pigment | Abbreviation | Pigment | Abbreviation |
| --- | --- | --- | --- |
| Chlorophyll a | Chl a | 19’-Butanoyloxyfucoxanthin | 19-But |
| Sum of Chlorophyll a isomers | Sum Chl a | Fucoxanthin | Fucox |
| Chlorophyll b | Chl b | 19’-Hexanoyloxyfucoxanthin | 19-Hex |
| Chlorophyll c1 + c2 | Chl c1 + 2 | Diadinoxanthin | Diadin |
| Chlorophyll c3 | Chl c3 | Alloxanthin | Allox |
| PheophorbideDi | Ph-ide | Diatoxanthin | Diat |
| Pheophytin | Ph-tin | Lutein | Lutein |
| Peridinin | Perid | Zeaxanthin | Zeax |

Table 5.  HPLC Pigment Analysis Results.

All pigment values in g/l.

| Station | Chl a | Sum Chl a | Chl b | Chl c3 | Chl c1+2 | Ph-ide | Ph-tin | Perid | 19-But | Fucox | 19-Hex | Diadin | Allox | Diat | Lutein | Zeax |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | 0.119 | 0.266 | 0.000 | 0.014 | 0.040 | 0.000 | 0.006 | 0.022 | 0.025 | 0.102 | 0.302 | 0.049 | 0.015 | 0.021 | 0.000 | 0.025 |
| B2 | 0.144 | 0.247 | 0.000 | 0.016 | 0.043 | 0.014 | 0.009 | 0.022 | 0.027 | 0.106 | 0.532 | 0.074 | 0.021 | 0.024 | 0.010 | 0.018 |
| B3 | 0.189 | 0.261 | 0.032 | 0.015 | 0.037 | 0.000 | 0.000 | 0.024 | 0.024 | 0.079 | 0.503 | 0.063 | 0.018 | 0.022 | 0.009 | 0.024 |
| B4 | 0.126 | 0.289 | 0.000 | 0.028 | 0.092 | 0.032 | 0.008 | 0.050 | 0.024 | 0.231 | 0.519 | 0.110 | 0.039 | 0.037 | 0.028 | 0.000 |
| C1 | 0.173 | 0.343 | 0.027 | 0.035 | 0.080 | 0.006 | 0.006 | 0.030 | 0.036 | 0.232 | 0.538 | 0.114 | 0.023 | 0.048 | 0.013 | 0.028 |
| C2 | 0.276 | 0.595 | 0.091 | 0.040 | 0.122 | 0.008 | 0.000 | 0.036 | 0.038 | 0.388 | 0.560 | 0.115 | 0.037 | 0.034 | 0.018 | 0.042 |
| C3 | 1.452 | 1.579 | 0.302 | 0.061 | 0.168 | 0.035 | 0.054 | 0.042 | 0.041 | 0.521 | 0.821 | 0.111 | 0.033 | 0.038 | 0.012 | 0.034 |
| C4 | 1.222 | 1.451 | 0.366 | 0.052 | 0.150 | 0.060 | 0.041 | 0.072 | 0.029 | 0.342 | 0.627 | 0.078 | 0.037 | 0.024 | 0.011 | 0.048 |
| D1 | 1.300 | 1.558 | 0.213 | 0.077 | 0.192 | 0.058 | 0.057 | 0.062 | 0.033 | 0.618 | 0.355 | 0.130 | 0.037 | 0.037 | 0.013 | 0.028 |
| D2 | 1.038 | 1.269 | 0.181 | 0.061 | 0.164 | 0.059 | 0.046 | 0.038 | 0.025 | 0.513 | 0.393 | 0.134 | 0.030 | 0.038 | 0.012 | 0.028 |
| D3 | 0.865 | 1.040 | 0.183 | 0.055 | 0.127 | 0.023 | 0.038 | 0.035 | 0.033 | 0.350 | 0.589 | 0.099 | 0.029 | 0.039 | 0.012 | 0.028 |
| D4 | 1.086 | 1.300 | 0.276 | 0.055 | 0.138 | 0.012 | 0.028 | 0.042 | 0.031 | 0.389 | 0.490 | 0.122 | 0.046 | 0.043 | 0.019 | 0.043 |
| E1 | 0.237 | 0.312 | 0.039 | 0.025 | 0.049 | 0.002 | 0.011 | 0.026 | 0.027 | 0.099 | 0.432 | 0.075 | 0.019 | 0.038 | 0.011 | 0.028 |
| E2 | 0.251 | 0.334 | 0.030 | 0.027 | 0.051 | 0.000 | 0.005 | 0.028 | 0.026 | 0.103 | 0.534 | 0.072 | 0.019 | 0.033 | 0.010 | 0.029 |

## B.  Optical Data

The underwater SPMR unit was not available for the first two days, so only the surface SMSR was used at stations A1 to B4.  This made no difference to the measurements at these stations as they were very similar to those made with both the SPMR and SMSR on the return leg in the same water body (stations E1 and E2, Fig 5.).  The low reflectance spectra from Massachusetts Bay waters clearly show the influence of colored dissolved organic matter absorption.  The higher reflectance in Nantucket Shoals, especially at 670 nm suggest that they may be influenced by higher sediment content.

Figure 5.  Remote Sensing Reflectance Spectra.

## C.  Algorithm Evaluation

The OC2 algorithm (O’Reilly et al. 1998) consistently overestimates the chlorophyll concentration in Massachusetts Bay by a factor of 4 to 5, but appears to do better in the Nantucket Shoals region with an over-estimate factor of about 1.5 (Table 6.).  There was no relationship between chlorophyll concentration and the extent of overestimation.

Table 6.  Algorithm Evaluation.

| Station | A1 | A2 | A3 | B1 | B2 | B3 | B4 | C1 | C2 | C3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | 7/6/98 | 7/6/98 | 7/6/98 | 7/7/98 | 7/7/98 | 7/7/98 | 7/7/98 | 7/8/98 | 7/8/98 | 7/8/98 |
| TimeGMT | 16:30 | 18:30 | 20:30 | 13:35 | 15:05 | 17:00 | 18:35 | 11:05 | 17:05 | 19:05 |
| Latitude | 42.895 | 42.551 | 42.366 | 42.170 | 42.116 | 41.908 | 41.835 | 41.250 | 41.217 | 41.082 |
| Longitude | -70.373 | -70.448 | -70.779 | -70.575 | -70.380 | -70.228 | -70.454 | -70.417 | -70.166 | -69.867 |
| ChlF | 0.657 | 0.651 | 0.771 | 0.482 | 0.566 | 0.476 | 1.139 | 0.777 | 1.229 | 1.482 |
| Chla | -999 | -999 | -999 | 0.119 | 0.144 | 0.189 | 0.126 | 0.173 | 0.276 | 1.452 |
| Es380 | 54.990 | 58.910 | 20.920 | 49.190 | 65.800 | 63.980 | 33.150 | 25.455 | 15.736 | 11.424 |
| Es412 | 97.440 | 106.800 | 37.020 | 90.360 | 121.820 | 118.910 | 59.090 | 44.444 | 27.077 | 19.506 |
| Es443 | 111.180 | 118.980 | 40.020 | 100.050 | 134.500 | 130.270 | 64.240 | 47.418 | 28.657 | 20.596 |
| Es490 | 121.300 | 130.290 | 43.200 | 111.750 | 145.380 | 138.740 | 69.500 | 50.565 | 30.743 | 22.133 |
| Es510 | 122.500 | 128.970 | 43.680 | 112.750 | 146.300 | 140.120 | 69.860 | 50.239 | 30.119 | 21.504 |
| Es520 | 114.010 | 120.460 | 40.470 | 103.500 | 137.750 | 132.030 | 65.110 | 46.797 | 28.133 | 20.154 |
| Es555 | 113.230 | 121.170 | 40.210 | 104.640 | 137.110 | 130.740 | 65.070 | 46.172 | 27.589 | 19.645 |
| Es565 | 112.930 | 121.090 | 39.970 | 104.320 | 136.270 | 131.530 | 64.910 | 45.592 | 26.955 | 19.056 |
| Es619 | 104.410 | 109.150 | 36.430 | 97.270 | 126.130 | 120.310 | 59.440 | 41.255 | 24.415 | 17.283 |
| Es664 | 96.670 | 104.800 | 33.230 | 90.750 | 117.270 | 111.100 | 54.960 | 37.980 | 22.472 | 16.068 |
| Es670 | 96.400 | 96.930 | 33.930 | 90.550 | 116.390 | 110.960 | 54.850 | 37.933 | 22.551 | 16.131 |
| Es683 | 95.760 | 99.840 | 34.100 | 91.480 | 116.360 | 109.320 | 54.990 | 37.799 | 22.259 | 15.848 |
| ku380 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.456 | 0.471 | 0.468 |
| ku412 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.390 | 0.311 | 0.354 |
| ku443 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.314 | 0.188 | 0.264 |
| ku490 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.220 | 0.083 | 0.168 |
| ku510 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.198 | 0.060 | 0.149 |
| ku520 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.195 | 0.054 | 0.148 |
| ku555 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.168 | 0.021 | 0.131 |
| ku565 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.159 | 0.013 | 0.131 |
| ku619 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.304 | 0.143 | 0.356 |
| ku664 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.207 | 0.262 | 0.367 |
| ku670 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.192 | 0.218 | 0.314 |
| ku683 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.182 | 0.252 | 0.284 |
| Lw380 | 0.009 | 0.018 | 0.004 | 0.014 | 0.036 | 0.035 | 0.011 | 0.048 | 0.031 | 0.022 |
| Lw412 | 0.035 | 0.056 | 0.011 | 0.045 | 0.092 | 0.100 | 0.030 | 0.101 | 0.062 | 0.043 |
| Lw443 | 0.060 | 0.104 | 0.021 | 0.085 | 0.157 | 0.167 | 0.049 | 0.137 | 0.085 | 0.058 |
| Lw490 | 0.100 | 0.180 | 0.039 | 0.150 | 0.256 | 0.277 | 0.085 | 0.186 | 0.127 | 0.087 |
| Lw510 | 0.109 | 0.191 | 0.043 | 0.157 | 0.265 | 0.292 | 0.096 | 0.186 | 0.132 | 0.090 |
| Lw520 | 0.105 | 0.180 | 0.042 | 0.150 | 0.250 | 0.267 | 0.093 | 0.175 | 0.127 | 0.087 |
| Lw555 | 0.106 | 0.181 | 0.044 | 0.146 | 0.237 | 0.256 | 0.098 | 0.160 | 0.121 | 0.083 |
| Lw565 | 0.103 | 0.173 | 0.042 | 0.139 | 0.221 | 0.243 | 0.095 | 0.149 | 0.114 | 0.078 |
| Lw619 | 0.036 | 0.046 | 0.011 | 0.036 | 0.049 | 0.056 | 0.023 | 0.034 | 0.024 | 0.019 |
| Lw664 | 0.026 | 0.027 | 0.007 | 0.023 | 0.030 | 0.029 | 0.014 | 0.019 | 0.016 | 0.012 |
| Lw670 | 0.028 | 0.029 | 0.007 | 0.023 | 0.030 | 0.034 | 0.015 | 0.020 | 0.017 | 0.013 |
| Lw683 | 0.030 | 0.029 | 0.008 | 0.023 | 0.030 | 0.033 | 0.017 | 0.021 | 0.018 | 0.015 |
| nLw380 | 0.017 | 0.033 | 0.020 | 0.031 | 0.058 | 0.058 | 0.037 | 0.203 | 0.210 | 0.207 |
| nLw412 | 0.063 | 0.092 | 0.054 | 0.086 | 0.131 | 0.146 | 0.089 | 0.392 | 0.397 | 0.384 |
| nLw443 | 0.101 | 0.163 | 0.097 | 0.160 | 0.218 | 0.239 | 0.142 | 0.538 | 0.552 | 0.522 |
| nLw490 | 0.156 | 0.262 | 0.170 | 0.255 | 0.333 | 0.378 | 0.232 | 0.697 | 0.779 | 0.741 |
| nLw510 | 0.172 | 0.286 | 0.190 | 0.268 | 0.349 | 0.402 | 0.264 | 0.716 | 0.847 | 0.807 |
| nLw520 | 0.168 | 0.272 | 0.190 | 0.264 | 0.331 | 0.369 | 0.260 | 0.681 | 0.821 | 0.784 |
| nLw555 | 0.175 | 0.277 | 0.202 | 0.259 | 0.322 | 0.365 | 0.279 | 0.643 | 0.819 | 0.790 |
| Station | A1 | A2 | A3 | B1 | B2 | B3 | B4 | C1 | C2 | C3 |
| nLw565 | 0.168 | 0.264 | 0.195 | 0.247 | 0.299 | 0.340 | 0.270 | 0.601 | 0.781 | 0.759 |
| nLw619 | 0.058 | 0.071 | 0.050 | 0.063 | 0.067 | 0.079 | 0.066 | 0.141 | 0.168 | 0.190 |
| nLw664 | 0.042 | 0.040 | 0.032 | 0.039 | 0.040 | 0.041 | 0.039 | 0.077 | 0.109 | 0.116 |
| nLw670 | 0.045 | 0.046 | 0.032 | 0.039 | 0.040 | 0.047 | 0.042 | 0.081 | 0.113 | 0.124 |
| nLw683 | 0.046 | 0.044 | 0.033 | 0.037 | 0.039 | 0.045 | 0.045 | 0.082 | 0.123 | 0.141 |
| Rrs380 | 0.000 | 0.000 | 0.000 | 0.000 | 0.001 | 0.001 | 0.000 | 0.002 | 0.002 | 0.002 |
| Rrs412 | 0.000 | 0.001 | 0.000 | 0.000 | 0.001 | 0.001 | 0.001 | 0.002 | 0.002 | 0.002 |
| Rrs443 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.001 | 0.003 | 0.003 | 0.003 |
| Rrs490 | 0.001 | 0.001 | 0.001 | 0.001 | 0.002 | 0.002 | 0.001 | 0.004 | 0.004 | 0.004 |
| Rrs510 | 0.001 | 0.001 | 0.001 | 0.001 | 0.002 | 0.002 | 0.001 | 0.004 | 0.004 | 0.004 |
| Rrs520 | 0.001 | 0.001 | 0.001 | 0.001 | 0.002 | 0.002 | 0.001 | 0.004 | 0.005 | 0.004 |
| Rrs555 | 0.001 | 0.001 | 0.001 | 0.001 | 0.002 | 0.002 | 0.002 | 0.004 | 0.004 | 0.004 |
| Rrs565 | 0.001 | 0.001 | 0.001 | 0.001 | 0.002 | 0.002 | 0.001 | 0.003 | 0.004 | 0.004 |
| Rrs619 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.001 | 0.001 | 0.001 |
| Rrs664 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.001 | 0.001 | 0.001 |
| Rrs670 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.001 | 0.001 | 0.001 |
| Rrs683 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.001 | 0.001 | 0.001 |
| kd490 | -999 | -999 | -999 | -999 | -999 | -999 | -999 | 0.183 | 0.195 | 0.175 |
| X | -0.056 | -0.033 | -0.082 | -0.015 | 0.008 | 0.008 | -0.088 | 0.024 | -0.031 | -0.032 |
| Expov2 | 0.425 | 0.372 | 0.488 | 0.332 | 0.280 | 0.279 | 0.500 | 0.244 | 0.367 | 0.370 |
| OC2v2 | 2.569 | 2.263 | 2.981 | 2.054 | 1.814 | 1.806 | 3.072 | 1.660 | 2.235 | 2.254 |
| MeasChlF | 0.657 | 0.651 | 0.771 | 0.482 | 0.566 | 0.476 | 1.139 | 0.777 | 1.229 | 1.482 |
| sat/meas | 3.9 | 3.5 | 3.9 | 4.3 | 3.2 | 3.8 | 2.7 | 2.1 | 1.8 | 1.5 |

| Station | C4 | D1 | D2 | D2U | D3 | D3U | D4 | E1 | E2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | 7/8/98 | 7/9/98 | 7/9/98 | 7/9/98 | 7/9/98 | 7/9/98 | 7/9/98 | 7/10/98 | 7/10/98 |
| TimeGMT | 20:50 | 13:45 | 15:20 | 16:37 | 17:35 | 18:08 | 18:45 | 15:35 | 16:38 |
| Latitude | 40.999 | 41.250 | 41.149 | 41.053 | 41.015 | 41.042 | 41.117 | 42.121 | 42.207 |
| Longitude | -69.683 | -69.923 | -69.748 | -69.843 | -69.884 | -69.958 | -70.052 | -70.484 | -70.486 |
| ChlF | 1.518 | 1.988 | 1.609 | 1.645 | 1.374 | 1.374 | 1.392 | 0.398 | 0.434 |
| Chla | 1.222 | 1.300 | 1.038 | -999 | 0.865 | -999 | 1.086 | 0.237 | 0.251 |
| Es380 | 13.473 | 55.070 | 69.096 | -999 | 68.745 | -999 | 60.382 | 62.751 | 66.657 |
| Es412 | 23.230 | 99.988 | 126.854 | -999 | 127.028 | -999 | 109.340 | 111.680 | 120.823 |
| Es443 | 24.856 | 110.439 | 139.289 | -999 | 139.693 | -999 | 119.729 | 124.228 | 132.139 |
| Es490 | 27.043 | 122.757 | 151.119 | -999 | 151.380 | -999 | 130.944 | 137.225 | 145.401 |
| Es510 | 26.531 | 122.658 | 151.234 | -999 | 151.461 | -999 | 131.451 | 137.824 | 147.543 |
| Es520 | 24.739 | 112.790 | 140.940 | -999 | 142.006 | -999 | 121.506 | 128.262 | 135.650 |
| Es555 | 24.286 | 114.087 | 140.939 | -999 | 141.353 | -999 | 121.889 | 125.699 | 134.144 |
| Es565 | 23.771 | 113.672 | 141.776 | -999 | 141.448 | -999 | 120.809 | 127.162 | 136.505 |
| Es619 | 21.789 | 105.470 | 129.897 | -999 | 130.522 | -999 | 112.781 | 119.585 | 127.359 |
| Es664 | 20.323 | 97.549 | 119.844 | -999 | 120.884 | -999 | 104.124 | 110.340 | 119.445 |
| Es670 | 20.463 | 97.720 | 119.654 | -999 | 120.171 | -999 | 103.299 | 110.745 | 119.000 |
| Es683 | 20.265 | 98.088 | 118.204 | -999 | 118.782 | -999 | 104.057 | 110.070 | 116.437 |
| ku380 | 0.568 | 0.855 | 0.336 | -999 | 0.369 | -999 | 0.406 | 0.490 | 0.922 |
| ku412 | 0.420 | 0.635 | 0.321 | -999 | 0.366 | -999 | 0.374 | 0.371 | 0.586 |
| ku443 | 0.308 | 0.483 | 0.227 | -999 | 0.287 | -999 | 0.280 | 0.260 | 0.366 |
| ku490 | 0.197 | 0.325 | 0.111 | -999 | 0.201 | -999 | 0.175 | 0.168 | 0.205 |
| ku510 | 0.179 | 0.291 | 0.088 | -999 | 0.184 | -999 | 0.151 | 0.156 | 0.180 |
| ku520 | 0.178 | 0.287 | 0.084 | -999 | 0.183 | -999 | 0.147 | 0.156 | 0.177 |
| ku555 | 0.165 | 0.253 | 0.056 | -999 | 0.167 | -999 | 0.119 | 0.142 | 0.152 |
| ku565 | 0.162 | 0.248 | 0.050 | -999 | 0.165 | -999 | 0.114 | 0.140 | 0.148 |
| Station | C4 | D1 | D2 | D2U | D3 | D3U | D4 | E1 | E2 |
| ku619 | 0.450 | 0.536 | 0.228 | -999 | 0.308 | -999 | 0.319 | 0.263 | 0.352 |
| ku664 | 0.528 | 0.665 | 0.186 | -999 | 0.229 | -999 | 0.294 | 0.191 | 0.393 |
| ku670 | 0.468 | 0.612 | 0.154 | -999 | 0.218 | -999 | 0.246 | 0.151 | 0.351 |
| ku683 | 0.435 | 0.569 | 0.178 | -999 | 0.221 | -999 | 0.248 | 0.108 | 0.314 |
| Lw380 | 0.030 | 0.155 | 0.120 | -999 | 0.139 | -999 | 0.123 | 0.049 | 0.043 |
| Lw412 | 0.054 | 0.317 | 0.260 | -999 | 0.313 | -999 | 0.282 | 0.119 | 0.094 |
| Lw443 | 0.070 | 0.458 | 0.358 | -999 | 0.442 | -999 | 0.397 | 0.183 | 0.145 |
| Lw490 | 0.104 | 0.727 | 0.547 | -999 | 0.663 | -999 | 0.599 | 0.279 | 0.222 |
| Lw510 | 0.106 | 0.793 | 0.577 | -999 | 0.688 | -999 | 0.636 | 0.281 | 0.227 |
| Lw520 | 0.100 | 0.778 | 0.557 | -999 | 0.644 | -999 | 0.598 | 0.258 | 0.218 |
| Lw555 | 0.091 | 0.822 | 0.556 | -999 | 0.624 | -999 | 0.597 | 0.237 | 0.209 |
| Lw565 | 0.087 | 0.806 | 0.538 | -999 | 0.598 | -999 | 0.577 | 0.221 | 0.198 |
| Lw619 | 0.023 | 0.248 | 0.128 | -999 | 0.137 | -999 | 0.149 | 0.054 | 0.055 |
| Lw664 | 0.016 | 0.164 | 0.071 | -999 | 0.075 | -999 | 0.085 | 0.030 | 0.037 |
| Lw670 | 0.017 | 0.152 | 0.071 | -999 | 0.077 | -999 | 0.087 | 0.031 | 0.033 |
| Lw683 | 0.019 | 0.143 | 0.071 | -999 | 0.075 | -999 | 0.085 | 0.029 | 0.032 |
| nLw380 | 0.238 | 0.301 | 0.185 | -999 | 0.216 | -999 | 0.218 | 0.084 | 0.068 |
| nLw412 | 0.402 | 0.550 | 0.356 | -999 | 0.428 | -999 | 0.447 | 0.185 | 0.136 |
| nLw443 | 0.528 | 0.775 | 0.480 | -999 | 0.591 | -999 | 0.620 | 0.275 | 0.205 |
| nLw490 | 0.727 | 1.122 | 0.686 | -999 | 0.830 | -999 | 0.866 | 0.384 | 0.289 |
| nLw510 | 0.768 | 1.247 | 0.736 | -999 | 0.876 | -999 | 0.934 | 0.393 | 0.297 |
| nLw520 | 0.735 | 1.258 | 0.720 | -999 | 0.827 | -999 | 0.898 | 0.367 | 0.293 |
| nLw555 | 0.697 | 1.341 | 0.734 | -999 | 0.822 | -999 | 0.911 | 0.351 | 0.290 |
| nLw565 | 0.673 | 1.308 | 0.700 | -999 | 0.779 | -999 | 0.881 | 0.321 | 0.268 |
| nLw619 | 0.182 | 0.400 | 0.167 | -999 | 0.179 | -999 | 0.225 | 0.077 | 0.074 |
| nLw664 | 0.120 | 0.262 | 0.093 | -999 | 0.097 | -999 | 0.127 | 0.042 | 0.048 |
| nLw670 | 0.125 | 0.238 | 0.091 | -999 | 0.099 | -999 | 0.130 | 0.043 | 0.042 |
| nLw683 | 0.141 | 0.217 | 0.090 | -999 | 0.094 | -999 | 0.121 | 0.039 | 0.041 |
| Rrs380 | 0.002 | 0.003 | 0.002 | -999 | 0.002 | -999 | 0.002 | 0.001 | 0.001 |
| Rrs412 | 0.002 | 0.003 | 0.002 | -999 | 0.003 | -999 | 0.003 | 0.001 | 0.001 |
| Rrs443 | 0.003 | 0.004 | 0.003 | -999 | 0.003 | -999 | 0.003 | 0.002 | 0.001 |
| Rrs490 | 0.004 | 0.006 | 0.004 | -999 | 0.004 | -999 | 0.005 | 0.002 | 0.002 |
| Rrs510 | 0.004 | 0.007 | 0.004 | -999 | 0.005 | -999 | 0.005 | 0.002 | 0.002 |
| Rrs520 | 0.004 | 0.007 | 0.004 | -999 | 0.005 | -999 | 0.005 | 0.002 | 0.002 |
| Rrs555 | 0.004 | 0.007 | 0.004 | -999 | 0.004 | -999 | 0.005 | 0.002 | 0.002 |
| Rrs565 | 0.004 | 0.007 | 0.004 | -999 | 0.004 | -999 | 0.005 | 0.002 | 0.002 |
| Rrs619 | 0.001 | 0.002 | 0.001 | -999 | 0.001 | -999 | 0.001 | 0.001 | 0.000 |
| Rrs664 | 0.001 | 0.002 | 0.001 | -999 | 0.001 | -999 | 0.001 | 0.000 | 0.000 |
| Rrs670 | 0.001 | 0.002 | 0.001 | -999 | 0.001 | -999 | 0.001 | 0.000 | 0.000 |
| Rrs683 | 0.001 | 0.002 | 0.001 | -999 | 0.001 | -999 | 0.001 | 0.000 | 0.000 |
| kd490 | 0.263 | 0.345 | 0.118 | -999 | 0.195 | -999 | 0.218 | 0.142 | 0.184 |
| X | 0.012 | -0.086 | -0.035 | -999 | 0.000 | -999 | -0.027 | 0.022 | -0.028 |
| Expov2 | 0.272 | 0.498 | 0.376 | -999 | 0.297 | -999 | 0.360 | 0.248 | 0.361 |
| OC2v2 | 1.776 | 3.052 | 2.286 | -999 | 1.890 | -999 | 2.196 | 1.677 | 2.203 |
| MeasChlF | 1.518 | 1.988 | 1.609 | 1.645 | 1.374 | 1.374 | 1.392 | 0.398 | 0.434 |
| sat/meas | 1.2 | 1.5 | 1.4 | -999 | 1.4 | -999 | 1.6 | 4.2 | 5.1 |

## D.  Satellite Data

The skies were partly cloudy or completely overcast July 6, 7, and 8 so no matching SeaWiFS imagery are available for these days.  July 9 was a clear day with a good overpass (Figure 6.) and additional stations (D2u and D3u) were sampled for chlorophyll only.  The standard stations (D1, D2, D3, D4) were spaced 12 km apart, while D2u and D3u were approximately midway, about 6 km from D3.  Stations D2 and D3 were sampled within two hours of the overpass, with D3 directly coincident with the overpass, and thus can be used to validate the normalized water leaving values.  All stations on July 9 were occupied within six hours of the overpass and can be used to validate the OC2 chlorophyll algorithm.

Figure 6  SeaWiFS Chlorophyll Data for July 9, 1998 with Stations D1-D4 Marked.

Comparison of the chlorophyll products derived from the satellite and the in-situ optical measurement using the OC2 algorithm and fluorometrically determined in the lab show that the algorithm overestimates the actual chlorophyll concentrations by a factor of 1.5 (Figure 7.).  While there was a relatively large difference between the satellite and in-situ measurements of normalized water leaving radiance for the blue wavelengths (412 and 443 nm), the difference is quite small for the 490, 510 and 555 nm bands, especially at station D2 (<3%) (Figure 8.).

Figure 7.  Chlorophyll Product Determined Three Different Ways.

Figure 8.  Satellite Derived and In-situ Measured Normalized Water Leaving Radiance for July 9, 1998.

# V.  Summary

A regional algorithm needs to be developed for the Massachusetts Bay in order to make SeaWiFS data useful to the coastal managers administering a water body with high anthropogenic impact.  On the other hand, the OC2 algorithm tested using the in-situ data appears to work better in the Nantucket Shoals region, although it still overestimates chlorophyll by a factor of 1.5.  There is excellent radiometric agreement at station D2 for the 490, 510, and 555 nm bands, indicating that it is the OC2 algorithm and not the atmospheric correction algorithm is affecting the accuracy of the chlorophyll estimates.

# VI.  References

Morel, A. (1988). Optical modeling of the upper ocean in relation to its biogenous matter content (case 1 water). J. Geophys. Res. 93(C9): 10749-10768.

O'Reilly, J. E., S. Maritorena, B. G. Mitchell, D. A. Siegel, K. L. Carder, S. A. Garver, M. Kahru, and C. McClain. (1998).  Ocean color chlorophyll algorithms for SeaWiFS. J. Geophys. Res. 103(C11): 24937-24953.

Parsons, T. R., Y. Maita and C. M. Lalli (1984). A Manual For Chemical And Biological Methods For Seawater Analysis, Pergamon Press.

Smith, R. C., K. S. Baker and P. Dustan (1981). Fluorometric Techniques for the Measurement of Oceanic Chlorophyll in the Support of Remote Sensing. SIO Ref. 81-17, Visibility Laboratory, Scripps Institution of Oceanography, La Jolla, CA 92093,  14 pp.

Tester, P.A., M.E. Geesey, C. Guo. H.W. Paerl and D.F. Millie (1995). Evaluating phytoplankton dynamics in the Newport River estuary (North Carolina, USA) by HPLC-derived pigment profiles. Mar. Ecol. Prog. Ser. 124:237-245.

# VII.  Metadata

Identification Information:

Citation:

Citation Information:

Originator:  Ajit Subramaniam, REMSA, NOAA/NESDIS

Originator:  Mary Culver, TPMC, NOAA Coastal Services Center

Originator:  David Phinney, Bigelow Laboratory for Ocean Sciences

Originator:  Douglas Phinney, Bigelow Laboratory for Ocean Sciences

Originator:  Jeffery Brown, Bigelow Laboratory for Ocean Sciences

Originator:  Brian Scheiber, SeaWiFS Project Office, NASA GSFC

Originator:  John C. Brock, Center for Coastal Geology, USGS

Originator:  David Eslinger, NOAA Coastal Services Center

Originator:  Chrisropher Brown, NOAA/NESDIS

Publication Date: 1999

Title: NOAA CSC/CRS Cruise JUL98NAN: Nantucket Shoals Cruise

Geospatial Data Presentation Form:  profile

Series Information:

Series Name:  CSC Technical Report

Issue Identification: NOAA CSC 99043-PUB

Publication Information:

Publication Place:  Charleston, South Carolina

Publisher:  NOAA Coastal Services Center

Online Linkage:  http://www.csc.noaa.gov/crs/cruises/jul98nan/index.html

Description:

Abstract: See page iii.

Purpose: See objectives on page 1.

Time Period of Content:

Time Period Information:

Range of Dates/Times:

Beginning Date:  19980706

Ending Date:  19980710

Currentness Reference: Publication Date

Status:

Progress:  Complete

Maintenance and Update Frequency:  Unknown

Spatial Domain:

Bounding Coordinates:

West Bounding Coordinate:  -70.779

East Bounding Coordinate:  -69.748

North Bounding Coordinate:  42.895

South Bounding Coordinate:  40.999

Keywords:

Theme:

Theme Keyword Thesaurus:  None

Theme Keyword:  oceanography

Theme Keyword:  bio-optical

Theme Keyword:  turbidity

Theme Keyword:  water clarity

Theme Keyword:  algal blooms

Theme Keyword:  coastal water optics

Theme Keyword:  case II algorithms

Theme Keyword:  light attenuation

Theme Keyword:  reflectance difference

Theme Keyword:  in-situ optical profiling

Theme Keyword:  ocean color satellites

Theme Keyword:  coastal ocean algorithm development

Theme Keyword:  river plumes

Theme Keyword:  AVHRR

Theme Keyword:  downwelling irradiance

Theme Keyword:  upwelling radiance

Theme Keyword:  temperature

Theme Keyword:  chlorophyll

Theme Keyword:  particulate absorption

Theme Keyword:  colored dissolved organic matter

Theme Keyword:  salinity

Theme Keyword:  spectral attenuation

Theme Keyword:  spectral absorption

Theme Keyword:  beam attenuation

Theme Keyword:  scalar quantum irradiance

Theme Keyword:  light scattering

Theme Keyword:  fluorescence

Place:

Place Keyword Thesaurus:  None

Place Keyword:  Massachusetts Bay

Place Keyword:  Nantucket Shoals

Place Keyword:  Massachusetts

Place Keyword:  United States

Temporal:

Temporal Keyword Thesaurus: None

Temporal Keyword:  Summer

Temporal Keyword:  July, 1998

Access Constraints:  None

Use Constraints:  This data was acquired for scientific research and is applicable for algorithm validation purposes.  Knowledge of in-water optics is expected of users for interpretation of the data.  Users of this data are required to provide appropriate attribution in the form of co-authorship for any publications that use this data, unless formal permission to do otherwise is granted by NOAA/CSC.

Point of Contact:

Contact Information:

Contact Organization Primary:

Contact Organization: NOAA Coastal Services Center

Contact Address:

Address Type:  mailing and physical address

Address:  2234 Hobson Avenue

City:  Charleston

State or Province:  South Carolina

Postal Code:  29405-2413

Country:  USA

Contact Voice Telephone:  (843) 740-1200

Contact Facsimile Telephone:  (843) 740-1224

Contact Electronic Mail Address:  csc@csc.noaa.gov

Hours of Service:  8AM-5PM, M-F

Data Set Credit: Captain Paul Pelletier and the crew of the R/V Gulf Challenger.  A NASA SIMBIOS contract to Dr. Chris Brown and John Brock as well as a NOAA Coastal Ocean Program grant to Dr. Charles Yentsch and David Phinney contributed to this cruise.

Data Quality Information:

Attribute Accuracy:

Attribute Accuracy Report:  Refer to the Process Step section for specific calibration information.  The primary instrumentation on the cruise are sent to the respective manufacturers for calibration at least once per year. Calibration certificates for the relevant instrumentation are available in the full written report.  Secondary instrumentation were calibrated only upon purchase.  Calibration of the WetStar in situ Fluorometer is conducted during the cruise by comparing the fluorescence reading of a series of samples placed in the instrument with the measured chlorophyll concentration measured by extracting the samples for laboratory analysis.  Laboratory calibrations of the Turner Designs fluorometer and the HPLC are conducted as needed using known concentrations of purified photosynthetic pigment extracts (measured using a spectophotometer) purchased commercially or isolated from algal cultures.

Logical Consistency Report:  The HydroScat-6 data were processed using IDL software.  A calibrated data file was created using software provided by HOBI Labs, Inc.  Data less than the dark threshold was replaced by -9.9x1035.  The data were despiked, in two passes, with a difference threshold. A moving average was calculated for these channels.  The data were separated into upcast and downcast profiles and then binned to 0.5-m bins.

Completeness Report: Refer to the separate sections of Logical Consistency, Methodology, and Process Steps for descriptions of  completeness of the data.

Lineage:

Methodology:

Methodology Type:  Shipboard Deployments and sample and data collection and laboratory analyses..

Methodology Description:  See Methods section pages 1-8.

Process Step:

Process Description:  Calibration of the Hydroscat6 in situ Backscattering Sensor.

Process Date:  19970527

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  HOBI Labs

Contact Address:

Address Type:  mailing and physical address

Address:  55 Penny Lane, Suite 104

City:  Watsonville

State or Province:  California

Postal Code:  95076-6017

Country:  USA

Contact Voice Telephone:  (408) 768-0680

Process Step:

Process Description:  Calibration of the SeaCat CTD.

Process Date:  19951012

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  Sea-Bird Electronics, Inc.

Contact Address:

Address Type:  mailing and physical address

Address:  1808 136th Place NE

City:  Bellevue

State or Province:  Washington

Postal Code:  98005

Country:  USA

Contact Voice Telephone: (206) 643-9866

Process Step:

Process Description:  Calibration of the AC-9 Spectral Absorption and Attenuation meter.

Process Date:  19960821

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  WET Labs, Inc.

Contact Address:

Address Type:  mailing and physical address

Address:  620 Applegate Street

City:  Philomath

State or Province:  Oregon

Postal Code:  97370

Country:  USA

Contact Voice Telephone:  (541) 929-5650

Process Step:

Process Description:  Calibration of the Microtops II Sun Photometer

Process Date:  19980129

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization: NASA/GSFC Code 970.2

Contact Person: Tom Riley, SIMBIOS Project

Contact Address:

Address Type: mailing and physical address

Address: Building 28, Room W108

City: Greenbelt

State or Province: Maryland

Postal Code:  20771

Country:  USA

Contact Voice Telephone:

Process Step:

Process Description:  Calibration of Satlantic SPMR (SeaWiFS Profiling Multichannel Radiometer).  See also Appendix C.

Process Date: 19980310

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  Satlantic, Inc.

Contact Address:

Address Type:  mailing and physical address

Address:  3295 Barrington Street

City:  Halifax

State or Province:  Nova Scotia

Postal Code:  B3K 5X8

Country:  Canada

Contact Voice Telephone:  (902) 492-4780

Process Step:

Process Description:  Calibration of SIMBAD

Process Date: 19980301

Process Contact:

Contact Information:

Contact Organization Primary:

Contact Organization: NASA/GSFC Code 970.2

Contact Person: Tom Riley, SIMBIOS Project

Contact Address:

Address Type: mailing and physical address

Address: Building 28, Room W108

City: Greenbelt

State or Province: Maryland

Postal Code:  20771

Country:  USA

Contact Voice Telephone:

Spatial Data Organization Information:

Indirect Spatial Reference:  Nantucket Shoals, Gulf of Maine, Maine, USA.

Distribution Information:

Distributor:

Contact Information:

Contact Organization Primary:

Contact Organization: NOAA Coastal Services Center

Contact Address:

Address Type:  mailing and physical address

Address:  2234 Hobson Avenue

City:  Charleston

State or Province:  South Carolina

Postal Code:  29405-2413

Country:  USA

Contact Voice Telephone:  (843) 740-1200

Contact Facsimile Telephone:  (843) 740-1224

Contact Electronic Mail Address:  csc@csc.noaa.gov

Hours of Service:  8AM-5PM, M-F

Resource Description:  NOAA CSC 99043-PUB:  JUL98NAN Cruise Report

Distribution Liability:  None

Custom Order Process:  Contact the distributor for a paper copy of the technical report, or the data can be accessed on-line at http://www.csc.noaa.gov/crs/cruises/jul98nan/.

Metadata Reference Information:

Metadata Date:  19990312

Metadata Review Date:  19990312

Metadata Contact:

Contact Information:

Contact Organization Primary:

Contact Organization:  NOAA, Coastal Services Center

Contact Position:  Metadata Specialist

Contact Address:

Address Type:  mailing and physical address

Address:  2234 Hobson Avenue

City:  Charleston

State or Province:  South Carolina

Postal Code:  29405-2413

Country:  USA

Contact Voice Telephone:  (843) 740-1200

Contact Facsimile Telephone:  (843) 740-1224

Contact Electronic Mail Address:  crs@csc.noaa.gov

Hours of Service:  8AM-5PM, M-F

Metadata Standard Name:  Content Standard for National Biological Information Infrastructure Metadata.

Metadata Standard Version:  December 1995

# VIII.  Appendix A - Water Column Profile Data Figures

# IX.  Appendix B - Example Header Information

Example Binned Surface Reference File Header

SATHDR CREATIONDATE 02-Oct-1998

SATHDR PROSOFT V5.0d

SATHDR INVESTIGATOR Ajit_Subramaniam

SATHDR AFFILIATION Chesapeake_Biological_Laboratory

SATHDR STATIONID StaA1

SATHDR CRUISEID JUL98NAN

SATHDR CAST CASTA

SATHDR DATE 187

SATHDR YEAR 1998

SATHDR RATE  6

SATHDR ZONE  0.0

SATHDR LATITUDE 42.895000

SATHDR LONGITUDE 70.372917

SATHDR STARTTIME 12.546389

SATHDR PROCLVL 3A

SATHDR NUM_ANCIL 17

SATHDR PROSTAT OFF

SATHDR REFSTAT WET

SATHDR REFCAL Ref024c.cal

SATHDR REFID SATREF0024

SATHDR REFDARK CAL

SATHDR REF_LS_DIST 0.000

SATHDR REF_ES_DIST 0.000

SATHDR REF_EU_DIST 0.000

SATHDR LSWAVES 339.0 380.1 412.4 442.8 489.9 509.7 519.9 554.4 565.3 619.3 664.4 669.8 682.7

SATHDR EVWAVES  0.0

SATHDR ESWAVES 339.2 380.1 411.2 442.0 490.4 510.5 519.3 554.9 564.9 619.2 665.4 670.1 682.2

Example Binned Profiler File Header

SATHDR CREATIONDATE 14-Sep-1998

SATHDR PROSOFT V5.0d

SATHDR INVESTIGATOR Ajit_Subramaniam

SATHDR AFFILIATION Chesapeake_Biological_Laboratory

SATHDR STATIONID StaC1b

SATHDR CRUISEID JUL98NAN

SATHDR CAST CASTB

SATHDR DATE 189

SATHDR YEAR 1998

SATHDR RATE  6

SATHDR ZONE  0.0

SATHDR LATITUDE 41.249717

SATHDR LONGITUDE 70.417133

SATHDR STARTTIME 15.942315

SATHDR PROCLVL 3C

SATHDR NUM_ANCIL 17

SATHDR REFCORPRO OFF

SATHDR PROSTAT WET

SATHDR PROCAL Pro024c.cal

SATHDR PROID SATPRO0024

SATHDR PRODARK CAL

SATHDR PRO_LU_DIST 1.142

SATHDR PRO_EU_DIST 0.000

SATHDR EUWAVES  0.0

SATHDR LUWAVES 379.8 411.2 443.3 490.3 510.4 519.2 554.3 565.1 619.1 665.6 670.0 683.8

SATHDR EDWAVES 339.3 379.8 412.4 443.2 489.6 509.4 520.8 554.9 565.5 619.1 665.3 669.7 683.9

SATHDR REFSTAT OFF

SATHDR COMMENTS profiler only

SATHDR MAX_DEPTH  -20

SATHDR START_TIME  15.942361[LOCAL]

SATHDR START_DATE 1998JUL08

SATHDR END_TIME  16.042970[LOCAL]

SATHDR END_DATE 1998JUL08

Example Lwn File Header

SATHDR CREATIONDATE 02-Oct-1998

SATHDR PROSOFT V5.0d

SATHDR INVESTIGATOR Ajit_Subramaniam

SATHDR AFFILIATION Chesapeake_Biological_Laboratory

SATHDR CRUISEID JUL98NAN

SATHDR STATIONID StaC1b

SATHDR CAST CASTB

SATHDR DATE 189.000000

SATHDR STARTTIME 15.942361

SATHDR LAT 41.249717

SATHDR LON 70.417133

SATHDR REF_ES_DIST 0.000000

SATHDR REF_LS_DIST 0.000000

SATHDR PROCAL Pro024c.cal

SATHDR REFCAL

SATHDR COMMENTS profiler only

SATHDR LWN_ALGORITHM I

SATHDR NUM_K_BINS 4.000000

# rho(lambda,theta) = 0.021

# refractive index seawater = 1.3450

# surface albedo = 0.043

# LWN processing using CASE I algorithms

# First Profile BIN depth =     0.0

# REFERENCE data not available - using Ed to estimate Es

# Ls sensor data not avaliable - Ls values set to zero

# Warning Kd565.5nm (0.3079/m) is less than pure water (0.3088/m)

# Warning Kd669.7nm (0.4628/m) is less than pure water (0.4653/m)

#AP = Lwn computed using Austin-Petzold/Morel method

#

# X.  Appendix C - Calibration Certificates

SPMR Calibration File

#

#SATPRO0024

#

# SPMR 024 / Maryland 97129

#

# cal file valid 10 March, 1998

#

INSTRUMENT SATPRO '' 6 AS 0 NONE

SN 0024 '' 4 AI 0 COUNT

RATE 6 'Hz' 0 BU 0 NONE

#LU sensor OCR-1000 S/N 036 calibrated for LO GAIN in IN SEAWATER

# by HEIKE on TUE MAR 10, 1998 at 10:24:27

# LO GAIN calibration LAMP: F497 TARGET: T13172G at DIST: 130.0cm

#LU sensor OCR-1000 S/N 036 calibrated for HI GAIN in IN SEAWATER

# by HEIKE on TUE MAR 10, 1998 at 10:42:01

# HI GAIN calibration LAMP: F497 TARGET: T13172G at DIST: 230.0cm

LU NONE '' 3 BU 0 NONE

LU 411.2 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389636.0 1.3617e-006  1.76

8389468.6 1.3668e-007  1.76

LU 379.8 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389363.6 1.4029e-006  1.77

8388788.0 1.3744e-007  1.77

LU 443.3 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389726.2 1.4399e-006  1.75

8389643.3 1.3768e-007  1.75

LU 519.2 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389625.4 1.4671e-006  1.74

8389475.2 1.3896e-007  1.74

LU 490.3 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389544.6 1.3311e-006  1.75

8389582.8 1.3766e-007  1.75

LU 510.4 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389508.8 1.2653e-006  1.74

8389384.8 1.3844e-007  1.74

LU 565.1 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389568.8 1.3868e-006  1.74

8389441.8 1.3848e-007  1.74

LU 554.3 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388710.5 1.3581e-006  1.74

8388586.7 1.3584e-007  1.74

LU 619.1 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388528.9 7.0767e-007  1.73

8389121.1 3.4410e-008  1.73

LU 665.6 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388720.5 7.0357e-007  1.73

8388519.4 3.3878e-008  1.73

LU 683.8 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388752.8 7.1725e-007  1.73

8391485.1 3.4738e-008  1.73

LU 670.0 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388669.4 7.2828e-007  1.73

8390558.6 3.4145e-008  1.73

LU DARK 'COUNTS' 3 BU 0 COUNT

#ED sensor OCI-1000 S/N 051 calibrated for LO GAIN in IN SEAWATER

# by HEIKE on FRI MAR 6, 1998 at 19:45:38

# LO GAIN calibration LAMP: F497 at DIST: 50.0cm

#ED sensor OCI-1000 S/N 051 calibrated for HI GAIN in IN SEAWATER

# by HEIKE on FRI MAR 6, 1998 at 19:50:44

# HI GAIN calibration LAMP: F497 at DIST: 110.0cm

ED 339.3 'uW/cm^2/nm' 3 BU 2 OPTIC1

8387669.8 2.8436e-005  1.36

8386995.9 1.3937e-005  1.36

ED 412.4 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388672.0 2.3524e-005  1.52

8388345.8 1.5840e-006  1.52

ED 379.8 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389084.6 2.5118e-005  1.42

8386149.5 6.1361e-006  1.42

ED 443.2 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388945.4 2.6061e-005  1.44

8388731.6 1.6618e-006  1.44

ED 520.8 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389160.5 2.6408e-005  1.42

8389041.4 1.6880e-006  1.42

ED 489.6 'uW/cm^2/nm' 3 BU 2 OPTIC1

8390161.4 2.6146e-005  1.43

8390494.3 1.6845e-006  1.43

ED 509.4 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389329.6 2.4565e-005  1.42

8389726.3 1.6821e-006  1.42

ED 565.5 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389057.1 2.7728e-005  1.39

8391088.2 1.7589e-006  1.39

ED 554.9 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388831.4 2.3869e-005  1.39

8388739.6 1.6988e-006  1.39

ED 619.1 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388581.8 1.6543e-005  1.40

8389299.4 4.2972e-007  1.40

ED 665.3 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388542.6 1.7915e-005  1.40

8388166.6 4.2322e-007  1.40

ED 683.9 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388439.4 1.7025e-005  1.38

8389407.1 4.3603e-007  1.38

ED 669.7 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388505.4 1.5738e-005  1.40

8388976.0 4.3234e-007  1.40

ED DARK 'COUNTS' 3 BU 0 COUNT

#

# Ancillary sensors

#

#Tilts calibrated Feb 24, 1998 by Darrell and Heike

# Tiltx Coeff

TILT X 'deg' 2 BU 1 POLYU

-8.4506634e+001   2.5696849e-003   2.7117824e-011

# Tilty Coeff

TILT Y 'deg' 2 BU 1 POLYU

-8.1487576e+001   2.5188603e-003   -3.2515903e-010

#

# SATCAL THERMAL (SATPRO0024)

# Bath run: SATCAL AT=10:48:10 1998-03-07, Jason MacFadyen

# Analysis: SATANL AT=09:30:55 1998-03-08, Jason MacFadyen

#

T i 'C' 2 BU 1 POLYU

-8.859455921e+000 1.170369024e-003 -1.823527569e-008 4.259527991e-013 -6.102609730e-018 5.228366850e-023 -1.614164093e-028

T r 'C' 2 BU 1 POLYU

-9.658634982e+000 1.273903354e-003 -2.592815753e-008 7.528743094e-013 -1.371316865e-017 1.433271313e-022 -5.961479868e-028

T w 'C' 2 BU 1 POLYU

-1.484749725e+001 1.382353307e-003 -2.491509161e-008 7.105451410e-013 -1.307112921e-017 1.427792586e-022 -6.131730331e-028

# Viatran 500 psi transducer 2224AU2AAE10 s/n 251396

Pres none 'm' 2 BU 1 POLYF

0.010853711  32685

#

#COND024B.cal

#Ocean Sensors CT probe s/n 379 calibrated March 5, 1998 by Darrell

#

COND NONE 'mmho/cm' 2 BU 1 POLYU

3.5010449236e+0 7.5044785486e-4 3.9190291424e-8 -1.3476666126e-12 2.2512104196e-17 -1.3920027126e-22

# USE IF NO CONDUCTIVITY PROBE

#Aux2 none '' 2 BU 0 NONE

Aux3 none '' 2 BU 0 NONE

FRAME none '' 1 BU 0 COUNT

#

#  padding for optional Paroscientific sensor

#

PAD none '' 17 AS 0 NONE

SMSR Calibration File

#

# SATREF0024

#

# SMSR 024 / Maryland 97129

#

# cal file valid 9 March, 1998

#

INSTRUMENT SATREF '' 6 AS 0 NONE

SN 0024 '' 4 AI 0 COUNT

RATE 6 'Hz' 0 BU 0 NONE

#LS sensor OCR-1000 S/N 037 calibrated for LO GAIN in IN SEAWATER

# by HEIKE on MON FEB 23, 1998 at 19:33:16

# LO GAIN calibration LAMP: F497 TARGET: T13172G at DIST: 130.0cm

#LS sensor OCR-1000 S/N 037 calibrated for HI GAIN in IN SEAWATER

# by HEIKE on MON FEB 23, 1998 at 19:38:36

# HI GAIN calibration LAMP: F497 TARGET: T13172G at DIST: 230.0cm

LS 339.0 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8390314.3 1.3631e-006  1.78

8391280.3 1.6356e-007  1.78

LS 412.4 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388756.1 1.4487e-006  1.76

8388848.7 1.4219e-007  1.76

LS 380.1 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389727.9 1.4641e-006  1.77

8391490.5 1.4272e-007  1.77

LS 442.8 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389093.0 1.3754e-006  1.75

8388986.8 1.4204e-007  1.75

LS 519.9 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389446.4 1.5161e-006  1.74

8389414.6 1.3511e-007  1.74

LS 489.9 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388784.7 1.3648e-006  1.75

8388697.2 1.3792e-007  1.75

LS 509.7 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8389320.3 1.3816e-006  1.74

8389271.1 1.4086e-007  1.74

LS 565.3 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388786.8 1.3988e-006  1.74

8388719.6 1.4266e-007  1.74

LS 554.4 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388735.4 1.4423e-006  1.74

8388765.5 1.4008e-007  1.74

LS 619.3 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388814.3 7.0402e-007  1.73

8388800.9 3.3583e-008  1.73

LS 664.4 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388423.7 7.2085e-007  1.73

8388414.0 3.4940e-008  1.73

LS 682.7 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388815.6 7.2432e-007  1.73

8388952.3 3.5161e-008  1.73

LS 669.8 'uW/cm^2/nm/sr' 3 BU 2 OPTIC1

8388173.8 7.2753e-007  1.73

8388205.1 3.4713e-008  1.73

LS DARK 'COUNTS' 3 BU 0 COUNT

#ES sensor OCI-1000 S/N 052 calibrated for LO GAIN in IN AIR

# by HEIKE on MON MAR 9, 1998 at 17:56:47

# LO GAIN calibration LAMP: F497 at DIST: 50.0cm

#ES sensor OCI-1000 S/N 052 calibrated for HI GAIN in IN AIR

# by HEIKE on MON MAR 9, 1998 at 18:01:16

# HI GAIN calibration LAMP: F497 at DIST: 50.0cm

ES 339.2 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388787.8 3.4965e-005  1.00

8385732.1 4.5416e-006  1.00

ES 411.2 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389313.1 3.5822e-005  1.00

8389278.1 2.4194e-006  1.00

ES 380.1 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389362.7 3.4385e-005  1.00

8387451.0 2.5411e-006  1.00

ES 442.0 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389889.9 3.7767e-005  1.00

8389839.3 2.4958e-006  1.00

ES 519.3 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389583.3 3.6962e-005  1.00

8389524.9 2.3948e-006  1.00

ES 490.4 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389367.0 3.7985e-005  1.00

8389385.9 2.4542e-006  1.00

ES 510.5 'uW/cm^2/nm' 3 BU 2 OPTIC1

8389469.7 3.7817e-005  1.00

8389421.5 2.4185e-006  1.00

ES 564.9 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388961.8 3.9324e-005  1.00

8388960.6 2.4324e-006  1.00

ES 554.9 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388663.1 3.5851e-005  1.00

8388624.3 2.4201e-006  1.00

ES 619.2 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388150.4 3.4395e-005  1.00

8388140.2 2.4216e-006  1.00

ES 665.4 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388536.1 3.8642e-005  1.00

8388511.9 2.4044e-006  1.00

ES 682.2 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388205.2 3.8207e-005  1.00

8388313.2 2.4679e-006  1.00

ES 670.1 'uW/cm^2/nm' 3 BU 2 OPTIC1

8388033.0 3.8312e-005  1.00

8388036.1 2.4081e-006  1.00

ES DARK 'COUNTS' 3 BU 0 COUNT

#

# Ancillary sensors

#

#Tilts calibrated Feb 23 1998 by Darrell and Heike

# Tiltx Coeff

TILT X 'deg' 2 BU 1 POLYU

8.1434267e+001   -2.4903728e-003   9.0977616e-011

# Tilty Coeff

TILT Y 'deg' 2 BU 1 POLYU

8.2598038e+001   -2.4998651e-003   -2.8631035e-010

#

# SATCAL THERMAL (SATREF0024)

# Bath run: SATCAL AT=17:01:17 1998-02-26, Vern, Darrell & Jason

# Analysis: SATANL AT=09:50:04 1998-02-27, Jason & Darrell

#

T i 'C' 2 BU 1 POLYU

-8.966624303e+0 1.178159189e-3 -1.859307444e-8 4.448179627e-13 -6.638994363e-18 5.971746755e-23 -2.000428162e-28

T r 'C' 2 BU 1 POLYU

-9.574832669e+0 1.214497515e-3 -1.949485280e-8 4.360412326e-13 -5.706579775e-18 4.289859677e-23 -1.016340157e-28

Aux2 none '' 2 BU 0 NONE

Aux3 none '' 2 BU 0 NONE

Aux4 none '' 2 BU 0 NONE

Aux5 none '' 2 BU 0 NONE

FRAME none '' 1 BU 0 COUNT

#

# padding for optional Paroscientific sensor

#

PAD none '' 17 AS 0 NONE