LZSS De-Commutation Operations Tool

Verification Procedure

For The GOES-R

Space Environment In-Situ Suite (SEISS)

SEISS-XX-XXX Rev -		10/13/2016

Prepared for:

National Aeronautics & Space Administration

Goddard Space Flight Center

Greenbelt, MD 20771

Under Contract Number NNG06HX01C

This proposal or document includes sensitive information that NASA shall not disclose outside the Agency and its service providers that support management activities and administrative functions. To gain access to this sensitive information, a service provider's contract must contain the clause at NFS 1852.237-72, Access to Sensitive Information. Consistent with this clause, the service provider shall not duplicate, use, or disclose the information in whole or in part for any purpose other than to perform the services specified in its contract. This restriction does not limit the Government's right to use this information if it is obtained from another source without restriction.

Assurance Technology Corporation

84 South Street • Carlisle. MA 01741 • (978) 369-8848

RECORD OF CHANGES

| Revision Letter | Date | Title or Brief Description | Entered By |
| --- | --- | --- | --- |
| - | 7/5/2016 | LZSS De-Commutation Operations Tool Verification Procedure | B. Brown |

Test Operator	______________________

Date		______________________

# LZSS De-Commutation Tool Validation Procedure

- Launch the LZSS Batch Processing tool by running lzss_batch_processor.exe.

- Set the start time to 2015, 4, 14, 8, 39.

- Click on the ‘Other’ Radio button.

- Set the end time to 0, 0, 35.

- Click the done button.

- Validate that the start time that is printed is ‘2015-04-14 08:39:00’. 		P	F

- Validate that the end time that is printed is ‘2015-04-14 09:14:00’.		P	F

- When the dialog box opens, select the location of the test files.

- When the program has finished running the command prompt will read ‘Processing Complete’.

- Close the Batch Processing Tool.

- Navigate to ~/dist/output/ and verify that there a new file has been created with a name that corresponds to the time that the Batch Processing Tool was run. Write down the name of the file here                                                                      _________________________

- Open command prompt

- Navigate to the source code directory using the cd command

- Run the command /GOES/egse/EGSEMain_NoDAQ.exe –p XXXXX where XXXXX is the full path to the file created in step 11.

- Wait for the command prompt says ‘Command-line telemetry playback complted!’

- Open the save location of the EGSE Test Logs (default is C:\GOES\testLogs’)

- Validate that new files have been created with time stamps that correspond to the run time of the EGSE. Record the DPU Telemetry File name here		________________________

- Launch the Log Renamer tool by running log_renamer.exe.

- Click the ‘Select File’ button.

- When prompted, select the DPU Telemetry file that was recorded in Step 17.

- Record the new timestamp that was reported by the tool.  	________________________

- Navigate to C:\GOES\ProcessedLogs

- Verify that all the files in the directory ~/ProcessedLogs have a time stamp that matches what is recorded in Step 21.				P	F

- Verify that the renamed files have a name that corresponds to the date and time 
given in Step 6.							________________________

- Open the new DPU Telemetry log file.

- Verify that the header for the first second of telemetry in the DPU log file matches the date and time given in Step 6.						________________________