# SUVI Level 0 Data Reader User’s Manual

First Release 2019/03/22

Contents

Description	1

Output Data Formats	2

Cautionary Advice	2

SUVI FITS Files	2

FITS Primary Header	2

FITS Primary Header Fill	4

FITS Primary Image	4

FITS Despike Header #1	5

FITS Despike Data #1	5

FITS Despike Header #2	5

FITS Despike Data #2	6

SUVI ISP Files	6

User Interface	6

Configuration File	7

Advanced	11

Build Instructions	11

Acronyms	11

# Description

The SUVI Level 0 Data Reader is a file format converter that enables a user to quickly and easily convert unprocessed raw binary Level 0 (L0) data files pertaining to the GOES Solar UltraViolet Imager (SUVI) instrument into FITS files that are human- and machine-intelligible, machine-independent, self-describing and easy to ingest into downstream data processing and analysis tools.

The SUVI Level 0 data stream consists of binary data packets conforming to the Consultative Committee for Space Data Systems (CCSDS) standard and is difficult to decode without extensive knowledge about the internal commutation logic of the instrument.  The packets encode two major output data products from SUVI: raw solar images and housekeeping telemetry such as temperatures and voltages.

The SUVI Level 0 Data Reader requires the user to establish access to SUVI Level 0 data.  The Level 0 data, consisting of raw CCSDS packets grouped into 2-minute sections and placed into a HDF5-formatted NetCDF-compatible file for each satellite containing an active SUVI instrument, is released for public access by NOAA through a process that is not described in this document.

Before using the SUVI Level 0 Data Reader, users are encouraged to become familiar with the basic function of SUVI, at least through the overview provided in the GOES-R Data Book available on the www.goes-r.gov website.

# Output Data Formats

In the parlance of processing levels of NOAA data sets, the output of the SUVI Level 0 Data Reader is Level 1a data, consisting of telemetry that has been extracted and formatted into time-sequenced datasets for easier processing, then optionally reprocessed through a software "despike" algorithm to remove radiation spikes and other image artifacts.

## Cautionary Advice

Users should not expect the SUVI Level 1a solar images to be in the proper solar spin orientation (i.e., top of the L1a image is likely not to be perpendicular to the solar spin axis) or properly scaled, even between adjacent images.  Level 1b ground processing would normally rotate, yaw flip, and rescale these images based on the changing satellites' positions and orientations.  These navigation and calibration operations are outside the scope of the SUVI Level 0 Data Reader.

Users are further cautioned that while SUVI typically runs a fixed sequence of images on each day, there are occasional requests by the GOES Project's Science Team for out-of-sequence and off-solar images to be taken which may interrupt the standard sequences.

Solar images may be partially or completely obscured by occasional Lunar and Earth eclipses as well.

## SUVI FITS Files

Solar images are generated by SUVI approximately every 10 seconds and are encoded as self-describing Flexible Image Transport System (FITS) files.  Each FITS file contains a single image of either the Sun, periodic "dark" frames used to determine the on-going health of the camera, or occasionally commanded "off-pointed" images as determined by the GOES science team.

If enabled via the DESPIKE parameter, the raw solar images are reprocessed by the SUVI_L0 software through a process to "despike" radiation events and remove other image artifacts.  While the primary data section within the FITS file contains this despiked image, each pixel which was modified in the Level-0-to-Level-1a process is saved in a pair of extended FITS data sections following the image which can be used to restore the original L0 image if desired.  Conversely, if raw L0 images are requested, the extended FITS data sections contain information to allow the user to despike the image if desired.

## FITS Primary Header

Each FITS file contains ancillary data encoded in its primary header as described by the keywords below.  Additional SUVI housekeeping items are also contained within the FITS header beyond those listed, and the details of those items highlighted in green italics can be found within the SUVI Telemetry and Command Handbook (T&CH).

Table 1: FITS Primary Header keywords

| Keyword | Description |
| --- | --- |
| SIMPLE | Simple FITS format, always defined as 'T' |
| BITPIX | Number of bits per pixel, always 16 |
| NAXIS | Number of axis, always 2 |
| NAXIS1 | Number of columns, typically 1330 |
| NAXIS2 | Number of rows, typically 1292 |
| EXTEND | Allows FITS extensions, always defined as 'T' |
| FILE_CREATION_TIME | UTC time when file was created, uses the cdate format 'DOW Mmm DD HH:MM:SS YYYY'. For example: 'Fri Oct 26 23:53:10 2018' |
| VERSION_OF_FITS_HEADER | Version of FITS header, currently 2 (see notes in fields below for keywords added/modified/removed after version 2) |
| IMG_TIME | End of CCD integration time in 'YYYY-MM-DD HH:MM:SS.FFF' format. For example: '2018-03-01 02:28:42.854''. |
| SHT_EXP_TIME | Measured time in seconds of the shutter exposure |
| CMD_EXP_TIME | Commanded exposure time in seconds |
| EXP_TIME | Copy of SHT_EXP_TIME |
| DATE_OBS | Start of image integration time, defined as IMG_TIME - EXP_TIME |
| IMAGETYPE | Trigger reason for this image, most are 'SEQUENCE' |
| FRAMETYPE | 'LIGHT' for solar images, 'DARK' for non-shutter calibration images |
| FW1POS | Encoder position of Filter Wheel 1, see FW1NAME for human-readable name when possible, note that encoder values may be different for the same FW1NAME across different flight models |
| FW1STATUS | Value of 0 to 4 if filter wheel 1 is centered at one of the 5 filters, or -1 if between filters |
| FW1NAME | Name of the filter if filter wheel 1 is centered at one of the 5 filters ('OPEN', 'THIN_AL', 'THIN_ZR', 'THICK_ZR', or 'THICK_AL'), or '???' if between filters |
| FW2POS | Encoder position of Filter Wheel 2, see FW2NAME for human-readable name when possible, note that encoder values may be different for the same FW2NAME across different flight models |
| FW2STATUS | Value of 0 to 4 if filter wheel 2 is centered at one of the 5 filters, or -1 if between filters |
| FW2NAME | Name of the filter if filter wheel 2 is centered at one of the 5 filters ('GLASS', 'OPEN', 'THIN_AL', 'THIN_ZR', or 'THICK_AL'), or '???' if between filters |
| ASENC | Encoder position of the aperture selector, see ASNAME for human-readable name when possible, note that encoder values may be different for the same ASNAME across different flight models |
| ASSTATUS | Value of 0 to 5 if aperture selector is positioned at one of the 6 defined positions, or -1 if between positions |
| ASNAME | Name of the aperture if aperture selector is positioned at one of the 6 defined positions ('94A', '131A', '171A', '195A', '284A', '304A'), or '???' if between positions |
| WAVELNTH | Integer value of ASNAME, or -1 if between positions |
| RDOUTPORT | Value from SUV_CEB_RDOUT_PORT |
| GTYERR | Value from SUV_GT_OS_ERR_Y_NR |
| GTZERR | Value from SUV_GT_OS_ERR_Z_NR |
| YAW_FLIP | Value as follows: 0=NORMAL, 1=NEITHER, 2=INVERTED |
| CCDTEMP1 | Value from SUV_CEB_MUX_T_CCD_1 |
| CCDTEMP2 | Value from SUV_CEB_MUX_T_CCD_2 |
| LAST_BAKEOUT_TIME | Time when the last on-orbit bake-out for this specific flight model ended |
| AMPNAME | CCD readout amplifier ('LEFT', 'RIGHT', '???') |
| EXPTYPE | 'SHORT' if CMD_EXP_TIME is less than 1 second, otherwise 'LONG' |
| STCOL | Value from SUV_CA_FRM_SCOL |
| ENCOL | Value from SUV_CA_FRM_ECOL |
| STROW | Value from SUV_CA_FRM_SROW |
| ENROW | Value from SUV_CA_FRM_EROW |
| SUMM | Human-readable summing value from SUV_CA_FRM_SUMMD ("DEFAULT', '1x1', '2x2', '4x4', '8x8', '???') |
| FM | Flight model, 1=GOES-16, 2=GOES-17, etc. |
| STATION | Station flag from spacecraft, 0=Normal |
| ECLIPSE | Eclipse flag from spacecraft, 0=No eclipse, 1=Solar view partially or completely eclipsed |
| CALIBRATION | Calibration flag from spacecraft, 0=Normal, 1=In calibration |
| BAKEOUT | SUVI in bakeout, 0=No bakeout, 1=bakeout |
| SC_MANUVER | Spacecraft is in a manuver, 0=Not moving, 1=Moving |
| EXIS_OFFPOINT | Solar platform is off-pointed, 0=Normal position, 1=Off-pointed |
| SUV_ISP_HDR_ISN to SUV_CA_GT_OS_ERR_Z_NR | 441 items from SUVI housekeeping data, see SUVI T&CH |
| DESPIKE | Number of despiked pixels, 0=No secondary data follows image |
| DESPIKE_PARAMS | Parameter values used for despike, first parameter is L1a if despiked, or L0 if not depsiked |
| SEL_CNT | Number of single-event latchups detected in image |
| END | End of primary header |

## FITS Primary Header Fill

After the END keyword, the remainder of the header is filled with ASCII code 0x20 (space) up to the next 2880-byte boundary.  For SUVI FITS version 2, the primary header plus its fill uses the first 0x9D80 (40320) bytes of each FITS file.

## FITS Primary Image

If enabled using the parameter DESPIKE=YES, the primary image contains the despiked version of the Level 0 data, where the first pixel is the upper left of the CCD, 2-bytes long (from BITPIX), in big endian (high byte first).  This continues across each column of the top row until NAXIS1 pixels are output, then starts on the left side of row 2 until NAXIS2 rows are output.  The total bytes for the image is BITPIX * NAXIS1 * NAXIS2.  Once all pixels are output, the remainder of the primary image area is filled with 0x00 (NULL) up to the next 2880-byte boundary.

Note that if YAWFLIP is non-zero, the orientation of this solar image will be rotated or inverted from images where YAWFLIP is zero.  Also note that images where ECLIPSE is non-zero may have a solar image which is partially or completely dark.

If the parameter DESPIKE=NO, the primary image contains the original Level 0 image in the same orientation as described above followed by NULL-fill.

## FITS Despike Header #1

If the value of the FITS DESPIKE keyword from Table #1 is non-zero, two secondary headers with data arrays will follow the primary image.  Otherwise, the FITS file will end after the primary image NULL-fill.

The first extended header and data array contains the locations where pixels are different from the original L0 image to the L1a despiked image.  The following keywords are present in Despike Header #1, followed by ASCII code 0x20 (space) up to the next 2880-byte boundary.

Table 2: FITS Despike Header #1 keywords

| Keyword | Description |
| --- | --- |
| XTENSION | Always 'BINTABLE' |
| BITPIX | Number of bits per byte, always 8 |
| NAXIS | Number of axis, always 2 |
| NAXIS1 | Number of columns, always 4 (4 bytes per entry) |
| NAXIS2 | Number of rows, equal to DESPIKE in the Primary Header |
| PCOUNT | Size of special data area, always 0 |
| GCOUNT | Number of groups, always 1 |
| TFIELDS | Number of fields in each row, always 1 |
| TTYPE1 | Label of first field type, always 'Address' |
| TFORM1 | Format of first field type, always '1J ' (big-endian 4-byte integer) |
| EXTNAME | Name of extension table, always 'SPIKELOCS' |
| END | End of secondary header |

## FITS Despike Data #1

Each 4-bytes for NAXIS2 items is the big-endian location offset in the primary image where a pixel value in the despiked image is different from the original L0 image.  These locations are in the same order as the primary image starting in the upper left, then across columns of the first row, then down columns of subsequent rows.  Once all locations are output, the remainder of the Despike Data #1 area is filled with 0x00 (NULL) up to the next 2880-byte boundary.

## FITS Despike Header #2

The second extended header and data array contains the values corresponding to each despiked location from Despike Data #1.  If DESPIKE=YES (base image is L1a), these are the original L0 values; if DESPIKE=NO (base image is L0), these are the new L1a despiked values.  The following keywords are present in Despike Header #2, followed by ASCII code 0x20 (space) up to the next 2880-byte boundary.

Table 3: FITS Despike Header #2 keywords

| Keyword | Description |
| --- | --- |
| XTENSION | Always 'BINTABLE' |
| BITPIX | Number of bits per byte, always 8 |
| NAXIS | Number of axis, always 2 |
| NAXIS1 | Number of columns, always 2 (2 bytes per entry) |
| NAXIS2 | Number of rows, equal to DESPIKE in the Primary Header |
| PCOUNT | Size of special data area, always 0 |
| GCOUNT | Number of groups, always 1 |
| TFIELDS | Number of fields in each row, always 1 |
| TTYPE1 | Label of first field type, always 'Old_vals' (if the base image is L1a), or 'New_vals" (if the base image is L0) |
| TFORM1 | Format of first field type, always '1I ' (big-endian 2-byte integer) |
| EXTNAME | Name of extension table, always 'OLDVALS' (if the base image is L1a), or 'NEWVALS" (if the base image is L0) |
| END | End of secondary header |

## FITS Despike Data #2

Each 2-bytes for NAXIS2 items is the big-endian 16-bit value as the old L0 value or new L1a value based on the DESPIKE parameter as described in the FITS Despike Header #2 above.  Each value corresponds to the matching location from Despike Data #1.  Once all values are output, the remainder of the Despike Data #2 area is filled with 0x00 (NULL) up to the next 2880-byte boundary.

# SUVI ISP Files

If the ISP_CSV parameter is set to YES, Image Status Packet (ISP) information will be output into a spreadsheet file in Comma-Separated Value (CSV) format with one file for each UTC day.  Each column of this spreadsheet file contains most of the non-fixed information from Table 1 in each FITS file found in the output directory for that satellite on each UTC day processed.  Note that all FITS files for the UTC days contained within START to END will be included, even if they were not created during the same program run.

The first line of each CSV file will contain a keyword name from Table 1, excluding some fixed keywords such as SIMPLE, NAXIS, and other unchanging keywords.  Each subsequent line will contain the FITS filename as the first entry, then the values for each keyword found in that FITS file.  If all FITS files were extracted for a given day, there would be 8,641 (plus or minus 1) lines in a typical file.

CSV files are human-readable, also natively-readable by most spreadsheet programs such as Excel, LibreOffice Calc, and GnuPlot.  Other software such as Matlab and IDL have packages to read CSV files.

# User Interface

The SUVI Level 0 Reader is a command-line executable program.  It does not have a graphical user interface.  The settings are specified either at the command line or in a configuration file. SUVI_L0 reads SUVI L0 data files from a user-designated input directory, converts them, and writes FITS files into a user-designated output directory.  A log file is generated during each run to facilitate diagnostics and automated runs.

If no parameters outside of the base configuration file described below are required, only the SUVI_L0 command itself is needed on the command-line.  Basic help information can be displayed on the console using SUVI_L0 --help.  The current version information can be displayed on the console using SUVI_L0 --version.

## Configuration File

The base configuration file is named SUVI_configuration.cfg and is in the same directory as the SUVI_L0 executable.  Unless otherwise commanded, this is the configuration file used during a run.  Alternate configuration files may be specified by using the CONFIG=alternate_config parameter on the command line, and these alternate configuration files may be located at any location accessible by your system.  Note for Windows systems, if any part of the path or filename contains spaces, the entire alternate_config must be wrapped in double quotes (e.g. SUVI_L0 CONFIG="C:\got space\c.cfg").

A sample configuration file is provided below with explanations of each parameter.  Any parameter may be placed on the command line instead of the configuration file, where command line parameters will override any value in the configuration file.  For example, SUVI_L0 GOES=17 START=2018/11/01 END=2018/11/02 will use those 3 parameter values instead of the values in the configuration file.

A sample SUVI_configuration.cfg for Windows is shown below.  The actual keywords are bolded; all other lines are comments.  If you are using cygwin instead of Windows/DOS, the directory naming needs to follow a special format to interface with the Windows HDF and NetCDF modules as documented in the INPUT_ROOT_DIRECTORY comments.  Linux/UNIX pathnames are required to be fully specified instead of relative.

Additionally, while the ZIP file provided with the distribution has Input, Output, and Working directories below the main software directory, this is neither required nor particularly recommended.  Input HDF files and output FITS files are quite large, and, when possible, should be moved to secondary, external, or network drives to avoid overloading a user's primary disk with raw and processed data.  Configuration parameters can be changed individually to reflect these alternate locations.  The Working directory requires up to 10 GB of temporary space during a run, although it will be purged at the completion of each successful or failed SUVI_L0 run.

! $Id: SUVI_configuration.cfg 18553 2019-02-05 15:22:50Z baraze $

! Configuration file for SUVI Level-0 reader toolset

! All lines which do not start with a letter (A to Z) in the first column are comments

! All parameters contained herein can be used as command-line arguments too

! If parameters are present on the command-line and in the configuration file, the command-line version will be used

! If any parameter is not present in either the configuration file nor the command-line, an error will occur

! Values of INPUT_ROOT_DIRECTORY, OUTPUT_ROOT_DIRECTORY, WORKING_DIRECTORY, and H5DUMP_EXE are case-sensitive,

!  all other values and all parameter names are case-insensitive (e.g., Input_Type=HdF is valid)

! INPUT_TYPE= one of HDF or APID

! HDF is the default for HDF formatted files from the Harris Level-0 generator, and end with .nc extensions (typical)

! APID are daily files generated by SUVI internal software or SUVI's LZSS software and end with APID numbers *.0x0311 (unusual)

INPUT_TYPE=HDF

! INPUT_ROOT_DIRECTORY= the root path containing the input files

! Unix paths must be absolute and begin with a forward slash (/disk1/data), relative paths are not allowed

! Windows paths must either begin with a drive letter (e.g., C:\data) or a network path with a backslash pair (e.g., \\network\data)

! Cygwin paths must begin with a drive letter and use forward slashes instead of backslashes (e.g., C:/data)

! Note that pathnames containing spaces must be double quoted (e.g., INPUT_ROOT_DIRECTORY="C:\input data")

! Subdirectories below this base are either Gxx_YYYY_DOY or Gxx_YYYYMMDD where Gxx is the GOES satellite number (G16 to G19)

!  followed either by 4_3 digits representing the UTC year/day-of-year, or 8 digits representing the UTC year/month/day

! If both representations are found in the input path, the year/day-of-year will be used and YYYYMMDD will be ignored.

! While the default example may show INPUT_ROOT_DIRECTORY below the main software directory, this is neither required

!  nor particularly recommended.  Input HDF files are quite large, and, when possible, should be moved to secondary,

!  external, or network drives to avoid overloading a user's primary disk with input data.

INPUT_ROOT_DIRECTORY=C:\data\glm\06.0_Tools\LZSS\SUVI_L0\Input

! INPUT_SUBDIRECTORY= one of FLAT or HIERARCHICAL

! If INPUT_SOURCE=HDF then INPUT_SUBDIRECTORY=FLAT means all files are in the INPUT_ROOT_DIRECTORY itself, not in subdirectories

! If INPUT_SOURCE=HDF then INPUT_SUBDIRECTORY=HIERARCHICAL means files for each day are in subdirectories in one of the forms:

!  a) Gxx_YYYY_DOY where Gxx is the satellite (e.g., G16, G17, etc.), YYYY is the year, and DOY is the day of the year (001 to 366)

!     For example, G17_2018_365 is for GOES-17 on 2018/12/31

!  b) Gxx_YYYYMMDD where Gxx is the satellite (e.g., G16, G17, etc.), YYYY is the year, MM is month (01 to 12), DD is day (01 to 31)

!     For example, G16_20181231 is for GOES-16 on 2018/12/31

!  If both representations are found in the input path, the YYYY_DOY will be used and YYYYMMDD will be ignored.

! If INPUT_SOURCE=APID then INPUT_SUBDIRECTORY=FLAT means all daily files for 0x0311 packets are in the subdirectory 0x0311 (typical)

! If INPUT_SOURCE=APID then INPUT_SUBDIRECTORY=HIERARCHICAL means files for 0x0311 are in YYYYMM subdirectories (unusual for SUVI)

!   0x0311/YYYYMM where YYYY is the year and MM is the month of the year (01 to 12)

INPUT_SUBDIRECTORY=HIERARCHICAL

! WORKING_DIRECTORY= path to a empty directory used for temporary storage

! If the directory is not empty (aside from . and ..), the program run will be terminated

! Note that all files in the WORKING_DIRECTORY will be DELETED during the program run

! The SUVI_L0 program requires approximately 10 GB of temporary disk space in the WORKING_DIRECTORY location

WORKING_DIRECTORY=C:\data\glm\06.0_Tools\LZSS\SUVI_L0\Working

! H5DUMP_EXE= fully qualified location of h5dump program

! This value must include the h5dump file itself, not just the pathname

! Windows environments must include the .exe extension

! While an older distribution of the h5dump program is included with the SUVI_L0 software, newer versions may be downloaded

!  from https://portal.hdfgroup.org/display/support/Downloads

H5DUMP_EXE=C:\data\glm\06.0_Tools\LZSS\SUVI_L0\HDF\HDF5-1.8.15-utilities\h5dump.exe

! OUTPUT_ROOT_DIRECTORY= the root path where output files should be sent

! Unix/cygwin paths must be absolute and begin with a forward slash (/disk1/output), relative paths are not allowed

! Windows paths must either begin with a drive letter (e.g., C:\output) or a network path with a backslash pair (e.g., \\network\output)

! Note that pathnames containing spaces must be double quoted (e.g., OUTPUT_ROOT_DIRECTORY="C:\my output")

! Subdirectories below this base are created in the form Gxx/YYYY/MM/DD/Hhh00/, where xx is the sattelite number

!  16 to 19, YYYY/MM/DD is the UTC year/month/day, and Hhh00 is the hour from H0000 to H2300.  FITS files are

!  contained in the matching hour's cubdirectory.

! While the default example may show OUTPUT_ROOT_DIRECTORY below the main software directory, this is neither required

!  nor particularly recommended.  FITS files are quite large, and, when possible, should be moved to secondary,

!  external, or network drives to avoid overloading a user's primary disk with output data.

OUTPUT_ROOT_DIRECTORY=C:\data\glm\06.0_Tools\LZSS\SUVI_L0\Output

! DESPIKE= YES or NO

! If YES, apply a L1a despike algorithm to each FITS image, which includes secondary header information to respike the data

!  to return to the original L0 image.  The despike algorithm removes bright and dark pixel data which is typically caused by

!  radiation hits.  Most images have a few thousand pixels which are cleaned up by this despike algorithm.

! If NO, the original L0 data will appear in the FITS image, and the secondary headers will contain the despike information.

DESPIKE=YES

! ISP_CSV= YES or NO

! If YES, create Image Status Packet (ISP) daily summary files in Comma-Separated Value (CSV) format.  One file per

!  UTC day will be created for all files in the OUTPUT_ROOT_DIRECTORY on days from START to END, including files

!  which were not created on this run.  Aside from the header line, one line will be present in the CSV file for each FITS file

!  with an ending exposure time on that day.  ISP files are created in the ISP subdirectory of the OUTPUT_ROOT_DIRECTORY.

! If NO, no updates to ISP CSV files will occur on this run, although future runs for the same period where ISP_CSV=YES

!  may include FITS files from this run.

ISP_CSV=YES

! GOES= one of 16, 17, 18, 19, etc.

! This specifies which satellite's data to use for input and output, where 16 is GOES-16, etc.

GOES=17

! START= UTC start date in one of the formats listed below

! END= UTC end date in one of the formats listed below

!   Format 1: YYYY/MM/ddThh:mm:ss - year/month/day/T-separator/hour/minute/second in UTC (T is the date/time separator)

!     For example, 2018/12/31t23:34:45 specifies 2018/12/31 at 11:34:45 PM UTC

!     A partial or missing time is acceptable, although certain assumptions will occur:

!     START=2018/11/01 END=2018/11/01 will cover 2018/11/01T00:00:00 to 2018/11/01T23:59:59

!     START=2018/10/02T12 END=2018/10/03T05 will cover 2018/10/02T12:00:00 to 2018/10/03T05:59:59

!     All leading zeros are required for all date and time fields (e.g., 2018/1/5 is invalid)

!   Format 2: YYYY/DOYThh:mm:ss - year/day-of-year/T-separator/hour/minute/second in UTC (T is the date/time separator)

!     For example: 2018/365T23:34:45 specifies 2018/12/31 at 11:34:45 PM UTC

!     Day-of-year is 001 to 365 (non-leap years) or 001 to 366 in leap years, leading zeros are required

!     Missing time fields follow the same rules as Format 1 above

!   For FITS files, only FITS files which are time-tagged within the specified time ranges

!    from (START + 0.000000) to (END + 0.999999) seconds will be output

!   Also for FITS files, input data for at least 30 seconds after the END time is required to ensure the final

!    image has all of its data

!   No more than 33 days can be included in one run between START and END to avoid human-typing errors,

!    including partial days on START and END with 2 minutes of margin on each end (31 full days)

START=2018/12/31t23

END=2019/001t00:29:30

# Advanced

## Build Instructions

There are 2 primary sources of software required for SUVI_L0: (1) the SUVI_L0 executable, and (2) the h5dump program (part of HDF5).  The h5dump software for Windows, DOS, and cygwin are contained within the released distribution, while versions for Linux/Unix can either be downloaded or built from sources as described on the internet at the following location (if this link does not work, perform an internet search for "HDF download"):

HDF5: https://portal.hdfgroup.org/display/support/Downloads

Note that only the h5dump program is required from the HDF5 distribution to run the SUVI_L0 software, and all other HDF5 files may be removed by the end user.  All HDF5 open-source files are contained in this distribution to avoid licensing issues.

As the SUVI_L0 reader software module contains one .cpp file, recompiling the code using command-line tools for DOS, cygwin, Linux, or UNIX is a single-line operation from a command-line window in the source directory.  This example uses the GNU C++ toolset, but other compilers such as mingw, tc++, and c++ can be substituted as needed:

g++ -o SUVI_L0 SUVI_L0.cpp

WIndows-based compilers such as Embarcadero C++ Builder or Microsoft Visual C++ require a bit more effort to create a new "console" or "command-line" output project called SUVI_L0 and attach the SUVI_L0.cpp file to that project.  Once the project is created and saved, you should be able to "make" or "build" the executable.  Note that depending on your compiler's strict warning settings, you may receive warnings about deprecated functions, but these will not affect the software operation.  One error for a non-POSIX module may occur, and is described in the paragraph below.

One non-POSIX compliant module which may need to be downloaded from the internet is the dirent.h module (VC++ requires this, Embarcadero and mingw do not).  Github is one of the recommended sites to download dirent if needed, but an internet search for "dirent download" if this link does not work:

https://github.com/tronkko/dirent

If a manual download of dirent.h is required, you may need to change the #include <dirent.h> line in the SUVI_L0.cpp file to include the path to the downloaded dirent module if the download location is not in your PATH environment variable (e.g. #include "path\dirent.h").

# Acronyms

ADC	Analog to Digital Converter
BGMSB	Background Most Significant Bits
CCD	Charge Coupled Device
CCSDS	Consultative Committee for Space Data Systems
CSV	Comma-Separated Value (format/file)
DF	Data Formatter
DOS	Disk Operating System
FIFO	First In First Out queue
FITS	Flexible Image Transport System
FPAA	Focal Plane Array Assembly
GB	Gigabyte
GNU	GNU's Not Unix!
GOES	Geostationary Operational Environmental Satellite
HDF	Hierarchical Data Format
ID	Identifier
ISP	Image Status Packet
L0	Level Zero
L1a	Level One Alpha
NOAA	National Oceanic and Atmospheric Administration
POSIX	Portable Operating System Interface
RTEP	Real Time Event Processor
SEB	Sensor Electronics Box
SUVI	Solar UltraViolet Imager
UTC	Universal Time Coordinated
VC++	Microsoft Visual C++