TABLE 1

STATION METADATA

 FOR USCRN STATIONS
Station-specific Metadata are collected throughout the stages of selecting, installing and operating a USCRN station.  A definitive list of USCRN Metadata is presented, illustrating the cumulative growth of station metadata, which metadata parameters are added in each step, from Site Survey, Approval, Pre-Installation, Installation, Acceptance, Commissioning, to Operations.  

	METADATA
	SOURCE
	REPOSITORY
	CONTROL

	
	
	CRN

SITES
	WSSRD
	DATA

ADMIN
	NM
	CM
	CI

CCB

	
	
	
	
	
	
	
	

	Involved parties and agencies
	Survey
	X
	X
	X
	X
	X
	X

	Landowner
	Survey
	
	X
	
	
	
	

	USCRN partner
	Survey
	
	X
	
	
	
	

	Contact information
	Survey
	X
	
	
	X
	
	

	Site Survey Review slides (including digital photographs (surface in octants, fish-eye view of sky above station, etc.) LANDSAT imagery, and aerial imagery, layout and exposure, and location of buildings, trees, surrounding topography, etc)
	Survey
	
	X
	
	
	
	

	Vegetation types and condition
	Survey
	
	X
	
	X
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	Decision Paper
	Approval
	
	X
	
	
	
	

	Site License Agreement
	Approval
	
	X
	X
	
	
	

	Site Specific Legislative Authorizations
	Approval
	
	X
	
	
	
	

	Driving instructions to station
	Installation
	X
	
	
	
	
	

	Site Survey Score
	Approval
	X
	X
	
	
	X
	

	Site Representatives Score
	Approval
	
	X
	
	
	X
	

	Site Homogeneity Score
	Approval
	
	X
	
	
	X
	

	Station Long Name
	Pre-Install
	X
	
	
	
	X
	

	Station Short Name (city and vector)
	Pre-Install
	X
	
	
	
	X
	

	GOES ID#
	Pre-Install
	X
	
	
	
	X
	

	UTC offset
	Pre-Install
	X
	
	
	X
	X
	

	State
	Pre-Install
	X
	
	
	X
	
	

	WBAN #
	Pre-Install
	X
	
	X
	X
	X
	

	Paired site relationship
	Pre-Install
	X
	
	
	
	X
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	Pre and post installation photos
	Installation
	
	X
	
	
	
	

	As Built Drawings (including instrument locations, height above ground level, exposure details)
	Installation
	
	X
	
	
	
	

	Site Install Issues Form
	Installation
	
	X
	
	
	
	

	Site Install Checklist
	Installation
	
	X
	
	
	
	

	Site Install Components Checklist
	Installation
	
	X
	
	
	
	

	NOAA Environmental Checklist
	Installation
	
	X
	
	
	
	

	Soil Type
	Installation
	X
	
	
	X
	
	

	Event (New station, maintenance)
	Installation
	X
	
	
	X
	X
	

	Event Date
	Installation
	X
	
	
	
	
	

	Latitude and longitude in decimal degrees and measurement method (GPS)
	Pre-Install
	X
	
	
	
	
	

	Elevation
	Installation
	X
	
	
	
	
	

	Government Property ID
	Installation
	X
	
	
	
	X
	

	
	
	
	
	
	
	
	

	Sensors
	
	
	
	
	
	
	

	Temperature Sensors
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Formula
	Installation
	X
	
	
	
	X
	

	#1, #2, #3 Sensor Serial  Number
	Installation
	X
	
	
	X
	
	

	Calibration Coefficient 0, 1, 2
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	Temperature Aspirated Shield
	Installation
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Aspirated Shield Fan
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	#1, #2, #3 Serial Number
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	Wind Speed Sensors
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Formula
	Installation
	X
	
	
	
	X
	

	Sensor Serial  Number 
	Installation
	X
	
	
	
	X
	

	Calibration Coefficient 0,1
	Installation
	X
	
	
	
	X
	

	
	
	
	
	
	
	
	

	Rain Gauge (Primary)
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Formula
	Installation
	X
	
	
	
	X
	

	#1, #2, #3 Transducer 1 Serial Number
	Pre-Install
	X
	
	
	X
	
	

	Calibration Coefficient 0, 1, 2 for each of 3 transducers
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	Rain Gauge (Secondary)
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Sensor Serial Number 
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	Solar Radiation Sensor
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Formula
	Installation
	X
	
	
	
	X
	

	Sensor Serial Number 
	Installation
	X
	
	
	X
	
	

	Calibration Coefficient 0, 1
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	Surface Infrared Temperature Sensor
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	

	Formula
	Installation
	X
	
	
	
	X
	

	Sensor Serial Number 
	Installation
	X
	
	
	X
	
	

	Calibration Coefficient 0, 1
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	Relative Humidity/Temperature Sensor
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Sensor Serial Number 
	Installation
	X
	
	
	X
	
	

	RH Formula 
	Installation
	X
	
	
	
	X
	

	Calibration Coefficient 0, 1
	Installation
	X
	
	
	X
	
	

	Temperature Formula
	Installation
	X
	
	
	
	X
	

	Calibration Coefficient 0, 1
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	Data Logger
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Serial Number 
	Installation
	X
	
	
	X
	
	

	Software Version Number 
	Installation
	X
	
	
	
	X
	

	Datalogger Software
	Installation
	
	X
	
	
	
	

	
	
	
	
	
	
	
	

	Transmission
	
	
	
	
	
	
	

	Method of transmission
	Operations
	
	X
	
	
	X
	X

	Data format transmitted
	Operations
	
	X
	
	
	X
	X

	Transmission frequency
	Operations
	
	X
	
	
	X
	X

	
	
	
	
	
	
	
	

	GOES Transmitter
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Serial Number 
	Installation
	X
	
	
	X
	
	

	Transmission time
	Installation
	X
	
	
	
	X
	

	Transmission channel
	Installation
	X
	
	
	
	X
	

	Transmitter EPROM software version
	Installation
	X
	
	
	
	X
	

	Antennae angle
	Installation
	X
	
	
	X
	
	

	Antennae orientation
	Installation
	X
	
	
	X
	
	

	GOES satellite assignment
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	Datalogger Charger/Controller
	
	
	
	
	
	
	

	Manufacturer
	Installation
	X
	
	
	
	X
	X

	Model Number
	Installation
	X
	
	
	
	X
	X

	Serial Number 
	Installation
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	State 
	Acceptance
	X
	
	
	X
	
	

	Climate Division 
	Acceptance
	X
	
	
	X
	
	

	County 
	Acceptance
	X
	
	
	X
	
	

	Climate Summary from nearby site
	Commission
	
	X
	
	X
	
	

	Coop number
	Acceptance
	X
	
	
	X
	
	

	Commission Code
	Commission
	X
	
	
	
	
	X

	Operational Status
	Operations
	X
	
	
	X
	X
	

	
	
	
	
	
	
	
	

	Manual Quality Control Events
	
	
	
	
	
	
	

	Suspected data or instrument problems entered into ATS
	Operations
	
	
	X
	
	
	

	Known data problem notes
	Operations
	
	
	X
	
	
	

	
	
	
	
	
	
	
	

	Site Maintenance Metadata defined by
	
	
	
	
	
	
	

	Site Event Form – (Host routine maintenance)
	Operations
	
	X
	
	X
	
	

	Annual Visit Form – (Annual maintenance)
	Operations
	
	X
	
	X
	
	

	Site Visit Form – (Non-routine maintenance)
	Operations
	
	X
	
	X
	
	

	
	
	
	
	
	
	
	

	Temporal Changes to any metadata items logged as appropriate 
	Operations
	X
	
	
	X
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	


TABLE 2 
NETWORK METADATA
 FOR USCRN 

Metadata that describe the data for the network in its entirety are indispensable for later evaluation of the data.  The Categories of these metadata are Instrument Manuals and Documentation, Research Papers, Software, and Data Set Documentation.  Each of these categories if listed with details included.  The repository and control are also shown.
	METADATA
	SOURCE
	REPOSITORY
	CONTROL

	
	
	CRN

SITES
	WSSRD
	DATA

ADMIN
	NM
	CM
	CI

CCB

	Instrument Manuals and Documentation
	Manufacturers 
	
	X
	X
	
	
	

	instrument type 
	
	
	
	
	
	
	

	sampling interval 
	
	
	
	
	
	
	

	reporting frequency 
	
	
	
	
	
	
	

	recommended scheduled maintenance and calibration procedures, including frequency 
	
	
	
	
	
	
	

	manufacturer 
	
	
	
	
	
	
	

	model 
	
	
	
	
	
	
	

	phenomena measured (temperature, precipitation, wind speed, etc.) 
	
	
	
	
	
	
	

	how the instrument functions 
	
	
	
	
	
	
	

	data sampling and processing procedures
	
	
	
	
	
	
	

	sampling interval range 
	
	
	
	
	
	
	

	measurement range 
	
	
	
	
	
	
	

	resolution 
	
	
	
	
	
	
	

	accuracy 
	
	
	
	
	
	
	

	digital images, including photos, diagrams and scanned copies of specification sheets 
	
	
	
	
	
	
	

	service schedule and details, including 
	
	
	
	
	
	
	

	service type (installation, preventive maintenance, calibration, etc) 
	
	
	
	
	
	
	

	procedure description 
	
	
	
	
	
	
	

	standard values 
	
	
	
	
	
	
	

	required expertise & equipment 
	
	
	
	
	
	
	

	schedule/ frequency.
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	Research Papers
	USCRN Web
	
	
	
	
	
	

	CRN program experimental results
	
	
	X
	
	
	
	

	Peered reviewed papers by CRN program
	
	
	X
	
	
	
	

	Peered reviewed papers by the community at large
	
	
	X
	
	
	
	

	Presentations and working documents for meetings as appropriate
	
	
	X
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	Software Source Code and Documentation
	Operations
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	Ingest
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	CRN_INGEST/SRC
	
	
	X
	X
	X
	X
	

	CRN_IO
	
	
	X
	X
	X
	X
	

	LOAD_LRGS
	
	
	X
	X
	X
	X
	

	LOAD_SRRS
	
	
	X
	X
	X
	X
	

	LOAD_NOAAPort
	
	
	X
	X
	X
	X
	

	CRN_Ingest/EXE
	
	
	X
	X
	X
	X
	

	PDA_Ingest
	
	
	X
	X
	X
	X
	

	
	
	
	
	
	
	
	

	Automated Quality Control
	
	
	
	
	
	
	

	Quality Control Flags definitions 
	
	
	X
	X
	X
	X
	

	CRN_QC software
	
	
	X
	X
	X
	X
	

	
	
	
	
	
	
	
	

	Archive
	
	
	
	
	
	
	

	CRN2DB
	
	
	X
	X
	X
	X
	

	CRNARCHIVE
	
	
	X
	X
	X
	X
	

	
	
	
	
	
	
	
	

	DataSet TD3286 Documentation
	NCDC web
	
	
	
	
	
	

	Manual
	
	
	
	X
	X
	
	

	Known problems with data and their period of record
	
	
	
	X
	X
	
	

	ATS reports
	
	
	
	X
	X
	
	

	Science Source File of interesting data cases
	
	
	
	X
	X
	
	

	
	
	
	
	
	
	
	


Notes:

1.
Control indicates NM Network Monitor (NM), Configuration Management (CM) Configuration Item (CI) and Configuration Control Board (CCB).

2.
CRNSITES is the staging area  for the NCDC official Station History Database, MI3

3.
WSSRD is currently coming on-line as official Metadata storage for imagery and document metadata.

4.
Temporal changes to any of this information is recorded as well.

Appendix A Document Change Log

	Date
	Table # and section
	Description of Change
	Are Redlines Attached?

	08012003
	1 - Sensors
	Added mfg, model and serial # fields for tipping bucket rain gauge.
	N

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


Page 10 of 9

