










October 17, 2005
Photographically Documenting USCRN Sites

Photos should be made when there is good visibility at 100 meters.  On clear days, the pictures should be taken as close to noon as possible.  File names should include the compass direction where appropriate.  Archive file format is tif.

Prior to installation:

1)  One Ipix from the center of the tower site denoting objects within 100 meters and their heights.  The finished Ipix should have an aligned compass in the center label area with 4 compass points labeled.    The 186-degree fisheye component shots will be part of the official archive.  

2)  Sixteen overlapping annotated still photos from the center of the tower site taken every 22.5 degrees (the sixteen major compass points) denoting objects within 100 meters and their heights.

After installation:

1) Four Ipix taken at ten meters from the tower fence at the four points of the compass. Each Ipix should have an aligned compass in the center label area.  The 186-degree fisheye component shots will be part of the official archive.  If the annual site visit uncovers significant change, these shots are to be retaken.  

2) From a position four meters west of the tower and starting at due south, nine still photos taken every 22.5 degrees clockwise to due north denoting objects within 100 meters and their heights. Stitch photos into a single 180-degree image.  To be retaken at each annual site visit and photos placed in the archive.

3) From a position four meters east of the tower and starting at due north, nine still photos taken every 22.5 degrees clockwise to due south denoting objects within 100 meters and their heights.  Stitch photos into a single 180-degree image.  To be retaken at each annual site visit and photos placed in the archive.

4) From a position fifty meters south of the tower sixteen still photos taken every 22.5 degrees at the major compass points.  To be retaken at each annual site visit and photos placed in the archive.
5)  From a position fifty meters north of the tower sixteen still photos taken every 22.5 degrees at the major compass points.  To be retaken at each annual site visit and photos placed in the archive.
6) From a position four meters north of the tower take a photo of the ground below the instruments with the instruments at top of the frame.  Repeat this from positions four meters east, west and south of the tower.  To be retaken at each annual site visit and photos placed in the archive.
7) At least four general site views that represents the most informative overall perspective.  Two of the photos should be taken with the camera angle perpendicular to a line drawn between the DFIR and the tower, with the margin of sky no more than 1/3 of the height of the photo, and the station comprising 75% of the image. To be retaken at each annual site visit and photos placed in the archive.
Change Log

	Date
	Section and Paragraph Number
	Description of Change
	Are Redlines Attached?

	5/23/05
	
	Made 50 meter north, 50 meter south, and ground series required at annual maintenance visit.  DSB
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


